3GPP SA WG2#173	S2-2601307

Goa, India, 9-13 February, 2026	(Revision of S2-2600412)

Source:	OPPO, Huawei, NTT Docomo, NEC

Title:	Conclusion update for R20 AIoT KI#1

Spec:	3GPP TR 23.700-30

Agenda item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Document for:	Approval

Abstract of the contribution: This contribution is to update the interim conclusion for KI#1.

1. Discussion

Based on the endorsed last page in S2-2511016 at SA2#172, it’s proposed to resolve the ENs for KI#1 interim conclusion.

Since in TS38.413, the AMF UE NGAP ID and RAN UE NGAP ID are included in the NG-AP message and used to identify a UE, on top of endorsed S2-2511016 (AMF UE NG-AP ID and AMF set ID is used to identify the UE reader between NG-RAN and AIOTF), the AMF UE NGAP and RAN UE NGAP ID is used to identify the UE reader to align with TS38.413.

In TS 38.413, the reader index has the following format

9.3.1.276	Reader Index

This IE represents identifier of a Reader within a gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reader Index | M |  | INTEGER (1..65536, …) |  |

This may be used when then AIOTF has selected UE reader and provide the selected UE reader information to the NG-RAN

This is a merged version from the following contributions:

| S2-2600412 | 23.700-30: Conclusion update for R20 AIoT KI#1. | OPPO, Huawei, NTT Docomo, NEC |
| --- | --- | --- |
| S2-2600136 | 23.700-30: Update the interim agreements of the KI#1. | China Mobile |
| S2-2600318 | 23.700-30: Conclusions for KI#1. | Samsung |
| S2-2600687 | 23.700-30: Conclusions on Key Issue 1. | Lenovo |
| S2-2600696 | 23.700-30: KI#1 Conclusion: Resolve ENs on UE Reader Subscription Aspect. | ZTE |
| S2-2600703 | 23.700-30: KI#1 Conclusion on UE reader selection about last known reader. | LG Electronics |
| S2-2600960 | 23.700-30: [KI1, Conclusion] T2 conclusion update_UE selection aspect. | NTT DOCOMO |
| S2-2601081 | 23.700-30: [KI1#1] Conclusion update on UE Reader selection aspect. | InterDigital Inc. |
| S2-2601071 | 23.700-30: [KI1, Conclusion] Topology 2 Conclusion Update | Huawei, HiSilicon |

2. Proposal

It is proposed to agree the following changes to 3GPP TR23.700-30.

* * * * First Change * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

'Figure 7.1.1-1 depicts the AIoT System Architecture for Topology 2.

Figure 7.1.1-1: AIoT System Architecture for Topology 2

Figure 7.1.1-2 depicts the AIoT system architecture, using the reference point representation.

Figure 7.1.1-2: AIoT System Architecture for Topology 2 in reference point representation

Message and protocol stack

-	Messages between the UE Reader and the AIOTF are delivered using RRC between UE and NG-RAN and NGAP between NG-RAN and AMF and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 7.1.1-3.

Figure 7.1.1-3: Protocol Stack for the RRC option

UE Reader 	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

-	UE Reader subscription information is available to AMF.

UE Reader Authorization:

-	If AMF receives the capability indication as UE Reader in the UE MM Core Network Capability from UE in the Registration Request and the indication that the UE is authorized to operate as a UE Reader in the subscription information from UDM:

a)	AMF sends an authorization indication as UE Reader to NG-RAN that the UE is authorized to operate as a UE Reader.

UE Reader Revocation:

-	If the subscription information has been revoked, the indication that the UE is not authorized to operate as a UE Reader from the UDM, then the AMF informs NG-RAN that the UE is not authorized to operate as a UE Reader.

Radio resource management for UE Reader operation:

-	If the NG-RAN has received the indication that a UE is authorized to operate as a UE Reader, then the NG-RAN may assign radio resources to the UE for UE Reader operation.

UE Reader ID information at AIOTF:

-	UE Reader ID information is used by the AIOTF to select the UE reader. When receiving the AF Request, the AIOTF retrieves the UE reader ID information (AMF UE NGAP ID, RAN UE NGAP ID, and NG-RAN ID) from the serving AMF of the UE reader by sending SUPI to AMF. AMF UE NGAP ID, RAN UE NGAP ID and NG-RAN ID are used to identify the UE reader during AIoT Service operation between AIOTF and NG-RAN.

AIOTF Discovery and Selection when AF provides UE reader ID:

-	When receiving the UE reader ID, the NEF queries the UDM using the Nudm_UECM_Get service operation to get the serving AMF of the UE reader and selects the AIOTF according to the local configuration.

NOTE:	NEF is configured with the mapping between the AMF ID and AIOTF ID(s).

UE reader selection part:

The following scenarios will be supported for UE reader selection:

-	For AF providing UE reader ID case:

-	If UE Reader ID(s) is provided by the AF via the NEF for the operation, then that is used as the selected Reader(s) if authorized as UE reader. The AIOTF provides the selected UE Reader(s) to the NG-RAN.

-	AMF UE NGAP ID, RAN UE NGAP ID, and NG-RAN ID are used to identify the UE reader during AIoT Service operation between AIOTF and NG-RAN.

-	The AIOTF receives the AMF UE NGAP ID, RAN UE NGAP ID, and NG-RAN ID from the AMF by providing SUPI to AMF.

-	The AIOTF includes the AMF UE NGAP ID and RAN UE NGAP ID in the NGAP AIoT information (i.e., the area information is not included) and uses the procedures defined in clause 6.2.4 of TS 23.369 [3]. The response NGAP AIoT information from NG-RAN also includes AMF UE NGAP ID and RAN UE NGAP ID which the AIOTF uses to identify a specific UE Reader in the case there were multiple UE Readers in the request towards NG-RAN.

NOTE x:	The usage of AMF UE NGAP ID, RAN UE NGAP ID, and NG-RAN ID to identify the UE reader requires coordination with RAN3.

-	For AF providing only target area case:

-	AIOTF selects the UE Reader(s) based on its local configuration and the AIOTF provides the selected UE reader ID information to the NG-RAN.

NOTE x:	How to identify the configured UE reader between AIOTF and NG-RAN will be coordinated with RAN3.

-	Or AIOTF selects the NG-RAN node and determines a Requested Service Area Information for each selected NG-RAN node using the existing procedures as specified in clause 5.3.3 in TS 23.369 [3] and it is up to NG-RAN to then select the UE reader to use. The discussion will be coordinated with RAN3.

-	For Last known UE reader stored in AIOTF case:

-	If an AIoT service operation request received by the AIOTF includes individual target AIoT Device Permanent ID(s), the AIOTF may consider the last known UE Reader, if available from the AIoT Device context to determine the UE Reader(s) for the request. The AIOTF provides the last known UE Reader(s) to the NG-RAN.

* * * * Next Change * * * *

8	Conclusions

The agreed principles for KI#1 in clause 7.1.1 is used as basis for the normative work.

* * * * End of Changes * * * *