3GPP TSG-WG SA2#173	S2-2601147

# Goa, India, 9 – 13 February, 2026

Source:	Samsung, Nokia, Deutsche Telekom (Pen-holder(s))

Title:	KI#1.2:Control signaling for 6G System: generic mechanism for operator services – penholders input for SA2#173

Document for:	Approval

Agenda item:	20.6.2

Work Item:	FS_6G_ARC

Abstract of the contribution: The contribution provides solution variants for KI#1.2.

1.	Discussion

The contribution is a pen-holder input for KI#1.2 of KI#1 and generated based on the submitted input solutions listed in Annex X.

Key Issue #1.2 1. Study the support for control signalling for 6G System, including at least the following: a) Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services. NOTE 5: This KI#1.2 bullet 1a can include any transport mechanism such as using NAS or UP or new plane. KI#1.2 bullet 1a can have dependency on other key issues defined for e.g. KI#1.1 bullet 1a, KI#18, KI#19, KI#20, KI#21, KI#22. NOTE 6: 6G System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signalling interaction between RAN-CN need coordination and alignment with RAN WG3. NOTE 7: Overall Interworking and migration aspects are covered by key issues for WT#2 (KI#17). Impact to IWK due to features introduced by this key issue will be covered by this key issue in alignment with WT#2 (KI#17). NOTE 8: This KI#1 covers operator services.

This pCR proposes to capture

-	5 solutions variants based on selected input solutions for KI#1.2.

-	list of the submitted input solutions in Annex X

The solution variants are:

- Solution variant #1.2.1: Distributed NAS-based control signaling for operator services over NAS

- Solution variant #1.2.2: Backward compatible NAS-based control signaling for operator services over NAS

- Solution variant #1.2.3: NAS-based control signaling for operator services over user plane

- Solution variant #1.2.4: New control framework for operator services

- Solution variant #1.2.5: RAN-CN interactions for operator services

2.	Text proposal

It is proposed to agree the following changes vs. TR 23.801-01:

* * * First Change * * * *

6.1.2	Solutions to KI#1.2

6.1.2.1	Solution variant #1.2.1: Distributed NAS-based control signaling for operator services over NAS

6.1.2.1.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#1.2 and is extracted from Solution#1, Solution#2, Solution#3.

The high-level principles of the solution are listed as below.

-    NAS signaling is used as a common control signaling for operator services to support service discovery, connection management, and transportation of service specific signaling (e.g., UE service capability, service request)

-	NAS transport mechanism is used to support transport of 6G control signalling for operator services

-    6G Mobility Management (MM) and 6G Connection Management (CM) functionalities are separated into different network entities

-    A new network function (e.g., Connection Management Function (CMF)) is introduced to perform following handling between the UE and 6G service NFs

-    NF discovery/selection

-	Connection management

-	UL/DL control signaling distribution for operator services

Editor's note:	Any aspect dependent on design of NAS will be aligned with KI#1.1.

6.1.2.1.1	Description

6.1.2.1.2	Procedures

6.1.2.1.3	Services, Entities and Interfaces

6.1.2.1.4	Issues

* * * Next Change * * * *

6.1.2.2	Solution variant #1.2.2: Backward compatible NAS-based control signaling for operator services over NAS

6.1.2.2.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from Soltuion #4, Solution #5

This solution proposes the following principles:

- 	The proposed solution is based on a CN architecture evolved from 5GC (Solution #4)

-     Direct NAS transport and container based NAS transport supported by the evolved AMF can be used as a transport mechanism between UE and NF for operator service

- 	This solution assumes NAS that supports backward compatibility with 5G (Solution #5)

-    The NAS transport can be enhanced so that addition of services does not impact NAS-MM/AMF.

Editor's note:	Any aspect dependent on design of NAS will be aligned with KI#1.1.

6.1.2.2.1	Description

6.1.2.2.2	Procedures

6.1.2.2.3	Services, Entities and Interfaces

6.1.2.2.4	Issues

* * * Next Change * * * *

6.1.2.3	Solution variant #1.2.3: NAS-based control signaling for operator services over user plane

6.1.2.3.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from (Solution #1, Solution #2, Solution #3, Solution #7, Solution #8, Solution #9, Solution #15)

This solution proposes the following principles:

- The UE uses the NAS signalling to provide service request and a service specific capability for the operator service to the 6G NF (e.g., new NF (i.e., Connection Management Function), AMF, or SMF)

- The NAS supports functionality to handle authorization and discovery of the requested operator service.

- The NAS terminating NF uses the NAS signalling to provide to the UP address information of the NF to the UE

- The UE transmit signalling and/or data exchange with the NF that provides the operator service, via the user plane.

- Mechanisms to authenticate/authorize the application for operator service in the UE (Solution #7)

6.1.2.3.1	Description

6.1.2.3.2	Procedures

6.1.2.3.3	Services, Entities and Interfaces

6.1.2.3.4	Issues

* * * Next Change * * * *

6.1.2.4	Solution variant #1.2.4: New control framework for operator services

6.1.2.4.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from Solution #10, Solution #11, Solution #12, Solution #13, Solution #14, Solution #16, Solution #17, Solution #18, Solution #19, Solution #20, Solution #21

This solution proposes the following principles:

The high level summary is:

-  A new control framework for new operator services is introduced to minimize the impact on NAS

-  The control framework supports generic mechanisms for operator services such as transport of operator service signaling, service capability registration, service discovery, service authentication

-  A generic control signaling for operator services is defined to support interactions between the UE and NF for operator service

NOTE X:	Coordination with CT1 is required

Editor’s Note: What transport path the new control signaling for operator services is FFS (e.g., based on 6G N6, UPF SBI, RAN routing, etc.).

6.1.2.4.1	Description

6.1.2.4.2	Procedures

6.1.2.4.3	Services, Entities and Interfaces

6.1.2.4.4	Issues

* * * Next Change * * * *

6.1.2.5	Solution variant #1.2.5: RAN-CN interactions for operator services

6.1.2.5.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from Solution #11, Solution #17, Solution #21

This solution proposes the following principles:

- The solution considers new operator services in 6G that require interactions between RAN and CN

- A new interface between 6G RAN and NF for operator service other than 6G N2 is supported to support generic mechanism for operator services such as transport of operator service signaling

6.1.2.5.1	Description

6.1.2.5.2	Procedures

6.1.2.5.3	Services, Entities and Interfaces

6.1.2.5.4	Issues

* * * Next Change * * * *

Annex X: Submitted solutions

X.Y	List of submitted solutions for KI#Y

Table X.Y: List of submitted solutions

| Meeting | Solution# | TDoc | Companies | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600150 | Vivo | [KI#1.2] Solution proposal on NAS-based UE-CN interaction mechanism |
| SA2#173 | 2 | S2-2600100 | China Mobile | [KI#1, all bullets] Solution for KI#1 of intent-based signalling framework |
| SA2#173 | 3 | S2-2600434 | Huawei, HiSilicon | [KI#1] Solution #1.X: Solution for E2E control signalling in 6G system |
| SA2#173 | 4 | S2-2600249 | Ericsson | [KI#1, bullet #1.1 and #1.2] New Solution: UE and CN control signalling based on evolvement of 5G NAS |
| SA2#173 | 5 | S2-2600594 | Qualcomm | [KI#1.2] Solution for Support for operator services – a framework |
| SA2#173 | 6 | S2-2600266 | LG Electronics | [KI#1] Distributed NAS signalling support for new operator services with extended NAS architecture |
| SA2#173 | 7 | S2-2600092 | Futurewei | [KI#1, Bullet 2] Operator Service over User Plane Transport |
| SA2#173 | 8 | S2-2600160 | OPPO | [KI#1.2, bullet #1a] control signalling over UP |
| SA2#173 | 9 | S2-2600236 | Lenovo | [KI#1.2, bullet #1] Solution for transport control signalling for operator services over user plane |
| SA2#173 | 10 | S2-2600216 | Lenovo | [KI#1.2, bullet 1] Support control plane signalling for over the top operator services over user plane |
| SA2#173 | 11 | S2-2600219 | Google | KI #1: Generic Service Stratum Framework via User-Plane Tunnels |
| SA2#173 | 12 | S2-2600308 | NEC | [KI#1.1, 2a] Procedure to support generic mechanisms for UE to Core Network interaction to support operator services. |
| SA2#173 | 13 | S2-2600388 | Rakuten Mobile, China Mobile | [KI#1, bullet #2] Solution for UE–Core Interaction in 6G for control signalling to support operator services |
| SA2#173 | 14 | S2-2600397 | TCL | [KI#1] Solutions for generic UE-CN interaction mechanism |
| SA2#173 | 15 | S2-2600472 | ZTE | [KI#1.2] Service request procedure for operator service |
| SA2#173 | 16 | S2-2600489 | CATT | [KI#1.2] Support for operator services based on RAN SBI interface |
| SA2#173 | 17 | S2-2600495 | Samsung | [KI#1.2] Introduction of service plane for operator services |
| SA2#173 | 18 | S2-2600524 | Xiaomi | [KI#1, bullet #2] Solution for generic transport mechanism for UE-CN control signalling |
| SA2#173 | 19 | S2-2600532 | Apple | [KI#1.1, KI#1.2] New solution on Evolved Control Signalling for 6G |
| SA2#173 | 20 | S2-2600572 | InterDigital | [KI#2, KI#1.2] A Generic Framework to Support Communication with Operator Services |
| SA2#173 | 21 | S2-2600198 | NTT DOCOMO | [KI#1] Re-use of 5GS RAN-CN interfaces |

* * * End of Changes * * * *