3GPP TSG-SA3 Meeting #126	S3-260993

# Goa, India, 9 – 13 February 2026

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0005 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the exact IE of producer PLMN ID verified by NF Producer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2026-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| 760 |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Originally from S3-260473, and during discussion, we notice that it is dependent of S3-260760, thus S3-260473 needs to be converted to a CR and aligned with S3-260760 in terms of description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make the description of Producer PLMN ID aligned with TS 33.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Disalignment with description in TS 33.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.2.2.2.3.2	Authorization token verification failure handling in different PLMNs

Requirement Name: Authorization token verification failure handling in different PLMNs

Requirement Reference: TS 33.501 [10], clause 13.4.1.2

Requirement Description:

The NF service producer checks that the home PLMN ID  in the access token claims matches its own PLMN identity.

Threat References:  TR 33.926 [4], clause 6.3.3.1, Incorrect Verification of Access Tokens

NOTE:	The test case below only applies to the NFs which support identifying and understanding the producerPlmnId (as of TS 33.501 [10] v15.3.0).

Test Case:

Test Name: TC_AUTHORIZATION_TOKEN_VERIFICATION_FAILURE_DIFF_PLMN

Purpose:

Verify that the NF service producer does not grant service access if the verification of authorization token from a NF service consumer in a different PLMN fails.

Procedure and execution steps:

Pre-Conditions:

-	Test environment with a NF service consumer and two SEPPs (one cSEPP, one pSEPP).

-	The NF service consumer and SEPPs may be simulated.

-	The network product under test has already mutually authenticated with the NF service consumer in a different PLMN via the SEPPs.

-	The tester has the NRF’s private key or the shared key.

-	The network product under test is preconfigured with the NRF’s public key or the shared key.

-	The tester shall have access to the interfaces of the NF service consumer and the network product under test.

Execution Steps

The network product under test receives the access token sent from the NF service consumer, verifies the access token in accordance with the execution steps in 4.2.2.2.3.1, with the following additional test cases:

Test Case 1: incorrect PLMN ID of the NF service producer in the access token

1)	The tester computes an access token correctly, except that the PLMN ID in the producerPlmnId claim of the access token is empty or different from the home PLMN ID of the network product under test, and then includes the access token in the NF Service Request sent from the NF service consumer to the network product under test through the SEPPs.

2)	The network product under test receives the access token sent from the NF service consumer through the SEPPs, verifies that the PLMN ID in the producerPlmnId claim of the access token is different from its own home PLMN identity.

Test Case 2: absent PLMN ID of the NF service producer in the access token

1)	The tester computes an access token correctly, except that no producerPlmnId claim is included in the access token, and then includes the access token in the NF Service Request sent from the NF service consumer to the network product under test through the SEPPs.

2)	The network product under test receives the access token sent from the NF service consumer through the SEPPs, verifies that the access token is not a token to be used by the NF service consumer in a different PLMN, based on the absence of PLMN ID of the NF service producer in the access token.

Expected Results:

For both test cases 1 and 2, the network product under test rejects the NF service consumer’s service request based on OAuth 2.0 error response defined in RFC 6749 [12].

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

* * * End of Changes * * * *