# 3GPP TSG-SA3 Meeting #126																			 draft-S3-260827_r1

# GOA, India, 9th – 13th Feb. 2026

Source:	OPPO

Title:	T-ID based AIOT device privacy protection

Document for:	Approval

Agenda item:	5.2.11

Spec:	3GPP TR 33.714

Version:	0.2.0

Work Item:	FS_AIoT_SEC_Ph2

Comments

It is proposed a new solution on T-ID based AIOT device privacy protection.

Proposed Changes

* * * First Change * * * *

5.Y	Solution #Y: T-ID based AIOT device privacy protection

5.Y.1	Introduction

This solution utilizes Temporary ID (T-ID) to protect AIOT device privacy, and considering following procedures: 1. Initial Registration procedure, 2. DO-A procedure, 3. Inventory and command procedure.

5.Y.2	Solution details

The solution makes the following assumptions:

- DO-A-capable devices are to be used for AIoT services supported in Topology 1 and Topology 2 for deployment in public networks.

- DO-A-capable devices support the use of UICC and therefore support the security capabilities that are necessary to perform device privacy protection.

NOTE X: The exact form factor of UICC to be supported in DO-A-capable devices is left for implementation.

1. AIOT ID protection for initial registration procedure

- In initial registration, the AIOT device uses concealed permanent ID i.e., SUCI or the T-ID which is allocated by AIOTF or ADM, in the registration request message.

- The AIOT device conceals its permanent ID and generates SUCI as specified in Annex C in TS 33.501[9], in which null-scheme could be used if the device has been configured by the Network. If non-null scheme calculation SUCI is used, upon reception of the SUCI, the ADM retrieves device permanent ID from SIDF using SUCI.

- After security establishment, the AIOTF or ADM allocates and securely sends the new temporary ID to the AIOT device. The AIOT device stores the allocated T-ID and update its stored T-ID if any.

Editor’s Note: Deciphering SUCI in the network side is FFS.

2. AIOT ID protection for DO-A procedure

In DO-A procedure, the device autonomously sends message to the AIOTF, along with the temporary ID allocated by the AIOTF or ADM.  And new T-ID could be allocated by the AIOTF or ADM and sent to the device to update T-ID.

3. AIOT ID protection for Inventory and Command procedure

It is assumed that the inventory and command procedure only happen after initial-registration procedure.

- For individual inventory, the AIOTF uses network assigned T-ID to inventory AIOT device, and the device responses the AIOTF with the T-ID.

Editor’s Note: Whether and how the DO-A capable device supports group paging or paging all procedure and the corresponding ID protection procedure is FFS.

- For command procedure, the AIOTF/ADM could allocate and securely sends the new temporary ID to the AIOT device.

Editor’s Note: Whether SUCI is quantum-resistant is FFS.

Editor’s Note: Alignment with SA2’s conclusion is FFS.

5.Y.3	Evaluation

This solution requires the AIOT device to support SUCI based permanent ID concealing mechanism, and store new T-ID generated by the Network.

Editor’s Note: Further evaluation is FFS.

* * * End of Change * * * *