# 3GPP TSG-SA3 Meeting #126																			  draft-S3-260885_r1

# GOA, India, 9th – 13th Feb. 2026

Source:	OPPO

Title:	New solution on using NCAx as cipher and integrity algorithm

Document for:	Approval

Agenda item:	5.3.2

Spec:	3GPP TR 33.771

Version:	0.2.0

Work Item:	FS_AEAD

Comments

It is proposed to add new solution on using NCAx as cipher and integrity algorithm.

Proposed Changes

* * * First Change * * * *

6.Y	Solution #Y: Using NCAx as cipher and integrity algorithm

6.Y.1	Introduction

This solution addresses Key issue #2: AEAD algorithm interface.

6.Y.2	Solution details

It is assumed that key (256-bits KAEAD) has been generated and algorithm negotiation has been performed between UE and network, it is agreed to use AEAD algorithm NCAx as cipher and integrity algorithm.

To use 256-NCAx as both cipher and integrity algorithm, inputs and outputs to the algorithm interface as specified in TS 35.240 [2], TS 35.243 [3], or TS 35.246 [4] for 256-NCAx are set as follows, illustrated here with algorithm input and output parameters of 5G set as an example.

- The input KEY is set to 256-bits KAEAD.

- The COUNT input is set to NAS COUNT for NAS or PDCP COUNT for PDCP.

- The BEARER input is set to the NAS connection identifier for NAS or BEARER ID for PDCP.

- The DIRECTION bit input is set to 0 for uplink and 1 for downlink.

- The MODE input is set to 0

NOTE: Sender of message sets MODE to 0 while receiver of message sets MODE to 1.

- MAC_BYTES is set to the length of MAC-I.

- Associated Authentication Data (AAD) is set to the bit string of message to be only integrity protected.

- AAD_LENGTH input is set to the length of AAD.

- MESSAGE input is set to the bit string of message to be both integrity and encryption protected.

- MESSAGE_BYTES is set to the length of the message.

Outputs to the algorithm interface are as follows:

- For the sender, the output parameters are ciphertext and MAC-I.

- For the receiver, the output parameters are cleartext and the XMAC-I.

6.Y.3	Evaluation

Inputs and outputs of the above solution conform to the inputs and outputs as specified in TS 35.240 [2], TS 35.243 [3], and TS 35.246 [4] for 256-NCAx algorithms. No further evaluation is warranted.

* * * End of Change * * * *