3GPP TSG-SA3 Meeting #126	draft_S3-260178-r1

# Goa, India, 9 – 13 February 2026

Source:	Nokia

Title:	Pseudo-CR on Solution evaluation for solution 11

Document for:	Approval

Agenda item:	5.3.2

Spec:	3GPP TR 33.771

Version:	0.2.0

Work Item:	FS_AEAD

Comments

This pCR is introduces the solution evaluation for solution#11.

* * * Start of Change * * * *

## 6.11	Solution 11: Key Derivation for NAS and AS AEAD

### 6.11.1	Introduction

This solution addresses the key issue#3.

### 6.11.2	Solution details

Keys for NAS signalling:

-	KNASAEAD is a key derived for particular combined algorithm (256-NCA4/256-NCA5/256-NCA6).

Keys for UP traffic:

-	KUPAEAD is a key derived for a particular combined algorithm(256-NCA4/256-NCA5/256-NCA6).

Keys for RRC signalling:

- KRRCAEAD is a key derived for a particular integrity & encryption (combined) algorithm(256-NCA4/256-NCA5/256-NCA6).

Algorithm key derivation functions

When deriving keys for NAS integrity and NAS encryption algorithms from KAMF in the AMF and UE or ciphering and integrity keys from KgNB/ KSN in the gNB and UE, the following parameters shall be used to form the string S.

-	FC = 0x69

-	P0 = algorithm type distinguisher

-	L0 = length of algorithm type distinguisher (i.e. 0x00 0x01)

-	P1 = algorithm identity

-	L1 = length of algorithm identity (i.e. 0x00 0x01)

The algorithm type distinguisher shall be N-NAS-enc-alg for NAS encryption algorithms and N-NAS-int-alg for NAS integrity protection algorithms. The algorithm type distinguisher shall be N-RRC-enc-alg for RRC encryption algorithms, N-RRC-int-alg for RRC integrity protection algorithms, N-UP-enc-alg for UP encryption algorithms and N-UP-int-alg for UP integrity protection algorithms, N-NAS-AEAD-alg for NAS AEAD algorithms, N-RRC-AEAD-alg for RRC AEAD algorithm and N-UP-AEAD-alg for UP AEAD algorithm (see table A.8-1). The values 0x00 and 0x0a to 0xf0 are reserved for future use, and the values 0xf1 to 0xff are reserved for private use.

Table 6.11.2-1: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| N-NAS-enc-alg | 0x01 |
| N-NAS-int-alg | 0x02 |
| N-RRC-enc-alg | 0x03 |
| N-RRC-int-alg | 0x04 |
| N-UP-enc-alg | 0x05 |
| N-UP-int-alg | 0x06 |
| N-NAS-AEAD-alg | 0x07 |
| N-RRC-AEAD-alg | 0x08 |
| N-UP-AEAD-alg | 0x09 |

The algorithm identity (as specified in clause 5 of TS 33.501 [5]) shall be put in the four least significant bits of the octet. The two least significant bits of the four most significant bits are reserved for future use, and the two most significant bits of the most significant nibble are reserved for private use. The entire four most significant bits shall be set to all zeros.

For the derivation of integrity and ciphering keys or AEAD key used between the UE and gNB, the input key shall be the 256-bit KgNB// KSN. For the derivation of integrity and ciphering keys or AEAD key used between the UE and AMF, the input key shall be the 256-bit KAMF.

For an algorithm key of length n bits, where n is less or equal to 256, the n least significant bits of the 256 bits of the KDF output shall be used as the algorithm key.

### 6.11.3	Evaluation

The proposed derivation method extends the existing 5G Key derivative function, by introducing dedicated AEAD type distinguishers, ensuring backward-compatible key hierarchy while providing a single 256-bit AEAD key per algorithm. By binding the key to both the algorithm identifier and the underlying master key (KAMF or KgNB/KSN), it mitigates the risk of key-reuse across separate encryption/integrity functions.

* * * End of Changes * * * *