3GPP TSG-SA3 Meeting #126	S3-260222-r10

# Goa, India, 9 – 13 February 2026

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.369 | CR | 0074 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the Nadm_SecTID_Get service operation and out of sync detection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-SEC |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 5.4.3 “Procedure for AIoT Device Identifier protection with T-ID update during Individual inventory” describes in step 1 the Nadm_SecTID_Get service operation to retrieve a T-ID. It is clarified that the T-ID fetched from the ADM is at the time T-IDn and that the Further clarifications are made in the clause 5.4.4 “Out-of-Synch detection and Resynchronization of T-ID” with respect to the device states and the related T-IDs for the out-of-sync scenarios. The T-IDn+1 is removed from the Nadm_SecTID_Get service operation according to the LS from CT4 (C4-255429). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections to the device states and the related T-IDs for the out-of-sync scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3, 5.4.4, 6.1.1, 6.1.5, 6.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of 1st Change

### 5.4.3	Procedure for AIoT Device Identifier protection with T-ID update during Individual inventory

For the protection of AIoT device permanent identifier during the inventory procedure with AIoT Device Identifier described in clause 5.2.2, the following changes shall apply:

-	In step 1, AIOTF shall invoke Nadm_SecTID_Get service operation to retrieve a T-ID (T-ID_n) in addition to the RANDAIOT_n from ADM using the AIoT Device Permanent Identifier. The ADM shall, based on T-ID handling information stored in the AIoT device profile, either fetch the stored T-ID in the AIoT Device profile or generate the T-ID based on the AIoT Device Permanent Identifier as specified in Annex B.1, and send the T-ID to the AIOTF.

-	In steps 2, 3, and 4, the T-ID shall be used as the AIoT Identification Information.

-	In step 2 the AIOTF includes the T-ID handling information in the Inventory Request message. T-ID handling information includes:

-	T-ID type that can be either a concealed type or a stored type. If the T-ID type is the concealed type, the T-ID is generated based on the AIoT Device Permanent Identifier. If the T-ID is the stored type, the T-ID is generated based on the stored T-ID as specified in Annex B.1.

-	Whether the stored T-ID type is updated with a command via a Command procedure or without a command during step 4.

NOTE A: If the T-ID is of stored type, the initial value of the T-ID is to be computed based on the AIOT Device Permanent ID.

-	In step 3, NG-RAN includes the T-ID handling information in the paging message.

-	In step 4, the AIoT device, based on the T-ID handling information in the paging message, generates the T-ID in the same way as the ADM did in step 1 if the T-ID type is concealed type, or retrieves the T-ID if the T-ID type is stored type. The AIoT device determines it needs to reply to the NG-RAN if the generated or retrieved T-ID matches the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new  T-ID_n+1 as specified in Annex B.1 and stores the new T-ID_n+1 as T-ID_n.

-	In steps 5 and 6, the AIoT Identification Information is not included in the D2R message and the Inventory Report message.

-	In step 7, the AIoT Device Permanent Identifier is used as the AIoT Identification Information.

-	In step 10, if the verification of RESAIOT is successful and if the T-ID type is stored type, then AIOTF shall instruct the ADM to derive a new T-ID (T-ID_n+1) as specified in Annex B.1 and to stores it as T-ID_n in the AIoT Device profile. If the T-ID is to be updated by command, the AIOTF shall also obtain the newly derived T-ID from the ADM.

NOTE 1:	The AIOTF identifies the AIoT Device by checking the received RESAIoT parameter. Therefore, the AIoT Identification Information is not needed in the D2R message and the Inventory Report message.

NOTE 2: 	In case of concealed T-ID type, every AIoT Device that receives an Inventory Request with T-ID needs to perform a T-ID matching by generating a T-ID based on the KAIoT_root and check if the generated T-ID matches the received T-ID. It is assumed that the AIoT Device that receives the Inventory Request has enough energy to perform this T-ID matching in addition to the Inventory procedure specified in clause 5.2.2.

NOTE 3: 	In the case of stored T-ID type, the stored T-IDs on the device side and network side can get out of synch. The handling of such situation is described in clause 5.4.4.

NOTE 4:	When the T-ID is updated using the command procedure, protection of AIoT device identifier privacy is only realized if the Command Request is encrypted.

-	After step 10, if the stored T-ID update is performed via a Command procedure and the AIoT Device supports this, the AIOTF sends an encrypted AIOT NAS Command Request message containing the new T-ID requested from the ADM to the AIoT Device. Upon receipt of the Command Request message, the AIoT Device stores the new T-ID.

Start of 2nd Change

### 5.4.4	Out-of-Synch detection and Resynchronization of T-ID

In case the network does not receive an Inventory Response from an AIoT Device after an Individual Inventory Request, it may occur because that the AIoT Device and the network are out of synch with the T-IDs. The out-of-synch can happen if e.g.:

-	The Inventory Response or Command Response from the Device was lost during transmission due to radio link issues e.g. interference, range, etc. in that case the AIoT Device would generate the T-ID_n+1, but the ADM would not generate the T-ID_n+1 or not know that the device has received the T-ID_n+1 as it did not get any response.

-	Something went wrong during the Inventory or Command procedure e.g., the AIoT Device managed to write to the NVM but not send the inventory response or command response, or the AIoT Device sent the inventory response or command response but was not able to write to the NVM.

If the AIOTF performed Individual Inventory with the stored T-ID and did not receive the response from the AIoT device, the AIOTF invokes Nadm_SecTID_Get service operation with Resynchronization indicator and RANDAIOT_n to indicate T-ID sequence recovery to the ADM, and retrieves the T-ID of the concealed T-ID type from the ADM. Then, the AIOTF performs Individual Inventory to the AIoT device with the concealed T-ID. The AIOTF may use the command message to update the stored type T-ID with a new T-ID in the device. The AIOTF invokes the Nadm_SecTID_Update service operation to update the T-ID in the ADM and the Nadm_SecTID_Get service operation to retrieve this T-ID in the AIOTF. After the command message, the T-IDs in the device and the ADM are in sync.

Start of 3rd Change

## 6.1	Services provided by ADM

### 6.1.1	General

The following table shows the Nadm_Sec service and Service Operations related to AIoT security.

Table 6.1.1-1: List of ADM Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_Sec | RAND_Get | Request/Response | AIOTF |
|  | Authentication_Get | Request/Response | AIOTF |
|  | SessionKey_Get | Request/Response | AIOTF |
|  | TID_Get | Request/Response | AIOTF |
|  | TID_Update | Request/Response | AIOTF |

Start of 4th Change

### 6.1.5	Nadm_SecTID_Get service operation

Service operation name: Nadm_SecTID_Get.

Description: Requester NF gets the Temporary ID (T-ID) for a given AIoT device from ADM.

Input, Required: AIoT Device Permanent ID

Input, Optional: Resynchronization indicator, RANDAIOT_n

Output, Required: T-ID, T-ID handling type.

Start of 5th Change

### 6.1.x	Nadm_SecTID_Update service operation

Service operation name: Nadm_SecTID_Update.

Description: Requester NF initiates the generation of a new Temporary ID (T-IDn+1) for a given AIoT device in the ADM. In the ADM storage, the new generated T-ID_n+1 becomes the new T-ID_n, which is then fetched with the next Nadm_SecTID_Get service operation.

Input, Required: AIoT Device Permanent ID, T-ID, RANDAIOT_n.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

End of Change