3GPP TSG-SA3 Meeting #126	S3-260224r2

# Goa, India, 9 – 13 February 2026

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.369 | CR | 0075 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New T-ID Update Service Operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-SEC |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 5.4.3 “Procedure for AIoT Device Identifier protection with T-ID update during Individual inventory” describes in step 10 that the AIOTF instructs the ADM to derive a new T-ID, but no service operation available to perform this operation. Step 10 is clarified more and a new service operation is introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define a new service opration for the ADM to update the T-ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | T-ID update cannot be performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3, 6.1.1, new clause 6.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of 1st Change

### 5.4.3	Procedure for AIoT Device Identifier protection with T-ID update during Individual inventory

For the protection of AIoT device permanent identifier during the inventory procedure with AIoT Device Identifier described in clause 5.2.2, the following changes shall apply:

-	In step 1, AIOTF shall invoke Nadm_SecTID_Get service operation to retrieve a T-ID in addition to the RANDAIOT_n from ADM using the AIoT Device Permanent Identifier. The ADM shall, based on T-ID handling information stored in the AIoT device profile, either fetch the stored T-ID in the AIoT Device profile or generate the T-ID based on the AIoT Device Permanent Identifier as specified in Annex B.1, and send the T-ID to the AIOTF.

-	In steps 2, 3, and 4, the T-ID shall be used as the AIoT Identification Information.

-	In step 2 the AIOTF includes the T-ID handling information in the Inventory Request message. T-ID handling information includes:

-	T-ID type that can be either a concealed type or a stored type. If the T-ID type is the concealed type, the T-ID is generated based on the AIoT Device Permanent Identifier. If the T-ID is the stored type, the T-ID is generated based on the stored T-ID as specified in Annex B.1.

-	Whether the stored T-ID type is updated with a command via a Command procedure or without a command during step 4.

NOTE A: If the T-ID is of stored type, the initial value of the T-ID is to be computed based on the AIOT Device Permanent ID.

-	In step 3, NG-RAN includes the T-ID handling information in the paging message.

-	In step 4, the AIoT device, based on the T-ID handling information in the paging message, generates the T-ID in the same way as the ADM did in step 1 if the T-ID type is concealed type, or retrieves the T-ID if the T-ID type is stored type. The AIoT device determines it needs to reply to the NG-RAN if the generated or retrieved T-ID matches the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new  T-ID_n+1 as specified in Annex B.1 and stores the new T-ID_n+1.

-	In steps 5 and 6, the AIoT Identification Information is not included in the D2R message and the Inventory Report message.

-	In step 7, the AIoT Device Permanent Identifier is used as the AIoT Identification Information.

-	In step 10, if the verification of RESAIOT is successful and if the T-ID type is stored type, then AIOTF shall instruct the ADM to derive a new T-ID as specified in Annex B.1 and to store it in the AIoT Device profile. If the T-ID is to be updated by command, the AIOTF shall also obtain the newly derived T-ID from the ADM.

NOTE 1:	The AIOTF identifies the AIoT Device by checking the received RESAIoT parameter. Therefore, the AIoT Identification Information is not needed in the D2R message and the Inventory Report message.

NOTE 2: 	In case of concealed T-ID type, every AIoT Device that receives an Inventory Request with T-ID needs to perform a T-ID matching by generating a T-ID based on the KAIoT_root and check if the generated T-ID matches the received T-ID. It is assumed that the AIoT Device that receives the Inventory Request has enough energy to perform this T-ID matching in addition to the Inventory procedure specified in clause 5.2.2.

NOTE 3: 	In the case of stored T-ID type, the stored T-IDs on the device side and network side can get out of synch. The handling of such situation is described in clause 5.4.4.

NOTE 4:	When the T-ID is updated using the command procedure, protection of AIoT device identifier privacy is only realized if the Command Request is encrypted.

-	After step 10, if the stored T-ID update is performed via a Command procedure and the AIoT Device supports this, the AIOTF sends an encrypted AIOT NAS Command Request message containing the new T-ID requested from the ADM to the AIoT Device. Upon receipt of the Command Request message, the AIoT Device stores the new T-ID.

Start of 2nd Change

## 6.1	Services provided by ADM

### 6.1.1	General

The following table shows the Nadm_Sec service and Service Operations related to AIoT security.

Table 6.1.1-1: List of ADM Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_Sec | RAND_Get | Request/Response | AIOTF |
|  | Authentication_Get | Request/Response | AIOTF |
|  | SessionKey_Get | Request/Response | AIOTF |
|  | TID_Get | Request/Response | AIOTF |
|  | TID_Update | Request/Response | AIOTF |

Start of 3rd Change

### 6.1.x	Nadm_SecTID_Update service operation

Service operation name: Nadm_SecTID_Update.

Description: Requester NF initiates the generation of a new Temporary ID (T-IDn+1) for a given AIoT device in the ADM. In the ADM storage, the new generated T-ID_n+1 becomes the new T-ID_n, which is then fetched with the next Nadm_SecTID_Get service operation.

Input, Required: AIoT Device Permanent ID, T-ID, RANDAIOT_n.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

End of Change