3GPP TSG-SA3 Meeting #126	S3-260292-r1

# Goa, India, 9 – 13 February 2026

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 33.369 | CR | 80 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of EN on randomness of RANDAIOT_d |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-SEC |  |  |  |  |  | Date: |  |  | 2026-02-03 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CR proposes to remove the EN as clause 4.2.1.2 already states that the device shall support a method for pseudo-random number bit generation. In step 4, the device shall use that generator to generate the RAND_AIoT_d. TS 33.369 do not further specify the quality of that generator implying its left for implementation. Minor typo corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The EN and typo remains |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS remains questioning the quality of the random number and hereby the trustworthiness of the authentication procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 5	Security procedures for Ambient IoT service

## 5.1	General

This clause describes the security procedures for Ambient IoT service. The requirements can be found in clause 4.

## 5.2	Authentication procedure

### 5.2.1	General

This clause describes the authentication procedure for Ambient IoT devices for both Inventory procedure and Command procedure when authentication is triggered by the network. Device authentication shall always be performed for the Inventory Procedure.

NOTE: KAIOT_root is the long-term key.

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.2-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 for the inventory procedur or clause 6.2.3 for the command procedure in TS 23.369 [2] is performed.

1. AIOTF shall invoke Nadm_SecRAND_Get service operation towards ADM. ADM shall generate and return RANDAIOT_n towards AIOTF.

2. AIOTF shall send an Inventory Request message including RANDAIOT_n in addition to the AIoT Identification Information specified in clause 6.2.2 of TS 23.369 [2]to NG-RAN.

3. NG-RAN shall include RANDAIOT_n in the paging message to the AIoT Device in addition to the AIoT Identification Information.

NOTE 1: An active attack may send a new paging message to the AIoT Device while there is an ongoing procedure in the AIoT Device. The AIoT Device will abort the ongoing procedure and respond to the new paging message. The security measure to such Denial-of-Service attack is not specified in present document.

NOTE 2: While a legitimate network is performing an inventory operation, an attacker may cause amplification of resource exhaustion at the legitimate network side by sending AIoT paging messages for all AIoT Devices or to a large group of AIoT Devices, which causes large number of AIoT Devices sending D2R messages to the legitimate network that the legitimate network does not expect to receive. The security measure to such amplification of resource exhaustion attack is not specified in present document.

4. Upon receiving the paging message, if the AIoT Device determines it needs to respond based on the AIoT Device Identification Information, the AIoT Device shall generate a pseudo-random number RANDAIOT_d, calculate RESAIOT using KAIoT_root, RANDAIOT_n, and RANDAIOT_d (see Annex A.2) for network authenticating the AIoT Device.

5. AIoT Device shall send a D2R message including an AIOT NAS message to the NG-RAN. The AIOT NAS message includes RESAIOT and RANDAIOT_d.

6. NG-RAN shall send an Inventory Report message to AIOTF, including the AIOT NAS message containing RESAIOT and RANDAIOT_d.

7. AIOTF shall invoke Nadm_SecAuthentication_Get service operation with the AIoT Device Identification Information, RANDAIOT_n, and RANDAIOT_d towards ADM.

NOTE 3: The authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), which has load impact to ADM.

8. ADM shall calculate XRESAIOT using the same method as in AIoT Device (see Annex A.2).

9. ADM shall return XRESAIOT and AIoT Device Permanent Identifier if not included in step 7 to AIOTF.

10. AIOTF shal verify RESAIOT.

11. If the verification is successful, for command case, the AIOTF shall invoke Nadm_SecSessionKey_Get service operation with AIoT Device Permanent Identifier, RANDAIOT_n, and RANDAIOT_d towards ADM. ADM shall calculate and return KAIOTF if it receives a request from AIOTF (see AnnexA.3).

The steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continue.

For the command procedure, the AIoT device implicitly authenticates the network via integrity check of the AIOT NAS Command Request message as specified in clause 5.3.2 of present document.

* * * End of Changes * * * *