3GPP TSG-SA3 Meeting #126	draft_S3-260353_r1

# Goa, India, 9 – 13 February2026

Source:	ChinaTelecom, ChinaMobile

Title:	Conclusion to key issue #1 in TR 33.746

Document for:	Approval

Agenda item:	5.2.13

Spec:	TR 33.746

Version:	v0.3.0

Work Item:	FS_NR_Femto_Sec_Ph2

Comments

Provide conclusion to key issue #1 in TR 33.746.

* * * First Change * * * *

7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

7.1	Key Issue #1: Detection of misconfigured/compromised 5G NR Femto devices

The following statements are agreed for key issue #1:

- For the requirements of detection of misconfigured 5G NR Femto nodes:

5G NR Femto supports reporting configuration information for detection and monitoring to the management function in accordance with pre-configured requirements. The Management function is a part of the 5G NR Femto MS.

NOTE 1:	The types of configuration information reported by 5G NR Femto at least include the configuration information listed in table 6.2.2.3-1.

The Management function configures the configuration information collection type to the 5G NR Femto nodes based on the operator’s policies, and then collects the configuration information from the 5G NR Femto nodes and performs detection and monitoring.

Editor’s Note:  Further conclusion is FFS.

* * * End of Changes * * * *