3GPP TSG-SA3 Meeting #126	S3-260604-r1

# Goa, India, 9 – 13 February 2026

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on Backward Compatible Hybrid SUCI Calculation

Document for:	Approval

Agenda item:	5.2.1

Spec:	3GPP TR 33.703

Version:	0.3.0

Work Item:	FS_CryptoPQC

Comments

A new backward compatible solution for SUCI calculation that utilizes the current ECIES-based design and MLKEM.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 33.938: "3GPP Cryptographic Inventory".

[3]	3GPP TS 33.180: "Security of the Mission Critical (MC) service".

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[5]	PQUIP draft-ietf-pquip-pqc-engineers: "Post-Quantum Cryptography for Engineers".

[6]	IETF RFC 6509: ''MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)''.

[7]	IETF RFC 9794: "Terminology for Post-Quantum Traditional Hybrid Schemes".

[8]	NIST IR 8547: "Transition to Post-Quantum Cryptography Standards".

[9]	SECG SEC 1: "Recommended Elliptic Curve Cryptography", Version 2.0, 2009. Available at http://www.secg.org/sec1-v2.pdf.

[10]	SECG SEC 2: "Recommended Elliptic Curve Domain Parameters", Version 2.0, 2010. Available at http://www.secg.org/sec2-v2.pdf.

[11]	EU, Roadmap for the Transition to Post-Quantum Cryptography
https://digital-strategy.ec.europa.eu/en/news/eu-reinforces-its-cybersecurity-post-quantum-cryptography

[12]	UK NCSC, Timelines for migration to post-quantum cryptography
https://www.ncsc.gov.uk/guidance/pqc-migration-timelines

[13]	NSA, The Commercial National Security Algorithm Suite 2.0 and Quantum Computing FAQ
https://media.defense.gov/2022/Sep/07/2003071836/-1/-1/0/CSI_CNSA_2.0_FAQ_.PDF

[14]	ANSSI, Guide des Mécanismes cryptoraphiques
https://cyber.gouv.fr/sites/default/files/2021/03/anssi-guide-mecanismes_crypto-2.04.pdf

[15]	ASD, Guidelines for cryptography
https://www.cyber.gov.au/business-government/asds-cyber-security-frameworks/ism/cybersecurity-guidelines/guidelines-for-cryptography

[16]	Canadian Centre for Cyber Security, Roadmap for the migration to post-quantum cryptography
https://www.cyber.gc.ca/en/guidance/roadmap-migration-post-quantum-cryptography-government-canada-itsm40001

[17]	Swedish NCSC, Kvantsäker kryptografi
https://www.ncsc.se/sv/aktuellt/kvantsaker-kryptografi/

[18]	NSM Cryptographic Recommendations
https://nsm.no/getfile.php/1314334-1742808614/NSM/Filer/Dokumenter/Veiledere/NSM%20Cryptographic%20Recommendations%202025.pdf

[19]	AIVD, The PQC Migration Handbook
https://english.aivd.nl/binaries/aivd-en/documenten/publications/2024/12/3/the-pqc-migration-handbook/The+PQC+Migration+Handbook+.pdf

[20]	3GPP, Release Timeline
https://www.3gpp.org/specifications-technologies/releases/release-20

[21]	NIST FIPS 203: "Module-Lattice-Based Key-Encapsulation Mechanism Standard"
https://doi.org/10.6028/NIST.FIPS.203

[22]	NIST FIPS 204: "Module-Lattice-Based Digital Signature Standard"
https://doi.org/10.6028/NIST.FIPS.204

[23]	NIST FIPS 205: "Stateless Hash-Based Digital Signature Standard"
https://doi.org/10.6028/NIST.FIPS.205

[24]	OpenSSH 10.0 Introduces Default Post-Quantum Key Exchange Algorithm https://quantumcomputingreport.com/openssh-10-0-introduces-default-post-quantum-key-exchange-algorithm

[25]	Cloudflare Radar https://radar.cloudflare.com/adoption-and-usage#post-quantum-encryption-adoption

[26]	A Coordinated Implementation Roadmap for the Transition to Post-Quantum Cryptography https://digital-strategy.ec.europa.eu/en/library/coordinated-implementation-roadmap-transition-post-quantum-cryptography

[27]	NCSC (UK): Next steps in preparing for post-quantum cryptography	https://www.ncsc.gov.uk/whitepaper/next-steps-preparing-for-post-quantum-cryptography

[28]	PQC Transition in France ANSSI Views https://cyber.gouv.fr/sites/default/files/document/pqc-transition-in-france.pdf

[29]	ANSSI plan for post-quantum transition https://pkic.org/events/2023/pqc-conference-amsterdam-nl/pkic-pqcc_jerome-plut_anssi_anssi-plan-for-post-quantum-transition.pdf

[30] 	ETSI TS 103 744: "Quantum-safe Hybrid Key Establishment". https://www.etsi.org/deliver/etsi_ts/103700_103799/103744/01.02.01_60/ts_103744v010201p.pdf

[31] 	FIPS 202: "SHA-3 Standard: Permutation-Based Hash and Extendable-Output Functions". https://nvlpubs.nist.gov/nistpubs/fips/nist.fips.202.pdf

[32] 	SP 800-185: "~SHA-3 Derived Functions: cSHAKE, KMAC, TupleHash, and ParallelHash". https://nvlpubs.nist.gov/nistpubs/fips/nist.fips.202.pdf

[33]	GSMA: "Post Quantum Cryptography – Guidelines for Telecom Use Cases - v2.0"

[34]		IETF RFC 5869 "HMAC-based Extract-and-Expand Key Derivation Function (HKDF)"

[35]	IETF RFC 7748: "Elliptic Curves for Security".

[36]	FN-DSA:  Falcon is a cryptographic signature algorithm submitted to NIST, Refer to https://falcon-sign.info/falcon.pdf

[37]	NIST: “Submission Requirements and Evaluation Criteria for the Post-Quantum Cryptography Standardization Process “,
https://csrc.nist.gov/CSRC/media/Projects/Post-Quantum-Cryptography/documents/call-for-proposals-final-dec-2016.pdf

[38]	Bernstein, D.J. (2009): "Introduction to post-quantum cryptography ", 2009. Available at https://doi.org/10.1007/978-3-540-88702-7_1

[39]	NIST IR 8545: “Status Report on the Fourth Round of the NIST Post-Quantum Cryptography Standardization Process”, 2025. Available at https://csrc.nist.gov/pubs/ir/8545/final

[40] 	NIST, "Considerations for Achieving Cryptographic Agility: Strategies and Practices," CSWP 39, Jul. 2025. [Online]. Available: https://csrc.nist.gov/pubs/cswp/39/considerations-for-achieving-cryptographic-agility/2pd

[41]	IETF RFC 7696: “Guidelines for Cryptographic Algorithm Agility and Selecting Mandatory-to-Implement Algorithms”.

[42]	IETF: “About RFCs”. Available at https://www.ietf.org/process/rfcs/.

[43]	IETF RFC 9242: "Intermediate Exchange in the Internet Key Exchange Protocol Version 2 (IKEv2) "

[44]	IETF RFC 9370: "Multiple Key Exchanges in the Internet Key Exchange Protocol Version 2 (IKEv2) "

[45]	IETF Draft (Standards Track): "Post-quantum Hybrid Key Exchange with ML-KEM in the Internet Key Exchange Protocol Version 2 (IKEv2) ", https://datatracker.ietf.org/doc/draft-ietf-ipsecme-ikev2-mlkem/.

[46]	IETF RFC 9593: "Announcing Supported Authentication Methods in the Internet Key Exchange Protocol Version 2 (IKEv2)"

[47]	IETF RFC 8784: "Mixing Preshared Keys in the Internet Key Exchange Protocol Version 2 (IKEv2) for Post-quantum Security"

[48]	IETF Draft (Standards Track): " Signature Authentication in the Internet Key Exchange Version 2 (IKEv2) using PQC ", https://datatracker.ietf.org/doc/draft-ietf-ipsecme-ikev2-pqc-auth/.

[49]	IETF RFC 7383: "Internet Key Exchange Protocol Version 2 (IKEv2) Message Fragmentation". https://www.rfc-editor.org/rfc/rfc7383

[50]	IETF RFC 9763: "Related Certificates for Use in Multiple Authentications within a Protocol "

[51]	IETF RFC 9802: "Use of the HSS and XMSS Hash-Based Signature Algorithms in Internet X.509 Public Key Infrastructure"

[52]	IETF Draft (Standards Track): "Internet X.509 Public Key Infrastructure - Algorithm Identifiers for the Module-Lattice-Based Key-Encapsulation Mechanism (ML-KEM) ", https://datatracker.ietf.org/doc/draft-ietf-lamps-kyber-certificates/.

[53]	IETF Draft (Standards Track): "Internet X.509 Public Key Infrastructure: Algorithm Identifiers for SLH-DSA", https://datatracker.ietf.org/doc/draft-ietf-lamps-x509-slhdsa/.

[54]	IETF Draft (Standards Track): "Internet X.509 Public Key Infrastructure - Algorithm Identifiers for the Module-Lattice-Based Digital Signature Algorithm (ML-DSA)", https://datatracker.ietf.org/doc/draft-ietf-lamps-dilithium-certificates/https://datatracker.ietf.org/doc/draft-ietf-lamps-x509-slhdsa/.

[55]	IETF Draft (Standards Track): "Composite ML-KEM for use in X.509 Public Key Infrastructure", https://datatracker.ietf.org/doc/draft-ietf-lamps-pq-composite-kem/https://datatracker.ietf.org/doc/draft-ietf-lamps-x509-slhdsa/.

[56]	IETF Draft (Standards Track): "A Mechanism for X.509 Certificate Discovery", https://datatracker.ietf.org/doc/draft-ietf-lamps-certdiscovery/https://datatracker.ietf.org/doc/draft-ietf-lamps-x509-slhdsa/.

[57]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2"

[58]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3"

[59]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security"

[60]	IETF Draft draft-ietf-tls-tls12-frozen-08: "TLS 1.2 is in Feature Freeze "

[61]	https://datatracker.ietf.org/meeting/123/materials/slides-123-tls-wg-status-00

[62]	https://datatracker.ietf.org/liaison/2058/

[63]	IETF Draft draft-ietf-tls-hybrid-design-16: "Hybrid key exchange in TLS 1.3". https://datatracker.ietf.org/doc/draft-ietf-tls-hybrid-design/.

[64]	IETF Draft draft-ietf-tls-mlkem-04: "ML-KEM Post-Quantum Key Agreement for TLS 1.3". https://datatracker.ietf.org/doc/draft-ietf-tls-mlkem/.

[65]	IETF Draft draft-ietf-tls-ecdhe-mlkem-01: "Post-quantum hybrid ECDHE-MLKEM Key Agreement for TLSv1.3". https://datatracker.ietf.org/doc/draft-ietf-tls-ecdhe-mlkem/.

[66]	IETF Draft draft-ietf-tls-mldsa-01: "Use of ML-DSA in TLS 1.3", https://datatracker.ietf.org/doc/draft-ietf-tls-mldsa/

[67]	IETF Draft draft-ietf-jose-pqc-kem-03: "Post-Quantum Key Encapsulation Mechanisms (PQ KEMs) for JOSE and COSE"

[68]	IETF Draft draft-ietf-cose-dilithium-08: "ML-DSA for JOSE and COSE"

[69]	IETF Draft draft-ietf-cose-sphincs-plus-05: "SLH-DSA for JOSE and COSE"

[70]	IETF Draft draft-ietf-cose-falcon-01: "JOSE and COSE Encoding for Falcon"

[71]	IETF Draft (Standards Track): “Use of Hybrid Public Key Encryption (HPKE) with JSON Object Signing and Encryption (JOSE)”, https://datatracker.ietf.org/doc/draft-ietf-jose-hpke-encrypt/.

[72]	IETF Draft (Standards Track): “Use of Hybrid Public-Key Encryption (HPKE) with CBOR Object Signing and Encryption (COSE)”, https://datatracker.ietf.org/doc/draft-ietf-cose-hpke/.

[73]	NIST SP 800-227 Recommendations for Key-Encapsulation Mechanisms, url: https://csrc.nist.gov/pubs/sp/800/227/ipd

[74]	3GPP TS 23.003: "Numbering, addressing and identification".

[75]	NIST.SP.800-56 Recommendation for Pair-Wise Key-Establishment Schemes Using Discrete Logarithm Cryptography. url: https://nvlpubs.nist.gov/nistpubs/SpecialPublications/NIST.SP.800-56Ar3.pdf

[76]	Galois Counter Mode with Strong Secure Tags (GCM-SST). https://datatracker.ietf.org/doc/html/draft-mattsson-cfrg-aes-gcm-sst

[77] 	Ericssons comments on NIST SP 800-227 (Initial Public Draft). https://csrc.nist.gov/files/pubs/sp/800/227/ipd/docs/sp800-227-ipd-public-comments-received.pdf

[78]	IETF Draft (Standards Track): " Mixing Preshared Keys in the IKE_INTERMEDIATE and in the CREATE_CHILD_SA Exchanges of IKEv2 for Post-quantum Security", https://datatracker.ietf.org/doc/draft-ietf-ipsecme-ikev2-qr-alt/.

[79] 	BSI: "Cryptographic Mechanisms", https://www.bsi.bund.de/EN/Themen/Unternehmen-und-Organisationen/Standards-und-Zertifizierung/Technische-Richtlinien/TR-nach-Thema-sortiert/tr02102/tr02102_node.html

[80]	IETF RFC 7296: " Internet Key Exchange Protocol Version 2 (IKEv2)".

[81]		IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".[82]	3GPP TS 33.310: " Network Domain Security (NDS); Authentication Framework (AF)".

[83]	IETF RFC 7515: "JSON Web Signature (JWS)".

[84]	IETF RFC 7516: "JSON Web Encryption".

[85]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”[86]	NIST SP-800-90A

[x1]		Jonathan Katz and Yehuda Lindell: Introduction to Modern Cryptography, 3rd edition, CRC Press, 2020. https://doi.org/10.1201/9781351133036

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AE			Authenticated (Symmetric) Encryption

AEAD			Authenticated Encryption with Associated Data

ANSSI			Agence Nationale de la Sécurité des Systèmes d'Information

CA			Certification Authority

CBOR			Concise Binary Object Representation

COSE			CBOR Object Signing and Encryption

CRL			Certificate Revocation Lists

CRQC			Cryptographically Relevant Quantum Computer

DSA			Digital Signature Algorithm

ECC			Elliptic Curve Cryptography

ECDH			Elliptic Curve Diffie–Hellman key Exchange

ECIES			Elliptic Curve Integrated Encryption Scheme

FN-DSA			Fast-Fourier Transform over NTRU-Lattice-Based DSA

GCI			Global Cable Identifier

GLI			Global Line Identifier

HBS			Hash-Based Signature

HN			Home Network

HQC			Hamming Quasi-Cyclic

HSS			Hierarchical Signature System

IKEv2			Internet Key Exchange Protocol Version 2

IMSI			International Mobile Subscriber Identifier

IND-CCA2			Indistinguishability under Adaptive Chosen Ciphertext Attack

JOSE			Javascript Object Signing and Encryption

JSON			JavaScript Object Notation

JWE			JSON Web Encryption

JWS			JSON Web Signature

KEM			Key Encapsulation Mechanism

MAC			Message Authentication Code

MCC			Mobile Country Code

LTS			Long Term Stable

MIKEY-SAKKE			Multimedia Internet KEYing – Sakai-Kasahara Key Encryption

ML-DSA			Module-Lattice-Based DSA

ML-KEM			Module Lattice-Based Key-Encapsulation Mechanism

MNC			Mobile Network Code

NAI			Network Access Identifier

NCSC 			National Cyber Security Centre

NSA			National Security Agency

NSI			Network Specific Identifier

NSM			National Security Memorandum

NTRU			Nth-degree Truncated Polynomial Ring Units

OCSP			Online Certificate Status Protocol

PKI			Public Key Infrastructure

PKIX			Public Key Infrastructure X.509

PLMN			Public Land Mobile Network

PQC			Post-Quantum Cryptography

PRNG			Pseudo Random Number Generator

SA			Security Association

SDO			Standards Development Organizations

SECG			Security Engineering & Consulting Group

SLH-DSA			Stateless Hash-Based DSA

SUCI			Subscription Concealed Identifier

SUPI			Subscription Permanent Identifier

TLS 1.2			Transport Layer Security Version 1.2

TLS 1.3			Transport Layer Security Version 1.3

UDM			Unified Data Management

UE			User Equipment

XMSS			eXtended Merkle Signature Scheme

* * * Next Change * * * *

#### 7.2.1.x	Solution #x to SUCI calculation: Backward Compatible Hybrid SUCI Calculation

Editor’s Note: Evaluation on impact of initial access due to increased length of SUCI is FFS.

Editor’s Note: Evaluation on computing overhead of SUCI calculation on both UE and network side is FFS.

Editor’s Note: Whether the solution work for case that user does not update USIM card is FFS.

Editor’s Note: Whether this solution meets IND-CCA-1 security and IND-CCA-2 security is FFS.

##### 7.2.1.x.1 	Introduction

This is a hybrid solution for SUCI calculation that combines the current Elliptic Curve Integrated Encryption Scheme (ECIES) based design with Module Lattice-Based Key-Encapsulation Mechanism (ML-KEM). This solution also uses an Authenticated (Symmetric) Encryption (AE). This solution is backward compatible and has a modular design such that once Cryptographically Relevant Quantum Computer (CRQC) becomes readily available the ECIES parts can be simply discarded without any major design/implementation changes. This solution doesn’t suffer from the Indistinguishability under Adaptive Chosen Ciphertext Attack (IND-CCA2) [x1] security weakness that other solution(s) may have. More specifically, this solution is:

-	IND-CCA2 secure against an adversary with access to CRQC if the PQC algorithm used doesn’t have any known vulnerability.

-	IND-CCA2 secure against a regular adversary (without access to CRQC) if the PQC algorithm used has some vulnerabilities, i.e., on par with the current SUCI calculation.

##### 7.2.1.x.2 	Solution details

As is currently done in TS 33.501, the public key of the Home Network (HN) is securely provisioned to the User Equipment (UE). Since this hybrid solution combines ECIES with ML-KEM, the public key of HN in this solution which is used for encrypting Subscription Permanent Identifier (SUPI), consists of two keys: ECIES public key shown in the figures as “Public key of HN” and ML-KEM encapsulation key denoted as ekM. Similarly, the private key of HN in this solution which is used for decrypting Subscription Concealed Identifier (SUCI), consists of two keys: ECIES private key shown in the figures as “Private key of HN” and ML-KEM decapsulation key denoted as dkM.

Note: To keep the presentation simple, here and in the rest of this solution SUPI is used to mean the subscription identifier part of the SUPI, what is referred in Annex C of TS 33.501 as the scheme-input. Similarly, SUCI refers to the part of SUCI that is cryptographically computed, what is referred in Annex C of TS 33.501 as the scheme-output.

###### 7.2.1.x.2.1 	Processing on UE Side

The processing at UE is illustrated in Figure 7.2.1.x.2.1-1.

For the first part (PQC part) of this solution the inputs are: ML-KEM encapsulation key ekM and SUPI as the message m. For the second part (3GPP TS 33.501) of this solution, everything is exactly as per Annex C.3.2 of 3GPP TS 33.501 except the Plaintext block, where SUPI is replaced with the two ciphertexts obtained in the PQC part: encrypted symmetric key c1 and encrypted SUPI c2. The final output of the second part is also the final output of this solution, i.e., the receiver (i.e., HN) in this solution doesn’t receive (c1, c2).

Input consists of SUPI denoted as m, the “Public key of HN”, and the ML-KEM encapsulation key ekM.

Output is SUCI, the final output of the second part (3GPP TS 33.501), where:

-	The ML-KEM encapsulation key ekM is used in the ML-KEM encapsulation algorithm to generate a symmetric key denoted as sk and the encapsulation of sk denoted as c1, i.e., (sk, c1)  ML-KEM Encapsulation (ekM).

-	The ML-KEM symmetric key sk generated above is used in the AE encryption algorithm to encrypt SUPI to obtain an intermediate ciphertext denoted as c2, i.e., c2  AE Encryption (sk, m).

-	The ML-KEM encapsulated symmetric key c1 and the intermediate ciphertext c2 generated above are used as the Plaintext block for the second part of this solution to obtain SUCI.

Figure 7.2.1.x.2.1-1: SUPI encryption

###### 7.2.1.x.2.2 	Processing on HN Side

The processing at HN is illustrated in Figure 7.2.1.x.2.1-2.

For the decryption in this solution, first SUCI is decrypted as per Annex C.3.3 of 3GPP TS 33.501 to obtain two ciphertexts: encrypted symmetric key c1 and intermediate ciphertext (encrypted SUPI) c2. For the PQC part of this solution, the decryption key is the ML-KEM decapsulation key, dkM.

Input consists of the ciphertext SUCI, the “Private key of HN”, and the ML-KEM decapsulation key dkM

Output is the SUPI denoted as m, where:

- 	The “Private key of HN” is used to decrypt SUCI as per Annex C.3.3 of 3GPP TS 33.501 to obtain (c1, c2).

- 	The ML-KEM decapsulation key dkM is then used in the ML-KEM decapsulation algorithm to decapsulate the encapsulated symmetric key c1 generated above to obtain the symmetric key sk, i.e., sk  ML-KEM Decapsulation (dkM, c1).

-	Finally, the symmetric key sk generated above is used in the AE decryption algorithm to decrypt the intermediate ciphertext c2 generated above to obtain SUPI denoted as m, i.e., m  AE Decryption (sk, c2).

Figure 7.2.1.x.2.1-2: SUCI decryption

##### 7.2.1.x.3	Evaluation

* * * End of Changes * * * *