3GPP TSG-SA3 Meeting #126	S3-260653-r1

# Goa, India, 9 – 13 February 2026

Source:	BSI (DE)

Title:	pCR on TS 33.117 security event logging

Document for:	Approval

Agenda item:	5.1.3

Spec:	3GPP TS 33.117

Version:	19.2.0

Work Item:	SCAS_5GA

Comments

There already is a „Length of session“ event data to be logged, but with only logging login attempts one cannot determine session length. The logout event is also needed.

Logging the logout is industry best practice as recommended or demanded by, e.g. NIST SP 800-12, CIS Security Logging Standard, ISO 27001, OWASP.

* * * First Change * * * *

##### 4.2.3.6.1	Security event logging

Requirement Name: Security event logging

Requirement Reference: In accordance with industry best practice

Requirement Description: Security events shall be logged together with a unique system reference (e.g. host name, IP or MAC address) and the exact time the incident occurred. For each security event, the log entry shall include user name and/or timestamp and/or performed action and/or result and/or length of session and/or values exceeded and/or value reached.

IETF RFC 3871 [3], section 2.11.10 specifies the minimum set of security events. Each vendor shall document what security events the product logs so that it can be verified by testing.

In particular, it shall be possible to log the following events (which are intended to be supported by the network product and which can be enabled by default at manufacturing time or at a later time by the network operator):

| EventTypes | Description | Event data to be logged |
| --- | --- | --- |
| Incorrect login attempts | Records any user incorrect login attempts to the network product | • Username, • Source (IP address) if remote access • Timestamp |
| Administrator session | Records any session related activities (login, logout, etc.) from accounts that have system privileges. | • Username, • Timestamp, • Length of session, • Source (IP address) if remote access |
| Account administration | Records all account administration activity, i.e. configure, delete, enable, and disable. | • Administrator username, • Administered account, • Activity performed (configure, delete, enable and disable) • Timestamp |
| Resource Usage | Records events that have been triggered when system parameter values such as disk space, CPU load over a longer period have exceeded their defined thresholds. | • Value exceeded, • Value reached (Here suitable threshold values shall be defined depending on the individual system.) • Timestamp |
| Configuration change | Changes to configuration of the network device | • Change made • Username |
| Reboot/shutdown/crash | This event records any action on the network device that forces a reboot or shutdown OR where the network device has crashed. | • Action performed (reboot, shutdown, etc.) • Username (for intentional actions) • Timestamp |
| Interface status change | Change to the status of interfaces on the network device (e.g. shutdown) | • Interface name and type • Status (shutdown, missing link, etc.) • Timestamp |
| Sensitive data access | Records any access attempts to sensitive data | • Action performed (read, write, delete, etc.) • Username • Timestamp |

In addition, optionally it shall be possible to log also the following event (if supported):

| EventTypes | Description | Event data to be logged |
| --- | --- | --- |
| Change of group membership or accounts | Any change of group membership for accounts | • Administrator username, • Administered account, • Activity performed (group added or removed) • Timestamp. |

Threat References: TR 33.926 [4], clause 5.3.4.4, Log Tampering

Test case:

Test Name: TC_SECURITY_EVENT_LOGGING

Purpose:

To verify that the network product correctly logs all required security event types.

Pre-Conditions:

-	The following information shall be provided by the documentation accompanying the network product:

-	The log where the event is recorded and how it can be accessed (e.g. the complete path).

-	If the event type is enabled by default or how to enable it.

-	What O&M services can be used on the Network Product in the configuration according to the pre-requisites for testing in clause 4.1 and how to use them.

-	Whether and how sensitive data is stored on the Network Product (e.g., directory services databases)

-	The tester has the needed administrative privileges to perform the tests

-	If needed for testing specific O&M services, a tester machine is available.

Execution Steps

For each O&M service perform the following test steps

1.-	The Tester sequentially triggers each security event listed in the requirement, while covering each option detailed in the individual security event descriptions.

2.-	The Tester verifies whether the security events, and their individual options, were correctly logged. In particular it is verified whether they include at least the event data specified as required to be logged.

Expected Results:

All security events are appropriately logged, including all required event data.

Expected format of evidence:

-	List of O&M services

-	Commands executed per O&M services

-	The relevant parts of the logs in appropriate form (e.g. file, screenshot)

* * * End of Changes * * * *