3GPP TSG-SA3 Meeting #126	S3-260660

- Goa, India, 9 – 13 February 2026

Source:	BSI (DE)

Title:	pCR on TR 33.730 completion of trusted image repo test case

Document for:	Approval

Agenda item:	5.2.8

Spec:	3GPP TR 33.730

Version:	0.3.0

Work Item:	FS_SCAS_CP

Comments

Completion of „Using trusted image repositories for container image handling“ test case.

* * * First Change * * * *

6.1.1.3	Using trusted image repositories for container image handling

Requirement Name: Securing container function source by using trusted image repositories

Requirement Description:

The containerized NF shall use trusted/private source image repositories while building the container image.

Test Name: TC_SECURE_CONTAINER_IMAGE_REPOSITORIES

Purpose:

Ensure that containers are built using trusted image bases. Images coming from untrusted/public source code repositories (e.g., Public-DockerHub) shall not be used due to risk factors.

-	HTTPS protocol for accessing internal repositories shall be used.

-	Trust level of image content shall be checked to ensure source and integrity of the image.

Pre-Conditions:

-	Vendor documentation describing the container build procedure, configuration of trusted repositories and image integrity verification

Execution Steps:

- The tester reviews the documentation provided by the vendor describing the container build procedure and listing trusted image repositories.

- The tester verifies that the build procedure enforces image integrity verification using at least cryptographic verification, by performing a build with a valid image and with a tampered image.

- For both static and dynamically built containers, the tester reviews the build files (e.g., Dockerfile, CI/CD pipeline scripts, OCI) to verify the image sources specified are only trusted repositories and there are no references to public or untrusted repositories (e.g., Public-DockerHub).

- The tester verifies the image repositories referenced in the build files are accessed via HTTPS.

Expected Results:

-	The vendor documetation contains a description of the build procedure, how to configure trusted repositories and how to setup image integrity verification.

-	The build procedure enforces image integrity verification. The valid image is used by the build procedure, while the tampered image gets rejected.

-	The image sources consist only of trusted repositories accessed via HTTPS.

Expected format of evidence:

-	Snapshots of the configuration or documentation.

-	Logs of the build procedures

-	Logs or network traces/pcaps showing the build procedures use only trusted repositories over HTTPS

* * * End of Changes * * * *