3GPP TSG-SA3 Meeting #126	draft_S3-260686-r2

# Goa, India, 9 – 13 February 2026

Source:	Ericsson

Title:	Pseudo-CR on new KI on authenticated encryption

Document for:	Approval

Agenda item:	5.3.2

Spec:	3GPP TR 33.771

Version:	0.2.0

Work Item:	FS_AEAD

Comments

This document proposes to add a key issue to the study in TR 33.771 about authenticated encryption.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 35.240 Specification of the Snow 5G based 256-bits algorithm set: specification of the 256-NEA4 encryption, the 256-NIA4 integrity, and the 256-NCA4 authenticated encryption algorithm for 5G; Document 1: algorithm specification

[3]	3GPP TS 35.243 Specification of the AES based 256-bits algorithm set: Specification of the 256-NEA5 encryption, the 256-NIA5 integrity, and the 256-NCA5 authenticated encryption algorithm for 5G; Document 1: algorithm specification

[4]	3GPP TS 35.246 Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: algorithm specification

[5]	3GPP TS 33.501: “Security architecture and procedures for 5G System”.

[6]	RFC 5116, “Authenticated Encryption with Associated Data”

[7]	3GPP TR 33.801-01: “Study on Security for the 6G System”.

[X1]		Breaking LTE on Layer Two | Montsecure

* * * Next Change * * * *

## 5.X	Key issue #X: Authenticated encryption

### 5.X.1	Key issue details

#### 5.X.1.1	General

The key issue is to analyze the potential use of mandatory integrity protection for encrypted data in order to utilize the security advantage of authenticated encryption schemes which introduce negligible computational overhead for the integrity protection.

#### 5.X.1.2	Mandatory integrity protection for encrypted data

The aLTEr attack on 4G [X1] made the importance of integrity protection for UP clear even to the general public. In 5G, integrity protection was made possible. For 6G it needs to be evaluated whether integrity protection for encrypted UP ought to be mandatory rather than optional.

The main arguments for having no or optional integrity protection of encrypted UP has in the past been related to the complexity of mounting practical attacks on the air-interface, and to the reduction of overhead and latency for real-time applications. Today, the threat environment is different and technology for mounting MITM attacks on the air-interface is available to a larger extent. Furthermore, it is unclear whether, or to what extent, real-time applications would be affected by UP integrity protection. In particular, in 5G, lower-layer error correction, detection and retransmission schemes ensure that the integrity protection function operates on virtually error-free data. That is, if retransmission due to bit errors was a problem, it would already exist due to lower-layer retransmissions regardless of if integrity protection was added above.

#### 5.X.1.3	Achieving authentication encryption using different mechanisms

Authenticated encryption as a property can be achieved by simultaneous use of integrity protection (message authentication) and encryption algorithms. In 5G, authenticated encryption is achieved in PDCP and NAS protocols by invoking different combinations of the standalone algorithms.

AEAD algorithms provide authenticated encryption in a single invocation. In fact, when using an AEAD algorithm,

it is possible to get "authentication only" but not "encryption only".

### 5.X.2	Security threat

Without the use of authenticated encryption, there are attacks possible such as the one described in the aLTEr [X1].

### 5.X.3	Potential requirements

To ensure 6G withstands the attacks possible against previous generations and resulting negative press such as aLTEr [X1], integrity protection for encrypted data should be considered for air-interface encrypted UP traffic to have authenticated encryption security benefits.

Potential use cases where encryption is applied but authenticated encryption is not acceptable should be identified i.e. if there is need for an encryption-only mode.

NOTE:	Solutions to this key issue are expected to provide analysis rather than call flows.

In case there is need for encryption only mode, it should be studied how this can co-exist with combined mode algorithms (NCA-4/5/6).

* * * End of Changes * * * *