3GPP TSG-SA3 Meeting #126	draft_S3-260757-r1

Goa, India, 9 – 13 February 2026                                         Merger of S3-260382 and 	S3‑260266

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.369 | CR | 0088 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the input parameter of enc and int keys |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-SEC |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When deriving a KCommand_enc or KCommand_int from KAIOTF, the input parameters (i.e, FC value and algorithm identity) are same. KCommand_enc and KCommand_int would be the same, which is not best practice for security. It is proposed to add new input (i.e., algorithm type distinguisher) to derive KCommand_enc and KCommand_int with different value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding algorithm type distinguisher as input to deriving a KCommand_enc and KCommand_int. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KCommand_enc and KCommand_int are same. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

A.4	KCommand_enc and KCommand_int derivation function

When deriving a 128-bit KCommand_enc or 128-bit KCommand_int from KAIOTF, the following parameters shall be used to form the input S to the  KDF:

-	FC = 0x91,

-	P0 = algorithm type distinguisher as specified in Table A.4-1.

-	L0 = length of algorithm type distinguisher (i.e. 0x00 0x01)

Table A.4-1: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| Encryption algorithm | 0x01 |
| Integrity algorithm | 0x02 |

The input key KEY shall be the KAIOTF.

The KCommand_enc / KCommand_int is identified with the 128 least significant bits of the output of the KDF.

* * * End of Changes * * * *