3GPP TSG-SA3 Meeting #126	S3-260761

Goa, India, 9 – 13 February 2026	(revision of S3-260548)

# Source:	Ericsson

# Title:	New WID on Post-quantum Cryptography Migration

# Document for:	Agreement

# Agenda Item:	6.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Post-quantum Cryptography Migration

Acronym: PQCMigration

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X | X | X |  |
| No |  |  |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_CryptoPQC | SA3 | 1080045 | Study on Transitioning to Post Quantum Cryptography in 3GPP |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

Countries and agencies around the world are generally aligned on the need to migrate to Post-Quantum Cryptography (PQC). The common recommendation is to complete migration for high priority systems by around 2030 and for all systems by approximately 2035. Examples of government-issued PQC migration timelines can be found in [1-11].  Whether a system is high priority or not is determined by a variety of factors such as how long the data needs to remain confidentiality protected and what level of risk is the data owner willing to bear. Some parts of telecommunications systems may be assessed by the network operator to be of high priority. More generally, there is a general consensus that the priority is to address the “store now decrypt later” attacks, which calls for upgrading cryptographic mechanisms ensuring confidentiality to achieve quantum resistance.

On the contrary, the migration of signature-based authentication in protocols such as TLS and IPsec is typically not prioritized for transition until 2035, because authentication is not at risk before the advent of CRQCs. Moreover, there is no consensus in this case on the way forward, with on-going discussions on the need to use hybrid certificates and the way to implement them concretely.

This divergence is clearly reflected by the status of IETF standards and existing real-world deployments. For example, while the draft for supporting hybrid ECDHE-MLKEM Key Agreement in TLS 1.3 [12] is set to become a standard very soon and is already massively implemented (e.g. in most common web browsers), standardisation of post-quantum signatures for TLS 1.3 clearly lags behind. This calls for separating the strategy related to confidentiality from the one related to authentication. While the former must prioritized to meet the 2030 deadline, the latter could be delayed, to allow time for standardisation (e.g. at IETF) but also clarification from governmental agencies.

For  PQC adoption in deployed systems, it is essential that standards are updated, and stable implementations are made available well in advance of those deployment milestones. The timelines for different stakeholders in the ecosystem, such as standards development organizations (SDO), equipment vendors, and operators deploying the systems are inherently different. Standards bodies need to finalize specifications early, vendors need sufficient lead time to implement, test, and certify solutions, and only then can large-scale deployments take place.

3GPP Rel-20 TSG RAN and CT Stage 3 specification is expected to be frozen in the mid-2027 [12] — while SA Stage 2 is expected to be frozen by December 2026 [12]. Rel-21 specification can be expected to be completed in the beginning of 2029 at the earliest. It should be considered that some vendors and operators require to meet the 2030 migration timeline for high priority systems.

References:

[1]	NIST IR 8547: "Transition to Post-Quantum Cryptography Standards".

[2]	EU, Roadmap for the Transition to Post-Quantum Cryptography
https://digital-strategy.ec.europa.eu/en/news/eu-reinforces-its-cybersecurity-post-quantum-cryptography

[3]	UK NCSC, Timelines for migration to post-quantum cryptography
https://www.ncsc.gov.uk/guidance/pqc-migration-timelines

[4]	NSA, The Commercial National Security Algorithm Suite 2.0 and Quantum Computing FAQ
https://media.defense.gov/2022/Sep/07/2003071836/-1/-1/0/CSI_CNSA_2.0_FAQ_.PDF

[5]	ANSSI, Guide des Mécanismes cryptoraphiques
https://cyber.gouv.fr/sites/default/files/2021/03/anssi-guide-mecanismes_crypto-2.04.pdf

[6]	ASD, Guidelines for cryptography
https://www.cyber.gov.au/business-government/asds-cyber-security-frameworks/ism/cybersecurity-guidelines/guidelines-for-cryptography

[7]	Canadian Centre for Cyber Security, Roadmap for the migration to post-quantum cryptography
https://www.cyber.gc.ca/en/guidance/roadmap-migration-post-quantum-cryptography-government-canada-itsm40001

[8]	Swedish NCSC, Kvantsäker kryptografi
https://www.ncsc.se/sv/aktuellt/kvantsaker-kryptografi/

[9]	NSM Cryptographic Recommendations
https://nsm.no/getfile.php/1314334-1742808614/NSM/Filer/Dokumenter/Veiledere/NSM%20Cryptographic%20Recommendations%202025.pdf

[10]	AIVD, The PQC Migration Handbook
https://english.aivd.nl/binaries/aivd-en/documenten/publications/2024/12/3/the-pqc-migration-handbook/The+PQC+Migration+Handbook+.pdf

[11]	https://cyber.gouv.fr/enjeux-technologiques/cryptographie-post-quantique/faq-pqc/),

[11]	3GPP, Release 20 – Milestones and endorsed deadlines, https://www.3gpp.org/specifications-technologies/releases/release-20

[12] 	https://datatracker.ietf.org/doc/draft-ietf-tls-ecdhe-mlkem/

# 4	Objective

The objectives of this work item are outlined in the following work tasks:

- WT#1: Introduce PQC algorithms to ensure confidentiality (i.e. ECDHE-MLKEM key exchange)in the 3GPP cryptographic profiles of (D)TLS 1.3, IKEv2, JWE, JWS, Public Key Certificate, CRL, and OSCP.

- WT#2: Add, in the cryptographic profile of TLS 1.2, a note about the threat of Cryptographically Relevant Quantum Computers (CRQCs) and a recommendation strongly discouraging the use of TLS 1.2.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 |  | 1.5 | Yes |  |
| WT#2 |  | 0.5 | Yes |  |
| WT#3 |  | 1 | No |  |

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.210 | Introduce PQC algorithms for confidentiality (i.e. ECDHE-MLKEM key exchange) in cryptographic profiles | SA#113 (Sept 2026) |  |
| TS 33.310 | Introduce PQC algorithms for confidentiality (i.e. ECDHE-MLKEM key exchange) in cryptographic profiles | SA#113 (Sept 2026) |  |
| TS 33.501 | Introduce ECIES profile(s) for computing quantum-resistant SUCIs | SA#113 (Sept 2026) |  |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

Not identified yet

# 9	Supporting Individual Members

Supporting IM name

Ericsson