3GPP TSG-SA3 Meeting #126	S3-260761-r8

Goa, India, 9 – 13 February 2026	(revision of S3-260548)

# Source:	Ericsson

# Title:	New WID on Post-quantum Cryptography Migration

# Document for:	Agreement

# Agenda Item:	6.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Post-quantum Cryptography Migration

Acronym: PQCMigration

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_CryptoPQC | SA3 | 1080045 | Study on Transitioning to Post Quantum Cryptography in 3GPP |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

Countries and agencies around the world are generally aligned on the need to migrate to Post-Quantum Cryptography (PQC). The common recommendation is to complete migration for high priority systems by around 2030 and for all systems by approximately 2035. Examples of government-issued PQC migration timelines can be found in [1-11].  Whether a system is high priority or not is determined by a variety of factors such as how long protection is needed as well as what level of risk is the data owner willing to bear. Some parts of telecommunications systems may be assessed by the network operator to be of high priority.

According to “A Coordinated Implementation Roadmap for the Transition to Post-Quantum Cryptography” [2] by European Commission, two threat scenarios are currently of concern:

- The “store now, decrypt later” scenario, where adversaries store encrypted data for decryption once a cryptographically relevant quantum computer emerges.

- Long transition periods, which occur for complex systems such as public-key infrastructures (PKIs) or devices with a long lifetime. Even if a system is not affected by ongoing attacks, as in the first scenario, there is a risk that the transition to quantum safe cryptography might not be completed in time, potentially compromising the confidentiality and authenticity of all communications

The end goal for mitigating the CRQC threat to TLS would be to only allow TLS 1.3 in all usage of TLS. This end goal can be accomplished for all network elements by requiring TLS 1.3 clients and servers to disable fallback/downgrade to TLS 1.2 and disabling all TLS 1.2 clients and TLS 1.2 servers. An operator might need a phased approach to allow a network element to negotiate TLS 1.2 for some uses while preserving TLS 1.3 for other uses. It needs to be up to the operator to determine whether a network element is allowed to negotiate TLS 1.2 for those certain uses.

For  PQC adoption in deployed systems, it is essential that standards are updated, and stable implementations are made available well in advance of those deployment milestones. The timelines for different stakeholders in the ecosystem, such as standards development organizations (SDO), equipment vendors, and operators deploying the systems are inherently different. Standards bodies need to finalize specifications early, vendors need sufficient lead time to implement, test, and certify solutions, and only then can large-scale deployments take place.

3GPP Rel-20 TSG RAN and CT Stage 3 specification is expected to be frozen in the mid-2027 [12] — while SA Stage 2 is expected to be frozen by December 2026 [12]. Rel-21 specification can be expected to be completed in the beginning of 2029 at the earliest. It should be considered that some vendors and operators require to meet the 2030 migration timeline for high priority systems.

References:

[1]	NIST IR 8547: "Transition to Post-Quantum Cryptography Standards".

[2]	EU, Roadmap for the Transition to Post-Quantum Cryptography
https://digital-strategy.ec.europa.eu/en/news/eu-reinforces-its-cybersecurity-post-quantum-cryptography

[3]	UK NCSC, Timelines for migration to post-quantum cryptography
https://www.ncsc.gov.uk/guidance/pqc-migration-timelines

[4]	NSA, The Commercial National Security Algorithm Suite 2.0 and Quantum Computing FAQ
https://media.defense.gov/2022/Sep/07/2003071836/-1/-1/0/CSI_CNSA_2.0_FAQ_.PDF

[5]	ANSSI, Guide des Mécanismes cryptoraphiques
https://cyber.gouv.fr/sites/default/files/2021/03/anssi-guide-mecanismes_crypto-2.04.pdf

[6]	ASD, Guidelines for cryptography
https://www.cyber.gov.au/business-government/asds-cyber-security-frameworks/ism/cybersecurity-guidelines/guidelines-for-cryptography

[7]	Canadian Centre for Cyber Security, Roadmap for the migration to post-quantum cryptography
https://www.cyber.gc.ca/en/guidance/roadmap-migration-post-quantum-cryptography-government-canada-itsm40001

[8]	Swedish NCSC, Kvantsäker kryptografi
https://www.ncsc.se/sv/aktuellt/kvantsaker-kryptografi/

[9]	NSM Cryptographic Recommendations
https://nsm.no/getfile.php/1314334-1742808614/NSM/Filer/Dokumenter/Veiledere/NSM%20Cryptographic%20Recommendations%202025.pdf

[10]	AIVD, The PQC Migration Handbook
https://english.aivd.nl/binaries/aivd-en/documenten/publications/2024/12/3/the-pqc-migration-handbook/The+PQC+Migration+Handbook+.pdf

[11]	https://cyber.gouv.fr/enjeux-technologiques/cryptographie-post-quantique/faq-pqc/),

[12]	3GPP, Release 20 – Milestones and endorsed deadlines, https://www.3gpp.org/specifications-technologies/releases/release-20

# 4	Objective

The objectives of this work item are outlined in the following work tasks:

- WT#1: Introduce PQC key encapsulation algorithms in the 3GPP cryptographic profiles of (D)TLS 1.3 and IKEv2.

- WT#2: Add, in the cryptographic profile of TLS 1.2, a note about the threat of Cryptographically Relevant Quantum Computers (CRQCs) and a recommendation strongly discouraging the use of TLS 1.2. Update the TLS profile to permit client and server to be able to enforce TLS1.3 and refuse to use TLS 1.2. To support the use of TLS 1.2 in certain operator use cases, provide a profile to configure the "supported versions" extension to allow TLS 1.2 in the client hello, and for servers to be configured to enable TLS1.2.

NOTE 1: Both standalone and hybrid mechanisms are in scope.

NOTE 2: This WID will be updated during Rel-20 timeframe to include a WT related to computing quantum-resistant SUCIs based on the progress in TR 33.703, and other WTs related to the publication of relevant IETF RFCs.

NOTE 3: This WID will be updated during the Rel-20 timeframe to include a WT related to introducing PQC algorithms in 3GPP cryptographic profiles of (D)TLS 1.3, IKEv2, JWE, JWS, Public Key Certificate, CRL, OCSP, and CMP.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 |  | 1.5 | Yes |  |
| WT#2 |  | 0.5 | Yes |  |

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.210 | Introduce PQC algorithms in cryptographic profiles | SA#113 (Sept 2026) |  |
| TS 33.310 | Introduce PQC algorithms in cryptographic profiles | SA#113 (Sept 2026) |  |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

Not identified yet

# 9	Supporting Individual Members

Supporting IM name

Ericsson

AT&T

NIST

BT

NCSC

Nokia

Sectra Communications

Deutsche Telekom

VODAFONE

ZTE

T-Mobile US

Qualcomm

BSI (DE)

China Mobile