3GPP TSG-SA3 Meeting #126	draft_S3-260778

Goa, India, 9 – 13 February 2026

Source:	Vodafone, Verizon, China Mobile, NTT DOCOMO, KDDI, AT&T, Deutsche Telekom, Telecom Italia, Ericsson ?, IIT Bombay

Title:	pCR to TS 33.502 – Detecting SBA layer parameters change

Document for:	Approval

Agenda item:	5.1.1

Spec:	3GPP TS 33.502

Version:	1.0.0

Work Item:	SECHAND

Comments

Most, if not all, the security related events that can occur at the SBA layer can have their origin in an erroneous or malicious configuration related (but not limited) to:

- Changes of the communication models (e.g. an NF expected to use indirect mode with delegated discovery is changed to use direct mode).

- Changes to local configurations as alternative to SBA dynamic procedures (e.g. static IP addresses, discovery, etc)

- Changes related to the types of NFs allowed to access a given NF instance (e.g. a UDR instance is allowed to be accessed by non-intended NFs)

- Changes related to authorizing scopes assigned to an NF-Consumer (e.g. an NF-Consumer of certain type is allowed to access another NF resources without restriction).

- Changes related to parameters used to establish security link between SEPPs

- Changes related to the NF status (e.g. an NF “canary release” condition is removed)

- Changes to security trust condition of AFs (e.g. an AF is changed from untrusted to trusted condition)

These example changes can be executed in conjunction or independently from other changes, leading to abnormal situations in the SBA layer with a potential malicious origin.

This pCR proposes adding a security related event every time an NF SBA related parameters are changed/updated. In addition, the time of the last NF configuration should be appended to the common information elements included in clause 6.2 for the events specified in clause 6.

* * * First Change * * * *

## 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 29.500 "Technical Realization of Service Based Architecture; Stage 3"

[3]		3GPP TS 33.501: "Security architecture and procedures for 5G System"

[4]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)"

[5]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security"

[x]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3”

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

IOC	Information Object Class

* * * Next Change * * * *

## 6.7	Security events related to SBA parameters configuration

An NF configuration is received in which related SBA level parameters are changed/updated.

NF shall detect a reconfiguration event and reports it to the security related event collection entity directly or indirectly.

The event delivery should be sent directly to the security related events collection entity.

The event delivery may be delivered to the to the security related events collection entity via a transmitter in the operator trust domain.

NOTE 2: Direct delivery is preferred over delivery via OAM, as this allows detection of security related events in case of a compromised OAM system.

Some examples of OAM configured SBA level parameters in case the information model follows 3GPP TS 28.541 [x]:

-	The ManagedNFProfile, contains the list of NFs that contact a specific NF. If such parameter is changed, affected NF can potentially be illegally accessed.

-	The commModelList, contains the communication model that a specific NF fulfills. If the parameter is changed, it can potentially mean that affected NF changes its behavior.

-	The certificate, contains NF identifiers like FQDN, Issuer information, subject public key info, etc. If the parameter is changed, it can potentially mean the affected NF is recognized as another NF.

-	For SEPPs, SBA level parameters is seppInfo which can impact the PRINS protection,

-	For SCPs, supportedFunction, address and scpInfo are essential to provision SCPs.

* * * End of Changes * * * *