3GPP TSG-SA3 Meeting #126	S3- 260228r5

Goa, India, 9 – 13 February 2026

Source:	Vodafone, Verizon, China Mobile, NTT DOCOMO, KDDI, AT&T, Deutsche Telekom, Telecom Italia, IIT Bombay, Samsung

Title:	pCR to TS 33.502 – Detecting events related to unexpected communication model flows

Document for:	Approval

Agenda item:	5.1.1

Spec:	3GPP TS 33.502

Version:	1.0.0

Work Item:	SECHAND

Comments

The communications model an NF is to use, as NF consumer, is part of the configuration of the particular NF, according to TS 23.501 (6.3.1) and TS 28.541 (5.3.69). The configuration attribute indicates the type of communication model (i.e. direct communication without NRF discovery, direct communication with NRF discovery, indirect communication without delegated discovery and indirect communication with delegated discovery) and the target list of NF services sharing same communication model and configuration.

Additionally, and according to the specifications, each NF can be configured with a list of different communication models for different sets of target NF services. For example, SMF can use direct communication for e.g. PCF services while using indirect model for e.g. UDM services. Specifications do not restrict using different models for different services of the same NF.

Additionally, neither NRF nor SCP are aware of the communication models an NF consumer is configured with and for which target NF services. Making the communication model per service part of the profiles registered in the NRF would simplify the detection of the event due to malicious attempts.

Considering the above, the detection of an event when the communication model flow is not the expected one becomes a complex task at this stage of the 5G specifications, and detection of the event will be limited to the cases related to the NF producer registration, or not, in the NRF and the event will be detected by different NFs involved :

- Direct communication with NRF discovery requires the NF producer registered in the NRF. NRF will detect an unexpected communication model flow when a discovery request is received for a non-registered NF or NF service

- Direct communication with NRF discovery requires OAuth tokens are used.  An NF producer will detect an unexpected communication model flow when receiving a service request without an OAuth token after having registered to the NRF.

- Direct communication without NRF discovery does not involve OAuth tokens. An NF producer will detect an unexpected communication model flow when not registered into the NRF and receiving a service request with an OAuth token.

- Indirect communication with or without delegated NRF discovery implies the NF producer is registered in the NRF. NRF or SCP will detect an unexpected communication model flow when a discovery request is received (by the NRF) for a non-registered NF or NF service or a discovery response is sent (to the SCP) indicating no profile exist for the NF producer.

- Indirect communication model with or without delegated NRF discovery implies the SCP to include the custom header 3gpp-Sbi-Target-apiRoot to be included in the service request to the NF producer. An NF producer will detect an unexpected communication model flow when 3gpp-Sbi-Target-apiRoot is received in a service request but the NF producer did not register into the NRF.

* * * First Change * * * *

## 6.X	Security events related to Unexpected SBA communication model flow

Unexpected SBA communication model flows can be detected when an NRF, SCP or an NF Producer receive requests not complying with the specified procedures related to communication models.

Editors note:	It is for FFS what (additional) events are reported by NF (NRF, SCP or NF producer).

* * * End of Changes * * * *