3GPP TSG-SA3 Meeting #126	S3-260783-r3

# Goa, India, 9 - 13 February 2026

Source:	Huawei, HiSilicon

Title:	Update to Overview of Security related Events Handling

Document for:	Approval

Agenda item:	5.1.1

Spec:	3GPP TS 33.502

Version:	1.0.0

Work Item:	SECHAND

Comments

Polishing the first and second paragraphs.

* * * First Change * * * *

# 4	Overview of Security related Events handling

Editor’s Note: This clause addresses the architectural view of the feature

The Service Based Architecture (SBA) is the dominant method for control plane as well as the Service Based Management Architecture (SBMA) which is for management communications. In addition to the many benefits of using SBA (e.g., agility to increase and decrease the number of service instances in coordination with demand), there may be potential attacks to the network, services and/or APIs.

The 5G system includes various Network Function (NF) deployments, where each and every NF has a specified behaviour according to 3GPP specifications. If any NF runs into unexpected behaviours (e.g., a violation of normal behaviour, abnormal access or unauthorised request), then the NF needs to be evaluated from a security perspective. The collection of security related events need to be performed for the evaluation of the NF behaviour. The related data is transmitted to a security entity which will execute the evaluation.

The following figure shows an example of trust domains in the overall architecture for the collection and transmission of the Security related Events detected by the NFs.

Figure 4-1 Example of trust domains in the Security related Events Handling architecture

The NF gets configuration of security related events from the Management entity and delivers security related events through the Event transmitter. The Event transmitter could be an independent function or part of the NF. The Event transmitter is recommended to be part of an NF. Whether the Event transmitter is inside the NF, outside the NF and inside the 5GC, or outside the 5GC, as depicted in Figure 4-1, is based on operator implementation.

The Event transmitter, Management Entity and Security related event collecting entity are the end points of the related configuration and collection interfaces. The interfaces for collection of security related events are in scope of the present document. Requirements on the configuration of security related events are in scope of the present document. The interfaces between the Event transmitter and the NFs are out of the scope of 3GPP.

NOTE 1: Operators will define the relationship between trust zones.

NOTE 2: The Security related events collecting entity is under operator control (e.g. through business agreements, policy, managed service, directly managed, etc) and it is out of the scope of 3GPP.

NOTE 3: Whether the security collecting entity is the same as the management entity is an operator decision.