3GPP TSG-SA3 Meeting #126	S3-260638

# Goa, India, 9 – 13 February 2026

Source:	Ericsson

Title:	Resolving Editors’ Note in Solution-9 Scope Parameter with Purpose Information

Document for:	Approval

Agenda item:	5.2.10

Spec:	3GPP TR 33.700-23

Version:	0.2.0

Work Item:	FS_CAPIF_Ph4_SEC

Comments

This pCR is proposed to resolve the following Editor’s Note in Solution #9 and provides an evaluation for the solution.

Editor’s Note: Weather the purpose of information is needed in the request is FFS.

A new text for further explanation about the solution is added for the resolution of the Editor’s Note.

A new Editor’s Note is added to highlight the need of further specification for purpose information in the SA6 working group.

* * * First Change * * * *

## 6.9	Solution #9: Augmenting scope parameter with purpose information

### 6.9.1	Introduction

This solution addresses key issue #3 and consists of augmenting the scope parameter in the token/authorization request and the token with purpose information; the resource owner authorization revocation request would likewise include purpose information.

As aligned with the key issue description, the solution is not aiming to specify different purpose values, but to describe the usage of purpose information in authorization.

NOTE: An example of purpose definition is found in W3C Data Privacy Vocabulary.

### 6.9.2	Solution details

-	For the client credential flow in CAPIF RNAA, the purpose information is included in the scope parameter of the token request.

-	For the authorization code flow in CAPIF RNAA, the purpose information is included in the scope parameter of authorization request.

-	The scope parameter in the issued token includes the purpose information.

NOTE: How to encode the purpose information into the scope parameter of the requests and of the token is not in the scope of this solution.

-	In the revocation procedure, it is proposed that the purpose information can also be sent to the CCF and the CCF can use this information in identification of CAPIF RNAA tokens to be revoked. This solution proposes to leave how to structure the resource owner authorization revocation request to the implementation because the details of CAPIF-8 reference point is out of scope of 3GPP.

The purpose information in the token request or authorization request is required to enable the CCF to perform authorization based on the purpose information.

### 6.9.3	Evaluation

This solution addresses the requirement of key issue #3 by making the authorization mechanism in CAPIF RNAA aligned with the other working groups architecture and CAMARA architecture, in terms of purpose information usage.

Since the purpose information is included in the token, that information will be a verified information by the AEF. So, there is no need for further check.

* * * End of Changes * * * *