3GPP TSG-SA3 Meeting #126																				S3-260299

Goa, India, 9 – 13 February 2026

# Source:	Samsung, Oppo, ZTE, Nokia

# Title:	Annex#B MAC-CE risk analysis (complete)

# Document for:	Approval

Agenda Item:	5.3.1

1	Decision/action requested

Propose to approve the risk analysis of MAC CEs described in this contribution

*** Start of Change ***

Annex B

Risk analysis of MAC-CE

Editor’s Note: Structure of annex is FFS.

Editor’s Note: Format of the framework capturing risk analysis is FFS.

Editor’s Note: Methodology for the risk analysis is FFS.

B.1	General

In LTE and 5GNR, security for Control Plane (CP) and User Plane (UP) traffic between the User Equipment (UE) and the base station is fundamentally anchored at the Packet Data Convergence Protocol (PDCP) layer. The risk of Medium Access Control (MAC) layer needs to be analysed. This Annex captures the security and privacy risk analysis of the MAC-CEs from clause 6.1.3 of TS 38.321.

The MAC Control Element (MAC-CE) is a signaling message used at the MAC layer to manage time-critical control functions. For example, MAC-CEs are used for Layer 2 operations, conveying control information for resource management, scheduling, power control, and link maintenance. MAC-CEs were introduced in Release 8 (LTE) and has been expanded in every subsequent release.

Editor’s Note: The alignment of above paragraph with RAN2 is FFS.

B.2	Risk Analysis Methodology

Editor’s Note: This clause describes agreed methodology for analyzing risks resulting from potential exploitation of functionality of MAC-CE control messages at the MAC layer.

B.3	Summary Analysis of MAC-CEs

Editor’s Note: This clause contains risk analysis output

This subclause provides a consolidated summary of the security and privacy risk analysis of MAC Control Elements (MAC CEs) specified in TS38.321[x]. The summary groups MAC CEs with similar functional characteristics. This summary is intended to support consistent interpretation of the analysis results and to facilitate alignment with RAN2inputs and SA3 risk prioritization study.

NOTE: The exact analysis below follows the methodology described in Clause B.2 of this Annex.

## Editor’s Note: the categories and description of MAC CEs need to be updated, e.g., based on the feedback from RAN2.

### 1. Identity, Access, and Contention Resolution MAC CEs: These MAC CEs support UE identification, access procedures, and contention handling during initial access and mobility. Typical functions include assignment or update of UE identifiers, Resolution of contention during random access, Support for successful connection establishment. Examples include C-RNTI MAC CE and UE Contention Resolution Identity MAC CE.

### Threat type: TBA.

### Threat range: TBA.

### Threat complexity: TBA.

### Threat consequence: TBA.

### Potential recovery means: TBA.

### 2. Timing and Synchronization Control MAC CEs: This category covers MAC CEs that control uplink timing alignment and synchronization between UE and network. Typical functions include, Timing Advance adjustments, Timing mode indication, Timing offset and delta signaling, Synchronization for special deployments (e.g. NTN, IAB). Examples include Timing Advance Command MAC CE, Absolute Timing Advance MAC CE, Timing Delta MAC CE, and Timing Case Indication MAC CE.

Threat type: TBA.

Threat range: TBA.

Threat complexity: TBA.

Threat consequence: TBA.

Potential recovery means: TBA.

### 3. Power Control and Power Headroom Reporting MAC CEs: These MAC CEs convey UE power capabilities and support uplink power control decisions. Typical functions include, Reporting power headroom, Supporting uplink power control for single or multiple cells, Power control for advanced features such as multi-TRP. Examples include Single Entry PHR, Multiple Entry PHR, and enhanced PHR MAC CEs.

Threat type: TBA.

Threat range: TBA.

Threat complexity: TBA.

Threat consequence: TBA.

Potential recovery means: TBA.

### 4. Discontinuous Reception (DRX) and Activity Control MAC CEs: This category controls UE activity cycles and energy-saving mechanisms. Typical functions include, Activation or deactivation of DRX operation, Switching between DRX modes (short/long). Examples include DRX Command MAC CE and Long DRX Command MAC CE.

Threat type: TBA.

Threat range: TBA.

Threat complexity: TBA.

Threat consequence: TBA.

Potential recovery means: TBA.

### 5. Scheduling, Resource, and Rate Control MAC CEs: These MAC CEs influence how radio resources are allocated and how traffic is scheduled. Typical functions include, Buffer and delay reporting, Recommended or enforced data rate signaling, Resource grant confirmation. Examples include Buffer Status Report MAC CE, Delay Status Report MAC CE, Recommended Bit Rate MAC CE, and UL Rate Control MAC CE.

Threat type: TBA.

Threat range: TBA.

Threat complexity: TBA.

Threat consequence: TBA.

Potential recovery means: TBA.

### 6. Beam Management and TCI-Related MAC CEs: This category includes MAC CEs that control beam configuration, TCI state activation, and spatial relations. Typical functions include, Activation/deactivation of TCI states, Indication of spatial relations for PUCCH/PUSCH/SRS, Beam failure detection and recovery. Examples include TCI States Activation/Deactivation MAC CEs, PUCCH Spatial Relation MAC CEs, BFR MAC CEs, and enhanced TCI MAC CEs.

Threat type: TBA.

Threat range: TBA.

Threat complexity: TBA.

Threat consequence: TBA.

Potential recovery means: TBA.

### 7. Mobility and Lower-Layer Triggered Mobility (LTM) MAC CEs: These MAC CEs support fast mobility procedures that bypass or complement RRC signaling. Typical functions include Cell switch commands, Candidate cell preparation, Mobility-related measurement triggers. Examples include LTM Cell Switch Command MAC CE and related enhanced variants.

Threat type: TBA.

Threat range: TBA.

Threat complexity: TBA.

Threat consequence: TBA.

Potential recovery means: TBA.

### 8. CSI, SRS, and Measurement Control MAC CEs: This category controls channel measurement and sounding procedures. Typical functions include, Activation/deactivation of CSI reporting, Control of SRS resources, Measurement configuration for mobility and positioning. Examples include CSI-RS activation MAC CEs, SRS spatial relation MAC CEs, and event-triggered measurement MAC CEs.

Threat type: TBA

Threat range: TBA.

Threat complexity: TBA.

Threat consequence: TBA.

Potential recovery means: TBA.

### 9. Positioning-Related MAC CEs: These MAC CEs support NR positioning features. Typical functions include, Activation/deactivation of positioning gaps, Control of positioning SRS and processing windows. Examples include Positioning Measurement Gap Activation/Deactivation MAC CEs and Positioning SRS MAC CEs.

Threat type: TBA.

Threat range: TBA.

Threat complexity: TBA.

Threat consequence: TBA.

Potential recovery means: TBA.

### 10. IAB, Sidelink, and Specialized Deployment MAC CEs: This category covers MAC CEs specific to particular deployment scenarios. Typical functions include, IAB timing, beam, and power coordination, Sidelink scheduling and reporting, Network-controlled repeater control. Examples include IAB timing MAC CEs, sidelink BSR MAC CEs, and NCR beam indication MAC CEs.

Threat type: TBA.

Threat range: TBA.

Threat complexity: TBA.

Threat consequence: TBA.

Potential recovery means: TBA.

Overall summary: the analysis indicates that risk level of MAC CEs is at different levels based on the functionality, feature supported and deployed. Some of the common characteristics are,

1.	They directly influence time critical radio behavior (e.g., timing, beam direction, mobility, or scheduling).

2.	Exploitation leads to immediate and user visible service disruption, to a single UE and in some cases a group of UEs.

Editor’s Note: Further security analysis of MAC CEs are FFS.

B.2.2	Risk Prioritization

Editor’s Note: This clause contains agreement on risk prioritization.

B.3	Interim Agreement

Editor’s Note: This clause contains agreed principles taking into account RAN WGs input.

B.4	Risk Tolerance

Editor’s Note: This clause contains risk tolerance / acceptance criteria for residual risk.

*** End of Change ***