3GPP TSG-SA3 Meeting #126	draft_S3-260801-r5

Goa, India, 9 – 13 February 2026

Title:	LS on MAC CE security

Response to:	N/A

Release:	Rel-20

Work Item:	FS_6G_SEC

Source:	SA3

To:	RAN2

Cc:	N/A

Contact person:	Todor Gamishev (Orange)

todor.gamishev@orange.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

SA3 has initiated study work on the security protection of MAC Control Elements (MAC-CEs) in the context of 6G. To ensure alignment between functional design and security architecture, SA3 kindly requests clarification from RAN2 on the following aspects:

1. In case MAC-CE protection is based on cryptographic mechanisms, please clarify the applicable functional constraints, including:

- Maximum allowable size (in bits/bytes) for additional security-related fields (e.g., integrity tag, sequence number, security header);

- Acceptable processing overhead budget at UE and gNB sides (e.g., computational complexity, hardware acceleration assumptions);

- Latency constraints in 6G RAN (e.g., budget in μs or TTIs) applicable to MAC-CE generation, protection, verification, and potential retransmission.

2. Please clarify the expected sequencing and state handling for MAC-CE transmission and processing:

- Whether MAC-CEs are expected to follow a strictly stateful sequence;

- Behavior upon out-of-sequence reception or loss of a MAC-CE;

- Whether MAC-CE reordering is foreseen in 6G;

- Whether certain MAC-CEs may be pre-processed or pre-authenticated for security purposes;

- Impact on related RAN procedures (including recovery procedures) in case of failed integrity verification or detection of manipulation (e.g., attacker-modified MAC-CE).

3. Please clarify whether there are protocol-level conventions or constraints regarding:

- Transmission of MAC-CEs individually versus grouping multiple MAC-CEs within a single MAC PDU;

- Functional or time-critical grouping considerations;

- UE-specific aggregation constraints that may impact the granularity of security protection.

4. Regarding future extensibility (Rel-20 and beyond):

- Is it expected that any MAC-CE security protection framework shall automatically accommodate newly introduced MAC-CE types without requiring redesign?

- What is the preferred coordination model between RAN2 (functional specification) and SA3 (security specification) for security impact assessment, risk severity evaluation, and definition of countermeasures for new MAC-CEs?

5. SA3 assumes that MAC-CEs will continue to be utilized in future releases and that new MAC-CE types are likely to be introduced as 6G evolves. Kindly confirm this assumption or provide alternative guidance.

6. Please identify MAC-CE categories that may be considered, e.g.:

- Time-critical (e.g., impacting scheduling, HARQ, beam management, power control);

- Overhead-sensitive (e.g., size-constrained or high-frequency transmission);

- Service-critical, where manipulation could cause significant degradation or denial-of-service.

SA3 understands that RAN2 expects preliminary security impact considerations to be available for discussion at SA#XXX (June 2026). Kindly confirm the expected timeline for providing the above clarifications to ensure alignment of the Rel-20 study and normative work.

In addition, please review the MAC CE security analysis captured in Annex-B of TR 33.801-01 and comment or suggest any enhancements.

# 2	Actions

To RAN2

ACTION: SA3 kindly asks RAN2 to provide answers to the above questions and confirm the timeline.

# 3	Dates of next TSG SA WG 3 meetings

SA3#127	13 – 17 April 2026		Malta

SA3#128		  18 – 22 May 2026						China