3GPP TSG-SA3 Meeting #126	draft_S3-260802-r1

# Goa, India, 9 – 13 February 2026			revision of S3-260610 S3-260328 S3-260511 S3-260538

Source:	Qualcomm Incorporated, Apple, Vivo, ZTE, Samsung, SK Telecom, Oppo, InterDigital

Title:	SA#2 – KI#NEW - MAC CE security

Document for:	Approval

Agenda item:	5.3.1

Spec:	3GPP TR 33.801-01

Version:	0.2.0

Work Item:	FS_6G_SEC

Comments

It is proposed to approve a new key issue on MAC CE security.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] 	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TR 38.914: “Study on 6G Scenarios and requirements”.

[3] 	3GPP TR 38.760-2: “Study on 6G Radio RAN2 aspects”.

[4] 	3GPP TR 23.801-01: “Study on Architecture for 6G System”.

[5] 	3GPP TR 33.771: “Study on supporting AEAD algorithms”.

[6]	3GPP TR 23.801-1: "Study on Architecture for 6G System".

[x]	“A Stealthy Location Identification Attack Exploiting Carrier Aggregation in Cellular Networks”, N. Lakshmanan, N. Budhdev, M. Kang, M. Chan, J. Han, USENIX Security 21

[y]	“Unprotected 4G/5G Control Procedures at Low Layers Considered Dangerous”, N. Ludant, M. Vomvas, G. Noubir, arXiv:2403.06717

[z]	S3-251154 Reply LS on security handling for inter-CU LTM in non-DC cases

* * * Second Change * * * *

### 5.2.3.y	Key issue # 2.y: MAC CE security

#### 5.2.3.y.1	Key issue details

In 5G, MAC layer handles mapping logical channels to transport channel, scheduling and managing resources. This MAC layer carries control information  through a MAC layer structure, called MAC Control Element (CE). The MAC CE includes several Information Elements, to accommodate new use cases, as outlined in TS 38.321 [xx], clause 6.1.3. The number of MAC CEs has grown significantly.

Due to lack of security protection of MAC CEs, there have been some security and/or privacy concerns raised in the past. Examples include privacy attack based on monitoring the SCell activation/deactivation MAC CE [x], service degradation by manipulating/injecting various MAC CEs such as Beam Failure Recovery (BFR), Buffer Status Report (BSR), Discontinuous Reception (DRX) Command MAC CEs [y], to name a few.

#### 5.2.3.y.2	Security threats

The security threats related to MAC CEs and their risk analysis are documented in Annex B of the present document.

#### 5.2.3.y.3	Potential security requirements

The 6G system should provide a security protection mechanism for critical MAC layer control information.

NOTE y: The protection mechanism needs to be generic and future proof and should be compatible for protection on any MAC control information in future 6G releases, if needed.

Editor’s Note: Whether protection is applicable to all MAC CEs or identified MAC CEs is FFS.

#### 5.2.3.y.4	Interim agreements

* * * End of Changes * * * *