3GPP TSG-SA3 Meeting #126	draft_S3-260802-r1

# Goa, India, 9 – 13 February 2026			revision of S3-260610 S3-260328 S3-260511 S3-260538

Source:	Qualcomm Incorporated, Apple, Vivo, ZTE, Samsung, SK Telecom, Oppo, InterDigital

Title:	SA#2 – KI#NEW - MAC CE security

Document for:	Approval

Agenda item:	5.3.1

Spec:	3GPP TR 33.801-01

Version:	0.2.0

Work Item:	FS_6G_SEC

Comments

It is proposed to approve a new key issue on MAC CE security.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] 	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TR 38.914: “Study on 6G Scenarios and requirements”.

[3] 	3GPP TR 38.760-2: “Study on 6G Radio RAN2 aspects”.

[4] 	3GPP TR 23.801-01: “Study on Architecture for 6G System”.

[5] 	3GPP TR 33.771: “Study on supporting AEAD algorithms”.

[6]	3GPP TR 23.801-1: "Study on Architecture for 6G System".

[x]	“A Stealthy Location Identification Attack Exploiting Carrier Aggregation in Cellular Networks”, N. Lakshmanan, N. Budhdev, M. Kang, M. Chan, J. Han, USENIX Security 21

[y]	“Unprotected 4G/5G Control Procedures at Low Layers Considered Dangerous”, N. Ludant, M. Vomvas, G. Noubir, arXiv:2403.06717

[z]	S3-251154 Reply LS on security handling for inter-CU LTM in non-DC cases

* * * Second Change * * * *

### 5.2.3.y	Key issue # 2.y: MAC CE security

#### 5.2.3.y.1	Key issue details

In 5G, MAC layer handles mapping logical channels to transport channel, scheduling and managing resources. This MAC layer carries control information  through a MAC layer structure, called MAC Control Element (CE) generated at the MAC layer. The MAC CE payload carries control commands in Control Elements, to accommodate new use cases, as outlined in TS 38.321 [xx], clause 6.1.3. The number of MAC CEs has grown significantly.

Due to lack of security protection of MAC CEs, there have been some security and/or privacy concerns raised in the past. Examples include privacy attack based on monitoring the SCell activation/deactivation MAC CE [x], service degradation by manipulating/injecting various MAC CEs such as Beam Failure Recovery (BFR), Buffer Status Report (BSR), Discontinuous Reception (DRX) Command MAC CEs [y], to name a few.

#### 5.2.3.y.2	Security threats

The security threats related to MAC CEs and their risk analysis are documented in Annex B of the present document.

#### 5.2.3.y.3	Potential security requirements

The 6G system should provide a security protection mechanism for critical MAC layer control information.

NOTE y: The protection mechanism needs to be generic and future proof and should be compatible for protection on any MAC control information in future 6G releases, if needed.

Editor’s Note: Whether protection is applicable to all MAC CEs or identified MAC CEs is FFS.

#### 5.2.3.y.4	Interim agreements

* * * End of Changes * * * *