3GPP TSG-SA3 Meeting #126	draft_S3-260802-r5

# Goa, India, 9 – 13 February 2026			revision of S3-260610 S3-260328 S3-260511 S3-260538

Source:	Qualcomm Incorporated, Apple, Vivo, ZTE, Samsung, SK Telecom, Oppo, InterDigital, Nokia

Title:	SA#2 – KI#NEW - MAC CE security

Document for:	Approval

Agenda item:	5.3.1

Spec:	3GPP TR 33.801-01

Version:	0.2.0

Work Item:	FS_6G_SEC

Comments

It is proposed to approve a new key issue on MAC CE security.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] 	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TR 38.914: “Study on 6G Scenarios and requirements”.

[3] 	3GPP TR 38.760-2: “Study on 6G Radio RAN2 aspects”.

[4] 	3GPP TR 23.801-01: “Study on Architecture for 6G System”.

[5] 	3GPP TR 33.771: “Study on supporting AEAD algorithms”.

[6]	3GPP TR 23.801-1: "Study on Architecture for 6G System".

[x]	“A Stealthy Location Identification Attack Exploiting Carrier Aggregation in Cellular Networks”, N. Lakshmanan, N. Budhdev, M. Kang, M. Chan, J. Han, USENIX Security 21

[y]	“Unprotected 4G/5G Control Procedures at Low Layers Considered Dangerous”, N. Ludant, M. Vomvas, G. Noubir, arXiv:2403.06717

[z]	S3-251154 Reply LS on security handling for inter-CU LTM in non-DC cases

* * * Second Change * * * *

### 5.2.3.y	Key issue # 2.y: MAC CE security

#### 5.2.3.y.1	Key issue details

In 5G NR, MAC Control Elements (MAC CEs) are used to convey time‑critical control information within the MAC layer, such as scheduling control, buffer status, timing advance, and resource management signaling.  The MAC layer forms a payload from the layers above and adds its own Control Elements to create a single MAC PDU. The MAC layer MAC PDU consists of multiple Sub-PDU(s) which contain either control commands in Control Element (MAC-CE) or user/control plane data in Service Data Unit (MAC SDU) from different logical channels.

Editor’s note: details to be updated based on the input from RAN2 on MAC CEs.

#### 5.2.3.y.2	Security threats

#### 5.2.3.y.3	Potential security requirements

#### 5.2.3.y.4	Interim agreements

* * * End of Changes * * * *