3GPP TSG-SA3 Meeting #126	draft_S3-260810-r2 (merger of 505+542+605)

# Goa, India, 9 – 13 February 2026

Source:	Xiaomi, Samsung, Qualcomm

Title:	New Key Issue for SA#2 on Security Enhancement for RAN Mobility

Document for:	Approval

Agenda item:	5.3.1

Spec:	3GPP TR 33.801-01

Version:	0.2.0

Work Item:	FS_6G_SEC

Comments

In 4G and 5G systems, during X2/Xn handover, key separation cannot be achieved immediately after the handover, because the source eNB/gNB which derives the KeNB*/KNG-RAN* knows the keys of the target eNB/gNB. To achieve key separation, the target eNB/gNB needs to initiate an intra-eNB/intra-gNB handover to take the new NH into use, once the new NH has arrived the target eNB/gNB via the PATH SWITCH REQUEST ACKNOWLEDGE message. Therefore, in 4G and 5G systems, key separation between the gNBs is achieved only after two handovers, i.e. the current security mechanism for X2/Xn handover only provides two-hop forward security, which has the following drawbacks:

-	If the target eNB/gNB does not initiate an intra-eNB/intra-gNB handover to take the new NH into use immediately after the X2/Xn handover, key separation between source and target eNBs/gNBs is delayed and there is the risk that an attacker who cracked the KeNB/KgNB of the source eNB/gNB is able to launch attacks on the connection between the UE and the target eNB/gNB.

-	If the target eNB/gNB initiates an intra-eNB/intra-gNB handover to take the new NH into use after each X2/Xn handover, additional RRC signalling for intra-eNB/intra-gNB handover is required at the target gNB, which causes more network resource consumption.

Based on the above analysis, it needs to be investigated whether and how two-hop security during Xn handover can be enhanced to address the drawbacks. Hence, this contribution proposes a new key issue on security enhancement for RAN mobility for the security area of RAN security in TR 33.801-01.

* * * First Change * * * *

5.2.3	Key issues

Editor’s note: This clause will contain the key issues that need to be addressed by SA3 on each security area. The exact contents are FFS.

5.2.3.y	Key issue #2.y: Security enhancement for RAN mobility

5.2.3.y.1	Key issue details

In 4G and 5G systems, during X2/Xn handover, key separation cannot be achieved immediately after the handover, because the source eNB/gNB which derives the KeNB*/KNG-RAN* knows the keys of the target eNB/gNB. To achieve key separation, the target eNB/gNB needs to initiate an intra-eNB/intra-gNB handover to take the new NH into use, once the new NH has arrived the target eNB/gNB via the PATH SWITCH REQUEST ACKNOWLEDGE message. Therefore, in 4G and 5G systems, key separation between the source and target RAN nodes is achieved only after two handovers, i.e., the current security mechanism for X2/Xn handover only provides two-hop forward security, which has the following drawbacks:

-	Delayed Forward Secrecy: If the target eNB/gNB does not initiate an intra-eNB/intra-gNB handover to take the new NH into use immediately after the X2/Xn handover, key separation between source and target eNBs/gNBs is delayed and there is the risk that an attacker who cracked the KeNB/KgNB of the source eNB/gNB is able to launch attacks on the connection between the UE and the target eNB/gNB.

-	Signaling Overhead: If the target eNB/gNB initiates an intra-eNB/intra-gNB handover to take the new NH into use after each X2/Xn handover for a forward secrecy, additional RRC signalling for intra-eNB/intra-gNB handover is required at the target eNB/gNB, which causes more network resource consumption.

-	Mobility Constraints: In scenarios requiring frequent or low-latency mobility (e.g., LTM), the dependency on the source RAN for key derivation limits the flexibility of key management.

In 6G, it is desired to investigate how key separation between the source and target RAN nodes during the mobility procedure can be enhanced to support various mobility scenarios while ensuring robust AS security.

Editor's Note: other aspects are FFS.

5.2.3.y.2	Security threats

Without immediate forward security, a compromise of the source node would lead to a compromise of the target node keys.

5.2.3.y.3	Potential security requirements

Editor's Note: potential security requirements are FFS.

5.2.3.y.4	Interim agreements

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#x.y. This may be populated directly or e.g. also when a topic in Area #x gets resolved and a principle is agreed. Where there is consensus, interim agreements pertaining to this key issue (e.g. solution principles descriptions, not specific solutions) should be documented in this clause as soon as possible during the study.

* * * End of Change * * * *