3GPP TSG-SA3 Meeting #126	draft_S3-260810-r4 (merger of 505+542+605)

# Goa, India, 9 – 13 February 2026

Source:	Xiaomi, Samsung, Qualcomm

Title:	New Key Issue for SA#2 on Security Enhancement for RAN Mobility

Document for:	Approval

Agenda item:	5.3.1

Spec:	3GPP TR 33.801-01

Version:	0.2.0

Work Item:	FS_6G_SEC

Comments

In 4G and 5G systems, during X2/Xn handover, key separation cannot be achieved immediately after the handover, because the source eNB/gNB which derives the KeNB*/KNG-RAN* knows the keys of the target eNB/gNB. To achieve key separation, the target eNB/gNB needs to initiate an intra-eNB/intra-gNB handover to take the new NH into use, once the new NH has arrived the target eNB/gNB via the PATH SWITCH REQUEST ACKNOWLEDGE message. Therefore, in 4G and 5G systems, key separation between the gNBs is achieved only after two handovers, i.e. the current security mechanism for X2/Xn handover only provides two-hop forward security, which has the following drawbacks:

-	If the target eNB/gNB does not initiate an intra-eNB/intra-gNB handover to take the new NH into use immediately after the X2/Xn handover, key separation between source and target eNBs/gNBs is delayed and there is the risk that an attacker who cracked the KeNB/KgNB of the source eNB/gNB is able to launch attacks on the connection between the UE and the target eNB/gNB.

-	If the target eNB/gNB initiates an intra-eNB/intra-gNB handover to take the new NH into use after each X2/Xn handover, additional RRC signalling for intra-eNB/intra-gNB handover is required at the target gNB, which causes more network resource consumption.

Based on the above analysis, it needs to be investigated whether and how two-hop security during Xn handover can be enhanced to address the drawbacks. Hence, this contribution proposes a new key issue on security enhancement for RAN mobility for the security area of RAN security in TR 33.801-01.

* * * First Change * * * *

5.2.3	Key issues

Editor’s note: This clause will contain the key issues that need to be addressed by SA3 on each security area. The exact contents are FFS.

5.2.3.y	Key issue #2.y: Security for RAN mobility

5.2.3.y.1	Key issue details

Mobility within 6G radio and between 5G NR and 6G radio introduces challenges related to security context handling. In legacy systems (4G/5G), key separation cannot be achieved immediately during X2/Xn handover.

This key issue investigates whether and how key separation between the source and target RAN nodes during the mobility procedure can support various mobility scenarios while ensuring robust AS security.

Editor's Note:	Present key issue details are based on the current understanding of 5GS RAN mobility procedures. It will be updated based on the progress in RAN WGs.

Editor's Note: other aspects are FFS.

5.2.3.y.2	Security threats

Editor's Note: threats are FFS.

5.2.3.y.3	Potential security requirements

Editor's Note: potential security requirements are FFS.

5.2.3.y.4	Interim agreements

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#x.y. This may be populated directly or e.g. also when a topic in Area #x gets resolved and a principle is agreed. Where there is consensus, interim agreements pertaining to this key issue (e.g. solution principles descriptions, not specific solutions) should be documented in this clause as soon as possible during the study.

* * * End of Change * * * *