3GPP TSG-SA3 Meeting #126	draft_S3-260815-r1 was S3-260568

# Goa, India, 9 – 13 February 2026

Source:	Thales, ZTE

Title:	KI #2: solution for DO-A capable AIoT devices

Document for:	Approval

Agenda item:	5.2.11

Spec:	3GPP TR 33.714

Version:	0.2.0

Work Item:	FS_AIoT_SEC_Ph2

Comments

This contribution proposes solution for Key Issue #2 for DO-A capable AoT devices.

* * * First Change * * * *

5.0	Mapping of solutions to key issues

Editor’s Note: This clause captures mapping between key issues and solutions.

Table 5.1-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 |
| 1 |  |  | X |  |  |
| 2 |  |  | X |  |  |
| 3 |  |  | X |  |  |
| 4 |  |  |  | X |  |
| 5 |  |  |  | X |  |
| 6 |  |  |  | X |  |
| 7 |  |  |  | X |  |
| 8 |  |  |  | X |  |
| #X |  | X |  |  |  |

* * * Next Change * * * *

5.X	Solution #X: AKA-based authentication for DO-A capable AIoT devices

5.X.1	Introduction

This solution addresses Key Issue #2 for DO-A capable AIoT devices.

5.X.2	Solution details

For DO-A capable AIoT device in public network or PNI-NPN: 5G AKA or EAP-AKA’ authentication procedure specified in clause 6 of TS 33.501 [9] is used to perform the mutual authentication between the DO-A Capable AIoT device and the network. The ADM plays the role of the AUSF/UDM, and the AIoTF plays the role of the SEAF.

For DO-A capable AIoT device in SNPN: 5G AKA, EAP-AKA’, EAP-TLS or any key-generating EAP-method is used to perform the authentication between the DO-A Capable AIoT device and the network, as specified in Annex I of TS 33.501 [9].

Editor's Note: Alignment with SA2 is FFS

Editor's Note: Whether this solution applies for AIoT device type2 is FFS

Editor's Note: How to support the inventory and command procedure is FFS

Editor's Note: The mapping between 5G-AKA and AIoT architecture is FFS

5.X.3	Evaluation

TBD.

* * * End of Changes * * * *