3GPP TSG-SA3 Meeting #126	S3-260825-r1

# Goa, India, 9 – 13 February 2026

Source:	Nokia

Title:	Solution to KI#4 concerning privacy protection of DOA request

Document for:	Approval

Agenda item:	5.2.11

Spec:	3GPP TR 33.714

Version:	0.2.0

Work Item:	FS_AIoT_SEC_Ph2

Comments

This is a solution to KI#4 but shows the fully flow to keep the holistic view. The pCR proposes a solution to identify a device which sends a DO-A message and maintains identity privacy. It utilizes the SUCI to protect the identity of the AIoT device bearing in mind REL 20 device types will support UICC.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23-700-13: "Study on Architecture Support of Ambient power-enabled Internet of Things".

[3]	3GPP TR 38.848: "Technical Specification Group Radio Access Network; Study on Ambient IoT (Internet of Things) in RAN".

[4]	3GPP TR 23700-30: "Study on Architecture support of Ambient power-enabled Internet of Things (AIoT); Phase 2".

[5]					3GPP TR 38.769: "Study on solutions for Ambient IoT (Internet of Things) in NR".

[6]	3GPP TS 22.369: "Service Requirements for ambient power-enabled IoT".

[7]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[8]	3GPP TS 33.369: "Security aspects of Ambient Internet of Things (AIoT) services for isolated private networks".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[a]	3GPP TS 23.003: "Numbering, addressing and identification"

* * * Next Change * * * *

## 5.0	Mapping of solutions to key issues

Editor’s Note: This clause captures mapping between key issues and solutions.

Table 5.1-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 |
| 1 |  |  | X |  |  |
| 2 |  |  | X |  |  |
| 3 |  |  | X |  |  |
| 4 |  |  |  | X |  |
| 5 |  |  |  | X |  |
| 6 |  |  |  | X |  |
| 7 |  |  |  | X |  |
| 8 |  |  |  | X |  |
| Y |  |  |  | X |  |

* * * Next Change * * * *

## 5.Y	Solution #Y: DO-A request using UICC services

### 5.Y.1	Introduction

The solution provides a procedure to protect the identity during DO-A request. The method uses a SUCI based approach, as this study addresses PLNM deployments, implying UICC services are available in the AIoT device including authentication methods and identifier concealment. The solution includes a T-ID in the DO-A message together with the SUCI, as an identifier which can be used during inventory procedure to identify the AIoT device. The T-ID furthermore enables the AIoT device and AIoTF to get into sync, in case of out of sync or initial message send, in case no valid T-ID is stored in the ADM. The inventory and command steps are mentioned in the solution to maintain a holistic view and the relation to proceeding steps after the DO-A message.

### 5.Y.2	Solution details

The procedure shown in figure 1 is initiated when a AIoT device determines that data must be sent as DO-A. This could be sensor data which periodically needs to be updated, warnings i.e. basement flooding, temperature surge/drop, or others.

Figure 1: DO-A signalling initiating an inventory + command procedure.

The device selects a suitable DO-A access occasion based on configuration.

- The AIoT device and AIoTF are pre-provisioned with the AIoT permanent device ID. It’s assumed that T-ID is not pre-configured.

- The device constructs a SUPI according to TS 23.003 [a] with the following modification. SUPI type is NSI, Home network identifier is the domain name of the AIoTF and AIoT device permanent identifier is set as the username. The T-ID* is an identifier which is generated by a pseudo-random number generator. The intent of T-ID* is to provide an identifier which can be used during inventory, until the device gets assigned a T-ID by the AIoTF. The device stores the T-ID* but doesn’t substitute it with the real T-ID*.

The AIoT device sends the SUCI* and T-ID* to the AIoT reader.

- The AIoT reader forwards the message to the AMF, which selects the AIoTF based on the home routing indicator, the temporary identifier or local policies and sends the SUCI* and T-ID* to the AIoTF.

- The AIoTF sends the SUCI* and T-ID* to the ADM.

- If the ADM receives a SUCI* and deconceals it to construct a SUPI.

The ADM verifies whether the AIoT permanent identifier exists in the AIoT device register. If exists, the ADM generates an AV for this device using SUPI* to identify the authentication keys and derive the AV. The ADM returns the SUPI* and AV. T-ID* is included as a session identifier to enable the AIoTF to authenticate the AIoT prior to fetching the sensor data. If the ADM doesn’t have a T-ID or is out of sync, it may decide to substitute the T-ID with the T-ID*.

- -7. The AIoTF will authenticate the device using EAP AKA’ as defined in TS 33.501 clause 6.1 and include the T-ID* as identifier in the request. This is similar to the first message of the inventory command. This step may happen at a later time, when the AIoTF initiates an inventory command procedure.

If the authentication is successful and the received T-ID* is matching the “remembered” T-ID* in step 1 or T-ID stored in the device, the AIoT device will update the T-ID * to T-ID if this is signaled as part of the inventory command.

- -9. If the AIoTF decides to update the T-ID in the device, it may be executed as a command message. If the command message is successful, the AIoTF updates the T-ID in the AIoT device register.

Editor’s note: Whether the SUCI is post quantum compliant is FFS.

Editor’s note: Whether the solution aligns with SA2 agreed procedures is FFS.

Editor’s note: Whether AKA and SUCI is applicable for type 2 devices is FFS.

### 5.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

TBD

* * * End of Changes * * * *