# 3GPP TSG-SA3 Meeting #126																			draft S3-260838-r2

# Goa, India, 09 – 13 February 2026

Source:	Huawei, HiSilicon

Title:	Solution for KI #2

Document for:	Approval

Agenda item:	5.2.14

Spec:	TR 33.724

Work Item:	FS_5G_WAB_SEC

Comments

This contribution addresses the Key Issue 2 on security protection of compromised WAB nodes and core network measures .

* * * First Change * * * *

## 6.Y 	Solution #Y: WAB-node Mutual Authentication and Message Protection

### 6.Y.1	Introduction

This solution addresses security risks from compromised WAB nodes in untrusted environments by reusing existing 3GPP security mechanisms specified in TS 33.501[4] and NDS/IP framework for authentication and message protection, enabling secure interaction with the 5GC and NG-RAN without introducing new security primitives.

6.Y.2	Solution details

This solution reuses existing 3GPP security mechanisms specified in TS 33.501[4] and the Network Domain Security for IP-based protocols (NDS/IP) framework. The WAB-MT applies 5G primary authentication (e.g. 5G-AKA) for mutual authentication with the AUSF/UDM, as specified in TS 33.501[4]. The WAB-gNB reuses certificate-based NDS/IP mechanisms, such as TLS or IPsec, for mutual authentication with the AMF, UPF and neighboring gNBs.

Message protection for N2, N3, Xn and OAM interfaces reuses existing NDS/IP mechanisms, in particular IPsec ESP, providing integrity, confidentiality and replay protection. These reused mechanisms mitigate rogue signaling and topology manipulation threats in untrusted WAB node environments.

### 6.Y.3	Evaluation

The solution leverages existing security mechanisms and procedures.

* * * End of Changes * * * *