3GPP TSG-SA3 Meeting #126	S3-260841-r2

# Goa, India, 09 - 13 February 2026															revision of S3-260739

Source:	Nokia

Title:	Replay prevention updates

Document for:	Approval

Agenda item:	5.2.12

Spec:	3GPP TR 33.755

Version:	0.2.0

Work Item:	FS_BSP4SBA

Comments

This document resolves the EN’s for security best current practice for (RFC 9700).

* * * First Change * * * *

## 5.2	BSP #2: Token replay prevention

### 5.2.1	Description of best practice

This best practice addresses token replay prevention as described in clause 2.2 of RFC 9700 [2].

RFC 9700 [2] cover access token and refresh token under token replay prevention. Both types of token can be replayed, hence replay prevention is necessary.

The best practice recommends to use sender-constrained access token scopes to prevent misuse of stolen and leaked access tokens. It is required to use sender-constrained refresh tokens or refresh token rotation for public clients.

### 5.2.2	Usage in 5G SBA

Refresh tokens are not utilised and applicable to 5G SBA.

In the 5G SBA, access tokens are bound to mTLS authentication state between the network functions, these checks are made either at the discovery, access token request or service request.

Reference: 13.4.1.1.2 of TS 33.501 [3]:

The access tokens request is validated by NRF based on the identity of the NF Service Consumer by comparing the NF Instance Id to the subjectAltName in the NFc TLS client certificate subsequently issuing the access token, which contains the subject claim ("sub"). This claim provides the identity of the NF Service Consumer and hereby ties the access token to the NFc instance ID.

In the direct communication case, the NF Service Producer checks whether the NF Instance ID in the subject claim within the access token matches the NF Instance ID in the subjectAltName in the NF Service Consumer's TLS client certificate. This enables the NF Service Producer to perform validation of the subject claim and constrains the access token to the sender NF Service Consumer.

In indirect communication, it is not possible to verify that the SCP or SEPP is using the access token on behalf of the NF Service Consumer that is identified by the subject access token claim.

Reference: clause 13.3.8.1 of TS 33.501 [3]:

In indirect communication, CCA token provides means to authenticate the NF Service Consumer towards the receiving end point (NRF, NF Service Producer) but it does not provide end-to-end integrity protection on the full-service request due to TLS being established only between two respective NFs (i.e., NFc – SCP, SCP – NRF, SCP – NFp). This makes CCA token prone to replay attacks.

### 5.2.3	Assessment

Refresh tokens are not applicable to 5G SBA.

For access tokens, the following assessment applies:

In direct communication scenarios, it can be verified that the NF Service Consumer that is identified by the subject access token claim, is the entity that presents the access token to the NF Service Producer. A replay of the token can be therefore detected.

In indirect communication, it is not possible to verify that the SCP or SEPP are authorized to use the access token or CCA token on behalf of the NF Service Consumer.

Consequently, it would be possible to replay access tokens by a compromised SCP or SEPP.

Short-lived tokens reduce the risk of exposure by minimizing the time window for using leaked or stolen access tokens. Using sender-constrained access tokens to prevent access token misuse in indirect communication use cases can be further studied.

Editor’s Note: Further assessment is FFS

* * * End of Changes * * * *