3GPP TSG-SA3 Meeting #126	draft_S3-260856-r3 (merger of 312+408+500+726)

# Goa, India, 9 – 13 February 2026

Source:	Xiaomi, Huawei, ZTE, OPPO

Title:	Update to Key Issue #4

Document for:	Approval

Agenda item:	5.2.7

Spec:	3GPP TR 33.777

Version:	0.3.0

Work Item:	FS_Sensing_SEC

Comments

In the current version of TR 33.777, there is a key issue on active attacks in sensing with an editor’s note stating that the feasibility of the attack is FFS. This pCR proposes to update the key issue #4 in TR 33.777 by resolving the editor’s note with additional analysis on the feasibility. The active attacks such as interference and jamming, are general threats to wireless systems and could impact communication as well. Presenting KI#4 as an ISAC-unique security problem could lead to disproportionate concern. This pCR also proposes to add some clarification to prevent misinterpretation that ISAC introduces a new vulnerability category.

* * * First Change * * * *

5.4	Key issue #4 on active attacks in sensing

5.4.1	Key issue details

One of the use cases for sensing technology is detection of aerial objects. In order to be useful, the result has to be reliable, ie. report an aerial object when there is one, and report empty airspace only when the airspace is in fact empty.

During the sensing operation, an attacker could generate a radio signal that would confuse the receiving sensing node into determining that there is an aerial objects at a location where there is none (e.g by sending a signal that exhibits the typical micro-Doppler shift typical for UAV rotors), or into determining that there is no aerial object where in fact there is one (e.g. by generating noise such that the response by a real aerial object is drowned out, or perceived to come from a different location).

While theoretically possible, the described attacks are operationally infeasible in the following ways:

- Attackers do not have prior knowledge of the specific sensing parameters and real-time channel state information that is unavailable without direct access to the transmitter’s configuration, which is not specified and up to implementations in 5GA ISAC.

- Drowning out authentic sensing returns makes the attack easily detectable hence physically impractical for covert attack deployment.

- Such attacks are even more difficult in a distributed multi-node sensing architecture supported in TR 38.765-02 [x], as attackers do not have prior knowledge of site-specific characterization.

NOTE: 	Active attack is a general threat applicable to wireless systems and may affect both communications and sensing, and is not unique to ISAC.

5.4.2	Security threats

Not applicable.

5.4.3	Potential security requirements

Not applicable.

NOTE: 	Active attack is a general threat applicable to both communications and sensing. Therefore, there is no new security requirement identified.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-14: "Study on Integrated Sensing and Communication; Stage 2".

[3]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[4]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[5]	3GPP TR 33.501: "Security architecture and procedures for 5G system".

[6]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[7]					3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security”.

- IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

- 3GPP TS 23.501: "System Architecture for the 5G System".

- 3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[11]	IETF RFC 6083: "Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".

[xx]	3GPP TS 38.765-02: "Study on Integrated Sensing And Communication (ISAC) for NR".

* * * End of Changes * * * *