3GPP TSG-SA3 Meeting #126	draft_S3-260881-r1

# Goa, India, 9 – 13 February 2026

Source:	vivo

Title:	New Solution on AEAD Algorithm Interface

Document for:	Approval

Agenda item:	5.3.2

Spec:	3GPP TR 33.771

Version:	0.2.0

Work Item:	FS_AEAD

Comments

This contribution proposes a new solution to key issue #2.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

| Solutions | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 |
| --- | --- | --- | --- | --- | --- |
| Solution 1: NAS and AS SMC enhancement with AEAD | X |  |  |  |  |
| Solution 2: enhancement for security mode command | X |  |  |  |  |
| Solution 3: NAS SMC enhancement to support AEAD algorithms | X |  |  |  |  |
| Solution 4: AEAD Algorithm negotiation | X |  |  |  |  |
| Solution 5: AEAD algorithm negotiation | X |  |  |  |  |
| Solution 6: AEAD algorithms negotiation | X |  |  |  |  |
| Solution 7: AEAD key usage for NAS and AS algorithm |  | X |  |  |  |
| Solution 8: Input & output definition |  | X |  |  |  |
| Solution 9: Interface of AEAD |  | X |  |  |  |
| Solution 10: Creation of EXTRA_IV |  | X |  |  |  |
| Solution 11: Key Derivation for NAS and AS AEAD |  |  | X |  |  |
| Solution Y AEAD Algorithm Interface |  | X |  |  |  |

* * * Next Change * * * *

6.Y	Solution Y: AEAD Algorithm Interface

6.Y.1	Introduction

This solution is proposed to address the key issue#2 on AEAD interface.

6.Y.2	Solution details

6.Y.2.1	Generic inputs and outputs of AEAD algorithm

The input parameters to the AEAD algorithm include:

- a 256-bit AEAD key named KEY,

- a 48-bit EXTRA_IV,

- a 32-bit COUNT,

- a 5-bit bearer identity BEARER,

- the 1-bit direction of the transmission i.e. DIRECTION. The DIRECTION bit shall be 0 for uplink and 1 for downlink.

- 1-bit MODE. The MODE bit shall be 0 for encryption and 1 for decryption.

- 5-bit MAC_BYTES. Length of output MAC in bytes. maximum supported value is 16.  If MAC_BYTES = 0, then no integrity verification is done.

Editor’s Note:	Usage of MAC_BYTES is ffs.

- Variable Additional Authenticated Data (AAD). Array of bytes containing additional integrity protected data without ciphering.

- 32-bit AAD_LENGTH. The number of bits of AAD.

- Variable Input Bit Stream(IBS). Array of bytes containing the input ciphering bit stream of plaintext.

- 32-bit S_LENGTH. The number of bits of IBS.

The output parameters to the AEAD algorithm include:

- Variable Output Bit Stream (OBS)

- 8 * MAC_BYTES-bit MAC

6.Y.2.2	Guidance for using AEAD algorithm

EXTRA_IV: May vary per message.

COUNT: NAS COUNT for NAS message, PDCP COUNT for AS message.

NOTE:	COUNT usage for MAC CE protection will be discussed in TR 33.801-01 [7].

MAC_BYTES: May vary per message.

Protection Parameters: IBS, S_LENGTH, AAD, and AAD_LENGTH: May differ across messages depending on the protection scheme applied.

The AEAD algorithm supports 4 protection scheme: cipher-only, integrity-only, full cipher and integrity, and partial cipher with full integrity. The configuration of inputs for each operation is detailed in Table 6.Y.2.2-1. This assumes that the MESSAGE is structured with IE1 and IE2.

Table 6.Y.2.2-1 Input sets for AEAD algorithm protections

|  | Cipher-only | Integrity-only | Full cipher and integrity | Partial cipher with full integrity |
| --- | --- | --- | --- | --- |
| IBS | MESSAGE | Null | MESSAGE | IE1 |
| S_LENGTH | Length of MESSAGE | 0 | Length of MESSAGE | Length of IE1 |
| AAD | Null | MESSAGE | Null | IE2 |
| AAD_LENGTH | 0 | Length of MESSAGE | 0 | Length of IE2 |
| MAC_BYTE | 0 | 4-16 (infer 32-128bit length MAC) | 4-16 (infer 32-128bit length MAC) | 4-16 (infer 32-128bit length MAC) |

Editor’s Note:	Analysis on complexity of partial ciphering with full integrity using NCA is ffs.

6.Y.3	Evaluation

TBA.

* * * End of Changes * * * *