# 3GPP TSG-SA3 Meeting #126																			S3-260394

# Goa, India, 09 – 13 February 2026

Source:	Huawei, HiSilicon

Title:	New solution about on-demand Extra-IV value generation

Document for:	Approval

Agenda item:	5.3.2

Spec:	3GPP TR 33.771

Version:	0.2.0

Work Item:	 FS_AEAD

Comments

EXTRA_IV filed is used to introduces extra entropy for AEAD IV input parameter, but setting the value of EXTRA_IV to a fixed value (e.g. 0x00) will not introduce extra entropy for IV. Therefore, different implementations of EXTRA_IV may lead to two different levels of security protection. In situations requiring higher security levels, using a pseudo-random number as the value for EXTRA_IV could be a better choice.

This contribution proposes a new solution about on-demand Extra-IV value generation.

* * * First Change * * * *

6.X	Solution #X: On-demand Extra-IV Value Generation

6.X.1	Introduction

This solution addresses key issue #1 and key issue #2.

6.X.2	Solution details

During the algorithm negotiation procedure, the network can decide whether a pseudo-random number or a fixed value (e.g. 0x00) is used as the value of EXTRA_IV, based on the UE security capability and network policy (e.g. service security requirement, UE security level, etc.). Then the network notifies the way of generating EXTRA_IV to the UE. And the UE generates the EXTRA_IV according to the network’s indication and performs AEAD protection with the network side using the generated EXTRA_IV.

Editor’s Note: whether EXTRA_IV is for future use is FFS.

Editor’s Note: whether UE security capability indicates the support of on-demand extra-IV value generation is FFS.

Editor’s Note: the details on the negotiation message is FFS.

6.10.3	Evaluation

Editor’s Note: Place holder for an evaluation if necessary.

* * * End of Change * * * *