3GPP TSG-SA3 Meeting #126	draft_S3-260889

# Goa, India, 9 – 13 February 2026													(revision of S3-260177)

Source:	Nokia

Title:	Pseudo-CR on Solution evaluation for solution 1

Document for:	Approval

Agenda item:	5.2.1

Spec:	3GPP TR 33.771

Version:	0.2.0

Work Item:	FS_AEAD

Comments

This pCR introduces the solution evaluation for solution#1.

* * * Start of Change * * * *

## 6.1	Solution 1: NAS and AS SMC enhancement with AEAD

Existing NAS Security mode command procedure, AS security mode procedure, RRC reconfiguration procedure is enhanced with AEAD algorithm selection.

### 6.1.1	Introduction

This solution addresses the key issue#1.

NAS and AS procedure (option1) – Integrity-only AEAD for NAS/AS SMC
The approach limits exposure by protecting the security-mode command (SMC) with integrity only, keeping the payload in clear-text until the UE confirms the selected AEAD algorithm. This reduces processing load on the network and UE during the initial handshake.

NAS and AS procedure (option2) – Full AEAD (integrity + encryption) for NAS/AS SMC
Encrypting and authenticating the SMC provides confidentiality for the selected algorithm identifier and any ancillary parameters, eliminating the clear-text exposure present in Option 1. The added cryptographic cost is modest for modern UE and offers stronger resistance to replay and downgrade attacks during the initial security-mode exchange.

### 6.1.2	Solution details

Editor’s Note: Definition/clarification of the AEAD mode is FFS.

Editor’s Note: Clarification on the reuse of the procedures is FFS.

Editor’s Note: Explanation of the purpose of sending the NAS SMC both in plaintext and encrypted is FFS.

NAS and AS procedure (option1)

Overview: With this approach, the AMF and RAN will use AEAD algorithm with NULL encryption option or integrity only mode selected, so only the NAS / AS SMC is integrity protected, when sending it towards UE. UE can verify the integrity of the NAS/AS SMC and send the response NAS SMC with integrity and ciphering, AS SMC with integrity protection alone.

Detailed steps

Step 1 Registration and Authentication is successful.

Step 2 and step 3 The AMF generates NAS AEAD Key. AMF will use selected Algorithm AEAD ID (example: 256-NCA4), the NAS_EXTRA_IV (this value could be newly generated or known value at AMF and UE could be used) to generate MAC-I (AEAD algo is used with NULL encryption or integrity only mode). NAS security mode command which is only integrity protected is sent with selected AEAD algorithm, AEAD mode.

Step 4 The UE will use NAS_Extra_IV (if this value is generated at AMF, then it is sent to UE or if configured at UE and AMF, the same is used) to verify the MAC-I. UE will start the integrity and encryption using AEAD algorithm for the response NAS security mode complete.

Step 3 NGAP initial context setup message with UE capabilities indicating supported AEAD algorithms, gNB key is sent to gNB. The gNB will generate the AS AEAD key.

Step 4 The RAN could generate AS Extra_IV and start integrity protection using AEAD algorithm with NULL encryption or encryption only mode. AS security mode command is sent to UE.

Step 5 UE will verify the MAC and AS_Extra_IV. UE will start integrity protection and ciphering using AEAD algorithm.

NAS and AS procedure (option2)

Overview: With this approach, the AMF and RAN will use AEAD algorithm, so only the NAS / AS SMC is both integrity protected and ciphered. When sending the NAS/AS SMC the AMF/RAN towards UE, the selected AEAD algorithm is ciphered / integrity protected and as plain text as well. UE can verify the integrity of the NAS/AS SMC and send the response NAS/AS SMC complete with integrity and ciphering.

Step 1 UE will have the RRC connection established.

Step 2 The initial NAS message is sent with UE security capabilities with AEAD expanded IE.

Step 3 Further AKA challenge of authentication procedure is completed.

Step 4 AMF will start the ciphering and integrity protection using selected AEAD algorithm. The NAS security mode command will carry the selected AEAD algorithm is ciphered / integrity protected and as plain text as well. UE can verify the MAC and de-cipher the message with help of the selected AEAD algorithm sent in plain text and after de-ciphering the same context is cross checked.

Step 5 RAN will start the ciphering and integrity protection using selected AEAD algorithm. The AS security mode command will carry the selected AEAD algorithm is ciphered / integrity protected and as plain text as well. UE can verify the MAC and de-cipher with help of the selected AEAD algorithm sent in plain text and after de-ciphering the same context is cross checked.

User Plane procedure

The RRC connection reconfiguration procedure is used to add DRBs as part of the PDU session establishment after RRC security has been activated. The gNB sends UP AEAD activation indication including AEAD mode IE for the activation of either UP integrity and ciphering or combined integrity+ ciphering or even NULL ciphering and NULL integrity for each DRB. UE verifies the RRC connection reconfiguration message.

### 6.1.3	Evaluation

-
--

--

TBD

Editor’s Note: Further evaluation to be added.

* * * End of Changes * * * *