3GPP TSG-SA3 Meeting #126	draft_S3-260895

# Goa, India, 9 – 13 February 2026												(revision of S3-260179)

Source:	Nokia

Title:	Pseudo-CR on Solution evaluation for solution 7

Document for:	Approval

Agenda item:	5.3.2

Spec:	3GPP TR 33.771

Version:	0.2.0

Work Item:	FS_AEAD

Comments

This pCR is introduces the solution evaluation for solution#7.

* * * Start of Change * * * *

## 6.7	Solution 7: AEAD key usage for NAS and AS algorithm

### 6.7.1	Introduction

This solution addresses the key issue#2.

Like the existing NAS algorithms and AS algorithms for integrity protection and ciphering (reference from Annex D of TS 33.501 [5]), the combined algorithm needs to be shown for the AS and NAS module usage.

### 6.7.2	Solution details

Figure 6.7.2-1: Derivation of MAC-I

The input parameters to the NCA (NG Combined Algorithm) algorithm are 256-bit (array of 32 bytes) security Key (example: KNASAEAD or KRRCAEAD or KUPAEAD), 32-bit NAS or PDCP COUNT (UL or DL COUNT),1 bit of MODE of 0(encrypt) or 1(decrypt), 5-bits of BEARER identity, DIRECTION bit could be 0 for uplink and 1 for downlink and 6 bytes of Extra entropy for the IV(Initial Value).

EXTRA_IV called as EXTRA_IVNAS or EXTRA_IVRRC or EXTRA_IVUP is a Random number generated by AMF/RAN and exchanged with UE during NAS or AS Security mode command or RRC Reconfiguration message.

### 6.7.3	Evaluation

The proposed interface abstracts AEAD operations as a black-box, preserving algorithm-independent parameter definitions. By explicitly incorporating the extra-entropy IV field and standardized AAD handling (applicable for combined mode), it mitigates the risk of IV reuse and ensures forward-secure confidentiality and integrity. Solution also aligns with Annex D.2 and D.3 of TS 33.501.

* * * End of Changes * * * *