3GPP TSG-SA3 Meeting #126	draft_S3-260921-r1

# Goa, India, 9 – 13 February2026                                                                          was S3-260356

Source:	ChinaTelecom

Title:	New solution on security protection for N2 interface for NR Femto

Document for:	Approval

Agenda item:	6.1.3

Spec:	TR 33.746

Version:	v0.3.0

Work Item:	FS_NR_Femto_Sec_Ph2

Comments

Add a new solution on security protection for N2 interface for NR Femto.

* * * First Change * * * *

6.Y	Solution #Y: Security protection for N2 interface for NR Femto

6.Y.1	Introduction

This solution addresses the requirements of key issue #1.

A misconfigured or compromised NR Femto device with valid credentials and subscription to connect to the SeGW can pose various threats including abnormal traffics, abnormal signalling messages, denial of service attacks to the NR Femto MS and the core network.

This solution propose to enhance the Security Gateway or NR Femto Gateway as defined in TS 33.545 [3] to prevent core network against the attacks through N2 interface, i.e. NG-C interface. 5G NR Femto securely communicate with AMF via Security Gateway or NR Femto Gateway in front of 5GC over N2 interface. All N2 related input/output traffic over the trust boundary should be delegated and protected by Security Gateway or NR Femto Gateway.

- Y.2	Solution details

6.Y.2.1	Security architecture

The security aspect enhancements to system architecture of clause 4.1 in TS 33.545 [3] for security protection for N2 interface are further depicted in Figure 6.Y.2.1-1.

UE

NR Femto

SeGW

Insecure link

Operator’s security domain(s)

NR Femto GW

NR Femto MS

NR Femto MS

SMF / AUSF / UPF / UDM

UPF

UE

NR Femto

SeGW

Insecure link

Operator’s security domain(s)

NR Femto GW

NR Femto MS

NR Femto MS

Core Network (AMF/UPF/ SMF)

UPF

Figure 6.Y.2.1-1: Enhancement for security architecture of NR Femto

In addition to security requirements for N2 interface as defined in TS 33.545 [3], security protections provided by the Security Gateway or NR Femto Gateway for the traffic through N2 interface between 5G NR Femto and AMF deployed in core network over the trust boundary also can be included in the following way:

- Signalling message filtration;

- Access control etc.

NOTE: Security protection function for N2 interface can be implemented by Security Gateway or NR Femto Gateway (if deployed).

6.Y.2.2	Signalling message filtration

The Security Gateway or NR Femto Gateway supports to discard malformed signalling messages sent from the 5G NR Femto nodes through N2 interface over the trust boundary according to 3GPP specifications.

The Security Gateway or NR Femto Gateway supports to block messages with wrong NF types sent from the 5G NR Femto nodes through N2 interface over the trust boundary according to 3GPP specifications.

The Security Gateway or NR Femto Gateway supports the rate-limiting functionalities to defend itself and core network NFs against excessive or overload signalling messages of N2 interface.

6.Y.2.3	Access control

The Security Gateway or NR Femto Gateway supports the access control mechanism for the 5G NR Femto nodes accessing the AMF deployed in core network, e.g. configure the access control list.

6.Y.3	Evaluation

In order to eliminate risks associated with compromised NR Femto nodes, e.g., preventing the abnormal traffic or signalling threats, this solution addresses the requirements of KI #1 by enhancing the Securtiy Gateway or NR Femto Gateway in the architecture of NR Femto to provide enhanced security protection for the N2 interface between the NR Femto nodes and the core network, including signalling message filtration and access control.

The security protection function for the N2 interface can be implemented by Security Gateway or NR Femto Gateway (if deployed) in this solution.

* * * End of Changes * * * *