3GPP TSG-SA3 Meeting #126	draft_S3-260949-r1 was S3-260335

# Goa, India, February 09-13 2026

Source:	China Telecom, ZTE, Nokia

Title:	Security requirements for KI#1

Document for:	Approval

Agenda item:	5.2.4

Spec:	3GPP TR 33.758

Version:	0.2.0

Work Item:	FS_PLMNNPN_Ph2

Comments

This contribution proposes security requirements for KI#1 in TR 33.758.

Proposed Changes

* * * First Change * * * *

7.1.2	Security threats

Attackers in one PNI-NPN operational domain can forge TEID and infer the TEIDs assigned to PLMN operational domains and other PNI-NPN operational domains and launch further attacks.

7.1.3	Potential security requirements

The 5G system shall support the generation of TEID values with high-entropy for each subsequent session on N9 interface.

NOTE: An example of high-entropy values are random numbers. The realization of high entropy is left for implementation .

* * * End of Changes * * * *