3GPP TSG-SA3 Meeting #126	draft_S3-260950-r2

# Goa, India, February 09-13 2026                                                                 Revision of S3-260337

Source:	China Telecom, ZTE

Title:	Solution to inter domain security on N9 interface

Document for:	Approval

Agenda item:	5.2.4

Spec:	3GPP TR 33.758

Version:	0.2.0

Work Item:	FS_PLMNNPN_Ph2

Comments

This contribution proposes solution to inter domain security on N9 interface for TR 33.758.

Proposed Changes

* * * First Change * * * *

8.Y	Solution #Y: Extended IPUPS for inter domain security on N9 interface between PLMN and PNI-NPN

8.Y.1	Introduction

This solution addresses KI#2 Inter domain security on N9 interface.

This solution proposes to extend the use of the IPUPS from inter-PLMN only to both inter-PLMN and intra-PLMN.

8.Y.2	Solution details

According to clause 4.2.2 TS 33.501[2], Inter-PLMN UP Security (IPUPS) is introduced at the perimeter of the PLMN, which enforces GTP-U security on the N9 interface between UPFs of the visited and home PLMNs.

This solution proposes to extend the IPUPS in intra-PLMN scenario. The IPUPS is deployed between PLMN and PNI-NPN, connecting UPFs in the PLMN operational domain and UPFs in the PNI-NPN operational domain.

Clause 5.9.3.4 TS 33.501[2] states:

The IPUPS shall only forward GTP-U packets that contain an F-TEID that belongs to an active PDU session and discard all others.

The IPUPS shall discard malformed GTP-U messages.

Therefore, the IPUPS deployed between PLMN and PNI-NPN can only forward GTP-U packets that contain an F-TEID that belongs to an active PDU session and discard all others. IPUPS also discards malformed GTP-U messages from PNI-NPN operational domain to enhance the security on N9 interface between PLMN and PNI-NPN.

8.Y.3	Evaluation

This solution extends the use of IPUPS to intra-PLMN.

The IPUPS can discard GTP-U packets without an active  F-TEID and malformed GTP-U messages from PNI-NPN.

This solution fulfills the security requirement in KI #2.

* * * End of Changes * * * *