# 3GPP TSG-SA3 Meeting #126																	draft_S3-260969-r1

# Goa, India, 9 – 13 February 2026

Source:	Ericsson

Title:	Solution for KI#2

Document for:	Approval

Agenda item:	5.2.7

Spec:	3GPP TR 33.777

Version:	0.3.0

Work Item:	FS_Sensing_SEC

Comments

The document proposes solution for KI#2 Security for sensing service operation.

* * * First Change * * * *

## 6.2	Solutions to KI#2

### 6.2.X	Solution #2.X: Security protection for sensing service operations between sensing entity and SF

6.2.X.1		Introduction

This solution is for security protection for sensing service operations between Sensing Entity (SE) and Sensing Function (SF). This solution assumes that Sensing Entity and Sensing Function support a direct connection/ interface for sensing control signalling and/or sensing data reporting.

#### 6.2.X.2			Solution details

For security protection of sensing service operations between Sensing Entity and Sensing Function, the interface between SE and SF is secured by the NDS/IP security procedures as specified in Clause 9 of TS 33.501 [5].

In order to protect the SE-SF interface reference point, it is required to implement IPsec ESP and IKEv2 certificates-based authentication as specified in Clause 9.1.2 of TS 33.501 [5]. IPsec is mandatory to implement on the Sensing entity (i.e. gNB) and the Sensing Function at core network side. On the core network side, a Security Gateway (SEG) may be used to terminate the IPsec tunnel.

The transport of sensing control signalling and sensing data reporting over SE-SF interface is required to be integrity, confidentiality and replay-protected. The security protection of transport data depends on the underlying transport protocol used on the SE-SF interface.

If the transport layer protocol is SCTP for transport of sensing control signalling and/or sensing data reporting, then in addition to IPsec, DTLS needs to be supported as specified in RFC 6083 [11] to provide mutual authentication, integrity protection, replay protection and confidentiality protection. Security profiles for DTLS implementation and usage need to follow the TLS profile given in clause 6.2 of TS 33.210 [7] and the certificate profile given in clause 6.1.3a of TS 33.310 [6]. In this case, existing security mechanisms for the N2 interface as specified in Clause 9.2 of TS 33.501 [5] can be reused.

If the transport layer protocol is WebSocket as specified in RFC 6455 [x] for transport of sensing control signalling and/or sensing data reporting, then in addition to IPsec, TLS needs to be supported to provide mutual authentication, integrity protection, replay protection and confidentiality protection. Security profiles for TLS implementation and usage needs to follow the TLS profile given in clause 6.2 of TS 33.210 [7] and the certificate profile given in clause 6.1.3a of TS 33.310 [6]. The WebSocket Protocol is an independent TCP-based protocol. Connection confidentiality and integrity is provided by running the WebSocket Protocol over TLS. The WebSocket server can use any client authentication mechanism available to a generic HTTP server, such as cookies, HTTP authentication, or TLS authentication.

Editor's Note: Transport layer security protection depends on the protocol agreed upon for SE-SF interface by RAN WGs and needs to align with RAN WGs.

The solution does not provide any authorization aspects/details between SE (gNB) and SF and is aligned with TR 23.700-14 [2] which has concluded that there is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

#### 6.2.2.3	Evaluation

Editor's Note: Evaluation is ffs.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non‑specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                    	3GPP TR 23.700-14: "Study on Integrated Sensing and Communication; Stage 2".

[3]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[4]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[5]	3GPP TR 33.501: "Security architecture and procedures for 5G system".

[6]                  	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[7]				3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security”.

[8] 				IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[9]				3GPP TS 23.501: "System Architecture for the 5G System".

[10] 			3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[11]                	IETF RFC 6083: "Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".

[x]						IETF RFC 6455: "The WebSocket Protocol".

* * * End of Changes * * * *