# 3GPP TSG-SA3 Meeting #126																			S3-260972

# Goa, India, 9 – 13 February 2026

Source:	BSI (DE), Montsecure, Rakuten Mobile Inc.

Title:	Add new CAPIF CF test case that verifies whether AEF_PSK is computed correctly by the CAPIF CF

Document for:	Approval

Agenda item:	5.1.7

Spec:	3GPP TS 33.531

Version:	0.1.0

Work Item:	SCAS_5GA_CAPIF

Comments

- Reason for change: There is no SCAS test case yet that verifies whether AEF_PSK is correctly returned to an API Exposing Function by the CAPIF CF. If the CAPIF CF does not return the correct key, it might be an indication of a security flaw that is exploitable by an attacker (the CAPIF CF may compute a different, wrong AEF_PSK* that can be used by an attacker, or the CAPIF CF may provide a key belonging to a different API Invoker).

- Summary of change: Added a new SCAS test case TC_VERIFY_AEF_PSK that verifies whether AEF_PSK is correctly returned to an API Exposing Function by the CAPIF CF.

- Consequences if not approved: API Exposing functions might be accessible by unauthorized parties due to the CAPIF CF incorrectly verifying the onboarding credential of an API Invoker.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[3]	3GPP TS 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[4]	3GPP TS 33.122: " Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[X]	3GPP TS 23.222: “Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs”.

* * * Next Change * * * *

#### 4.2.2.X	AEF_PSK Computation Verification

Requirement Name: AEF_PSK Computation Verification

Requirement Reference: TS 33.122 [4], clause 6.5.2.1

Requirement Description: The Key AEF_PSK is bound to an API Exposing Function and  derived as specified in Annex A of TS 33.122 [4].

Threat References: TR 33.926 [3], Annex [AA.2.2.X – Incorrect AEF_PSK returned]

Test Case:

Test Name: TC_VERIFY_AEF_PSK

Purpose:

Verify that the CCF computes the AEF_PSK correctly.

Pre-Condition:

-	The tester has access to the CCF via the CAPIF-1(e), CAPIF-3, and CAPIF-4 interface.

-	The tester is permitted to register an API invoker with the CCF.

-	The tester is permitted to register an API Exposing Function with the CCF.

Execution Steps:
Test Case 1: Verify AEF_PSK computation

1.	The tester triggers the CCF to register a new API Exposing Function by sending a Service API publish request to the CCF via the CAPIF-4 interface as explained in section 8.3.3 of 23.222 [X]. The Service API publish request must at least allow for the authentication via the TLS-PSK method.

2.	The tester triggers the CCF to register a new API Invoker by sending an Onboard API invoker request to the CCF via the CAPIF-1(e) interface as explained in section 8.1.3 of 23.222 [X].

3.	The tester derives AEF_PSK according to Annex A using the onboarding information received from the CCF in step 2 as specified in section 6.5.2.1 of 33.122 [4].

4.	The tester sends a security information request to the CCF via the CAPIF-3 interface as specified in section 6.5.2.1 of 33.122 [4] requesting the security information for the registered API Exposing Function and corresponding API invoker, and retrieves the AEF_PSK’ from the response.

5.	The tester compares the AEF_PSK derived in step 3 with the AEF_PSK’ received in step 4.

Test Case 2: Verify AEF_PSK selection

1.	The tester triggers the CCF to register a new API Exposing Function by sending a Service API publish request to the CCF via the CAPIF-4 interface as explained in section 8.3.3 of 23.222 [X]. The Service API publish request must at least allow for the authentication via the TLS-PSK method.

2.	The tester triggers the CCF to register two new API Invokers (API Invoker A and API Invoker B) by sending two Onboard API invoker requests to the CCF via the CAPIF-1(e) interface as explained in section 8.1.3 of 23.222 [X].

3.	The tester derives AEF_PSK for both API Invoker A and API Invoker B with the information received from the CCF in step 2 as specified in section 6.5.2.1 of 33.122 [4].

4.	The tester sends two security information requests for the security information for both API Invoker A and API Invoker B to the CCF via the CAPIF-3 interface as specified in section 6.5.2.1 of 33.122 [4] requesting the security information for the registered API Exposing Function and corresponding API invoker, and retrieves the AEF_PSK’ from both responses.

5.	The tester compares the AEF_PSK derived in step 3 with the AEF_PSK’ received in step 4 for API Invoker A and API Invoker B.

Expected Results:

For test case 1, the AEF_PSK derived in step 3 by the tester according to Annex A must match the AEF_PSK’ returned by the CCF for the registered API Exposing Function and API invoker in step 4.

For test case 2, AEF_PSK derived in step 3 for API Invoker A must match AEF_PSK received in step 4 for API Invoker A and AEF_PSK derived in step 3 for API Invoker B must match AEF_PSK received in step 4 for API Invoker B.

Expected format of evidence:

-	Evidence suitable for the interface, e.g., evidence can be presented in the form of log messages or a packet trace. A packet trace must at least contain the messages sent on the CAPIF-1(e), CAPIF-3, and CAPIF-4 interfaces.

-	The AEF_PSK values computed by the tester as well as the AEF_PSK values received from API invokers.

* * * End of Changes * * * *