Source:	Dolby Laboratories Inc., Ericsson LM, Fraunhofer IIS, Nokia, Orange, Philips, VoiceAge Corporation

Title:	IVAS Characterization Material Collection Entity Report

Agenda Item:	7.5

Document for:	Information

# Introduction

As one element of the IVAS Characterization listening tests, the input material to be processed by the CuT and reference conditions had to be collected, reviewed and prepared for processing. As outlined in IVAS-8b [1], this task was handled by the Material Collection entity (MC). The members of the MC were from the Public Collaboration Contributors: Dolby, Ericsson, Fraunhofer, Nokia, NTT, Orange, Philips and VoiceAge. The tasks of the MC are listed in IVAS-8b [1], Section 3.3.5:

“

- Collect the clean mono speech, background material, music and mixed content, real recorded stereo/immersive signals, and a pool of parameters for artificially created stereo/immersive sound material (e.g., impulse responses).

- Verify that the unprocessed sound material (for both artificially created and real recorded content) and parameters for artificially created stereo/immersive sound material meet the requirements defined by SA4.

- Choose the parameters and sound materials to be used in the experiments.

“

The MC responsibilities are further detailed in Annex C of IVAS-8b [1].

This is a report of the activity carried out by the MC. The MC reviewed the contributed material and the selected the material for the IVAS Characterization. The material collected for the IVAS Characterization test was shared under a Non-Disclosure Agreement (NDA). The MC further developed Item Generation Scripts that combined the selected raw input material into format expected by the IVAS Characterization Processing Scripts. The format and the nomenclature of Characterization Processing Scripts input material is described in the IVAS Characterization Processing Plan IVAS-7b [2]. This material was then made available to the Host Labs (HLs) and Cross-check Labs (CLs) to prepare the listening test material.

# Review and selection of material

Overall, the submitted test material was found to be of good quality, meeting the expectations of the listening test campaign. The material selected previously for the IVAS Selection test was reused as much as possible. Nevertheless, as the IVAS Characterization test is significantly more complex than the IVAS Selection test, new material was needed for several input formats.

Following the Characterization Test Plan IVAS-8b [1], some of the categories of the P.800 experiments required pre-produced material, similarly as required for the BS.1534 experiments. As not enough new audio material could be collected to cover all categories for all P.800 experiments, some of the material collected for the BS.1534 experiments was reused in the pre-produced content categories of the P.800 experiments.

Audio material for some of the IVAS input formats was more difficult to obtain than for other formats. This was particularly the case for some of the multi-channel configurations. It was thus decided to convert some of the collected HOA3 material to multi-channel formats.

During the review, the MC found some formal issues, along with occasional quality issues that required updates of the submitted material.

The input audio material for the Processing Scripts was created automatically from the submitted material using the MC Item Generation Scripts. For the pre-produced content of BS.1534 experiments and for some categories of the P.800 experiments, the Item Generation Scripts merely renamed the selected items from the contributed audio to the naming conventions of IVAS-7b [2]. For the audio content created artificially, the scripts created immersive audio scenes from the submitted raw mono audio recordings and supporting parameters. While this worked out correctly in general, on several occasions the resulting audio needed to be corrected. Thus, in addition to the updates to the submitted material, the MC also updated some of the resulting immersive audio. This was done by correcting the Item Generation Scripts or by manual editing of the immersive audio.

In some cases, the items were found difficult to replace, and a few problematic items were shifted from the main test corpus into the items used for the preliminary test sessions. These changes were made in coordination and agreement with the contributing party.

Annexes A and B provide details on the review and selection of material for the P.800 and BS.1534 experiments respectively.

Material review and selection P.800 DCR experiments

For most of the P.800 experiments, categories 1-4 relied on artificial immersive item creation implemented by the Item Generation Scripts by combining mono clean speech items, provided by the LLs, and supporting parameters, provided and selected by the MC. As per usual P.800 testing practice, the items were in native language of the listeners. Categories 5-6 used pre-produced immersive content. Further, in several P.800 experiments all the categories used only pre-produced content. All the pre-produced content was collected and selected by the MC. When the pre-produced items contained speech, items containing speech in the native language of the listeners were prioritized.

This section lists information about selection of the P.800 DCR tests material which is not already specified in Annex F of IVAS-8b [1].

Notes:

- Some of the selected pre-produced content was reused in both P.800 and BS.1534 experiments.

- Supporting parameters for some experiments were derived from parameters of other experiments.

- Supporting parameters for preliminaries were recycled from parameters of actual experiment.

- All the Stereo experiments reused the same supporting parameters.

- All the Stereo experiments reused the same background noises.

- Supporting parameters of the FOA experiment were reused in experiments derived from FOA (i.e. Multi-channel and MASA experiments).

- Background noises of the FOA experiment were reused in experiments derived from FOA (i.e. Multi-channel and MASA experiments).

P.800-1 Stereo

LL: VoiceAge Corporation

Language: North American French

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | VoiceAge | 84 |  |
| Car IRs | Nokia | 5 |  |
| Idle noise | Nokia | 1 |  |
| Background noise | Nokia | 1 |  |
|  | Orange | 1 |  |
| Mixed-content | VoiceAge | 7 |  |
| Music | VoiceAge | 7 |  |

Selected material

| Material | Category | Provider | Selected | Reused from |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | VoiceAge | 84 |  |
| SAMS Room IRs | 1 | ITU-T P.191 | 12 |  |
| LEAB Room IRs | 2 | ITU-T P.191 | 12 |  |
| SEBI Room IRs | 3 | ITU-T P.191 | 12 |  |
| Car IRs | 4 | Nokia | 5 |  |
| Idle noise | 1-2 | Nokia | 1 |  |
| Office noise | 3 | Orange | 1 |  |
| Car noise | 4 | Nokia | 1 |  |
| Mixed-content | 5 | VoiceAge | 7 |  |
| Music | 6 | VoiceAge | 7 |  |

P.800-2 Stereo

LL: NTT

Language: Japanese

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | NTT | 84 |  |
| Car IRs | Nokia | 5 |  |
| Idle noise | Nokia | 1 |  |
| Background noise | Nokia | 1 |  |
|  | Orange | 1 |  |
| Mixed-content | NTT | 7 |  |
| Music | NTT | 7 |  |

Selected material

| Material | Category | Provider | Selected | Reused from |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | NTT | 84 |  |
| SAMS Room IRs | 1 | ITU-T P.191 | 12 | P.800-1 |
| LEAB Room IRs | 2 | ITU-T P.191 | 12 | P.800-1 |
| SEBI Room IRs | 3 | ITU-T P.191 | 12 | P.800-1 |
| Car IRs | 4 | Nokia | 5 | P.800-1 |
| Idle noise | 1-2 | Nokia | 1 | P.800-1 |
| Office noise | 3 | Orange | 1 | P.800-1 |
| Car noise | 4 | Nokia | 1 | P.800-1 |
| Mixed-content | 5 | NTT | 7 |  |
| Music | 6 | NTT | 7 |  |

P.800-3 FOA

LL: Orange

Language: French

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Orange | 84 |  |
| SRIRs | Nokia | 48 |  |
|  | Dolby | 195 |  |
|  | Orange | 196 |  |
| Idle noise | Nokia | 1 |  |
| Background noise | Nokia | 4 |  |
|  | Dolby | 5 |  |
| Mixed-content | Orange | 7 |  |
| Generic audio | Orange | 5 |  |
|  | FhG | 2 |  |

Selected material

| Material | Category | Provider | Selected items | Comments |
| --- | --- | --- | --- | --- |
| Mono clean speech sentences | 1-4 | Orange | 84 |  |
| SRIRs | 1 | Nokia | 11 |  |
|  | 2 | Orange | 12 |  |
|  | 3 | Nokia | 12 |  |
|  | 4 | Dolby | 7 |  |
| Idle noise | 1-2 | Nokia | 1 |  |
| Background noise | 3 | Nokia | 1 |  |
|  | 4 | Dolby | 1 |  |
| Mixed-content | 5 | Orange | 7 |  |
| Generic audio | 6 | Orange | 5 |  |
|  |  | FhG | 2 |  |

P.800-4 HOA2

LL: Orange

Language: French

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Orange | 84 |  |
| SRIRs | Nokia | 53 |  |
|  | Dolby | 28 |  |
| Idle noise | Nokia | 1 |  |
| Background noise | Nokia | 4 |  |
|  | Dolby | 4 |  |
| Mixed-content | Orange | 7 |  |
| Generic audio | Orange | 5 |  |
|  | FhG | 2 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Mono clean speech sentences | 1-4 | Orange | 84 |  |
| SRIR | 1 | Nokia | 11 |  |
|  | 2 | Nokia | 8 |  |
|  | 3 | Nokia | 5 |  |
|  | 4 | Dolby | 8 |  |
| Idle noise | 1-2 | Nokia | 1 |  |
| Background noise | 3 | Nokia | 1 |  |
|  | 4 | Dolby | 1 |  |
| HOA2 mixed and music content | 5 | Orange | 7 |  |
| Generic audio | 6 | Orange | 5 |  |
|  |  | FhG | 2 |  |

P.800-5 HOA3

LL: Dolby

Language: English

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Dolby | 84 |  |
| SRIRs | Nokia | 34 |  |
|  | Dolby | 20 |  |
| Idle noise | Nokia | 1 |  |
| Background noise | Nokia | 7 |  |
|  | Dolby | 4 |  |
| Mixed-content | Dolby | 2 |  |
|  | FhG | 2 |  |
|  | VoiceAge | 6 |  |
| Generic audio | Dolby | 17 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Mono clean speech sentences | 1-4 | Dolby | 84 |  |
| SRIR | 1 | Nokia | 8 |  |
|  |  | Dolby | 6 |  |
|  | 2 | Nokia | 8 |  |
|  |  | Dolby | 3 |  |
|  | 3 | Nokia | 5 |  |
|  | 4 | Dolby | 5 |  |
|  |  | Nokia | 6 |  |
| Idle noise | 1-2 | Nokia | 1 |  |
| Background noise | 3 | Nokia | 1 |  |
|  | 4 | Dolby | 1 |  |
| HOA3 mixed content | 5 | Dolby | 2 |  |
|  |  | FhG | 2 |  |
|  |  | VoiceAge | 3 |  |
| HOA3 Generic audio | 6 | Dolby | 7 |  |

P.800-6 MC 5.1, 7.1

LL: Mesaqin

Language: Mandarin

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Mesaqin | 84 |  |
| SRIRs | Nokia | 48 | FOA-based |
|  | Dolby | 195 | FOA-based |
|  | Orange | 196 | FOA-based |
| Idle noise | Nokia | 1 | FOA-based |
| Background noise | Nokia | 4 | FOA-based |
|  | Dolby | 5 | FOA-based |
| Mixed-content | FhG | 7 |  |
| Generic audio | FhG | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | Mesaqin | 84 |  |
| SRIRs | 1 | Nokia | 8 |  |
|  |  | Dolby | 6 | FOA-based |
|  | 2 | Nokia | 5 | FOA-based |
|  |  | Dolby | 4 | FOA-based |
|  |  | Orange | 4 | FOA-based |
|  | 3 | Nokia | 5 | FOA-based |
|  |  | Dolby | 5 | FOA-based |
|  | 4 | Dolby | 12 | FOA-based |
| Idle noise | 1-2 | Nokia | 1 | FOA-based |
| Background noise | 3 | Nokia | 1 | FOA-based |
|  | 4 | Dolby | 1 | FOA-based |
| 5.1 music and mixed content | 5 | FhG | 7 |  |
| 7.1 music and mixed content | 6 | FhG | 7 |  |

P.800-7 MC 5.1+4, 7.1+4

LL: Force

Language: Danish

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Force | 84 |  |
| SRIRs | Nokia | 48 | FOA-based |
|  | Dolby | 195 | FOA-based |
|  | Orange | 196 | FOA-based |
| Idle noise | Nokia | 1 | FOA-based |
| Background noise | Nokia | 4 | FOA-based |
|  | Dolby | 5 | FOA-based |
| Mixed-content | FhG | 7 |  |
| Generic audio | FhG | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | Force | 84 |  |
| SRIRs | 1 | Nokia | 8 | FOA-based |
|  |  | Dolby | 6 | FOA-based |
|  | 2 | Nokia | 5 | FOA-based |
|  |  | Dolby | 4 | FOA-based |
|  |  | Orange | 4 | FOA-based |
|  | 3 | Nokia | 4 | FOA-based |
|  |  | Dolby | 10 | FOA-based |
|  | 4 | Dolby | 12 | FOA-based |
| Idle noise | 1-2 | Nokia | 1 | FOA-based |
| Background noise | 3 | Nokia | 1 | FOA-based |
|  | 4 | Dolby | 1 | FOA-based |
| 5.1+4 music and mixed content | 5 | FhG | 7 |  |
| 7.1+4 music and mixed content | 6 | FhG | 7 |  |

P.800-8 MC – Mixed CICP

LL: Mesaqin

Language: Czech

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Mesaqin | 84 |  |
| SRIRs | Nokia | 48 | FOA-based |
|  | Dolby | 195 | FOA-based |
|  | Orange | 196 | FOA-based |
| Idle noise | Nokia | 1 | FOA-based |
| Background noise | Nokia | 4 | FOA-based |
|  | Dolby | 5 | FOA-based |
| Mixed-content | FhG | 7 |  |
| Generic audio | FhG | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | Mesaqin | 84 |  |
| SRIRs | 1 | Nokia | 8 | FOA-based |
|  |  | Dolby | 6 | FOA-based |
|  | 2 | Nokia | 5 | FOA-based |
|  |  | Dolby | 4 | FOA-based |
|  |  | Orange | 4 | FOA-based |
|  | 3 | Nokia | 5 | FOA-based |
|  |  | Dolby | 5 | FOA-based |
|  | 4 | Dolby | 12 | FOA-based |
| Idle noise | 1-2 | Nokia | 1 | FOA-based |
| Background noise | 3 | Dolby | 1 | FOA-based |
|  | 4 | Dolby | 1 | FOA-based |
| 5.1 music and mixed content | 5 | FhG | 7 |  |
| 7.1+4 music and mixed content | 6 | FhG | 7 |  |

P.800-9 1-2 Objects

LL: Mesaqin

Language: Mandarin

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Mesaqin | 84 |  |
| Music and mixed content (1 object) | VoiceAge | 7 |  |
| speech + background (2 objects) | VoiceAge | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | Mesaqin | 84 |  |
| Music and mixed content (1 object) | 5 | VoiceAge | 7 |  |
| speech + background (2 objects) | 6 | VoiceAge | 7 |  |

P.800-10 3-4 Objects

LL: VoiceAge Corporation

Language: North American French

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Speech + effects (scene with 3 objects) | VoiceAge | 7 |  |
| Speech + music (scene with 3 objects) | VoiceAge | 7 |  |
| Music or effects (scene with 3 objects) | VoiceAge | 7 |  |
| Speech + effects (scene with 4 objects) | VoiceAge | 7 |  |
| Speech + music (scene with 4 objects) | VoiceAge | 7 |  |
| Music or effects (scene with 4 objects) | VoiceAge | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Speech + effects (scene with 3 objects) | 1 | VoiceAge | 7 |  |
| Speech + music (scene with 3 objects) | 2 | VoiceAge | 7 |  |
| Music or effects (scene with 3 objects) | 3 | VoiceAge | 7 |  |
| Speech + effects (scene with 4 objects) | 4 | VoiceAge | 7 |  |
| Speech + music (scene with 4 objects) | 5 | VoiceAge | 7 |  |
| Music or effects (scene with 4 objects) | 6 | VoiceAge | 7 |  |

P.800-11 1-4 Objects

LL: Force

Language: Danish

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Force | 84 |  |
| Speech + effects (3 objects) | VoiceAge | 7 | Effect objects reused from P.800-10 |
| Speech + music/effects or music only (4 objects) | VoiceAge | 7 | Music objects reused from P.800-10 |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | Mesaqin | 84 |  |
| Speech + effects (3 objects) | 5 | VoiceAge | 7 | Effect objects reused from P.800-10 |
| Speech + music/effects or music only (4 objects) | 6 | VoiceAge | 7 | Music objects reused from P.800-10 |

P.800-12 MASA 1 TC

LL: Mesaqin

Language: Mandarin

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Mesaqin | 84 |  |
| FOA-based SRIRs | Nokia | 48 | FOA-based |
|  | Dolby | 195 | FOA-based |
| HOA2-based SRIRs | Nokia | 34 | HOA2-based |
|  | Dolby | 20 | HOA2-based |
| FOA-based Idle noise | Nokia | 1 | FOA-based |
| HOA2-basded Idle noise | Nokia | 1 | HOA2-based |
| FOA-based Background noise | Nokia | 16 | FOA-based |
|  | Dolby | 5 | FOA-based |
| HOA2-based Background noise | Nokia | 1 | HOA2-based |
| Mixed-content | Nokia | 7 |  |
| Generic audio | Nokia | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | Mesaqin | 84 |  |
| HOA2-based SRIRs | 1 | Nokia | 8 | HOA2-based |
|  |  | Dolby | 6 | HOA2-based |
| FOA-based SRIRs | 2 | Nokia | 8 | FOA-based |
|  |  | Dolby | 6 | FOA-based |
|  | 3 | Nokia | 5 | FOA-based |
|  |  | Dolby | 5 | FOA-based |
|  | 4 | Nokia | 6 | FOA-based |
|  |  | Dolby | 8 | FOA-based |
| FOA-based Idle noise | 1 | Nokia | 1 | FOA-based |
| HOA2-based Idle noise | 2 | Nokia | 1 | HOA2-based |
| FOA-based Background noise | 3 | Nokia | 1 | FOA-based |
|  | 4 | Dolby | 1 | FOA-based |
| MASA 1 TC mixed content | 5 | Nokia | 7 |  |
| MASA 1 TC generic audio | 6 | Nokia | 7 |  |

P.800-13 MASA 2 TCs

LL: Force

Language: Danish

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Force | 84 |  |
| FOA-based SRIRs | Nokia | 53 | FOA-based |
|  | Dolby | 195 | FOA-based |
| HOA2-based SRIRs | Nokia | 34 | HOA2-based |
|  | Dolby | 20 | HOA2-based |
| FOA-based Idle noise | Nokia | 1 | FOA-based |
| HOA2-basded Idle noise | Nokia | 1 | HOA2-based |
| FOA-based Background noise | Nokia | 13 | FOA-based |
|  | Dolby | 5 | FOA-based |
| HOA2-based Background noise | Nokia | 1 | HOA2-based |
| Mixed-content | Nokia | 7 |  |
| Generic audio | Nokia | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | Force | 84 |  |
| HOA2-based SRIRs | 1 | Nokia | 8 | HOA2-based |
|  |  | Dolby | 6 | HOA2-based |
| FOA-based SRIRs | 2 | Nokia | 8 | FOA-based |
|  |  | Dolby | 6 | FOA-based |
|  | 3 | Nokia | 5 | FOA-based |
|  |  | Dolby | 5 | FOA-based |
|  | 4 | Nokia | 6 | FOA-based |
|  |  | Dolby | 8 | FOA-based |
| FOA-based Idle noise | 1 | Nokia | 1 | FOA-based |
| HOA2-based Idle noise | 2 | Nokia | 1 | HOA2-based |
| FOA-based Background noise | 3 | Dolby | 1 | FOA-based |
|  | 4 | Nokia | 1 | FOA-based |
| MASA 2 TCs mixed content | 5 | Nokia | 7 |  |
| MASA 2 TCs generic audio | 6 | Nokia | 7 |  |

P.800-14 MASA 1-2 TCs

LL: Nokia

Language: Finnish

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Nokia | 84 |  |
| FOA-based SRIRs | Nokia | 53 | FOA-based |
|  | Dolby | 195 | FOA-based |
| HOA2-based SRIRs | Nokia | 34 | HOA2-based |
|  | Dolby | 20 | HOA2-based |
| FOA-based Idle noise | Nokia | 1 | FOA-based |
| HOA2-basded Idle noise | Nokia | 1 | HOA2-based |
| FOA-based Background noise | Nokia | 13 | FOA-based |
|  | Dolby | 5 | FOA-based |
| HOA2-based Background noise | Nokia | 1 | HOA2-based |
| Mixed-content | Nokia | 7 |  |
| Generic audio | Nokia | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | Nokia | 84 |  |
| HOA2-based SRIRs | 1 | Nokia | 8 | HOA2-based |
|  |  | Dolby | 6 | HOA2-based |
| FOA-based SRIRs | 2 | Nokia | 8 | FOA-based |
|  |  | Dolby | 6 | FOA-based |
|  | 3 | Nokia | 5 | FOA-based |
|  |  | Dolby | 5 | FOA-based |
|  | 4 | Nokia | 6 | FOA-based |
|  |  | Dolby | 8 | FOA-based |
| FOA-based Idle noise | 1 | Nokia | 1 | FOA-based |
| HOA2-based Idle noise | 2 | Nokia | 1 | HOA2-based |
| FOA-based Background noise | 3 | Dolby | 1 | FOA-based |
|  | 4 | Nokia | 1 | FOA-based |
| MASA 1-2 TC mixed content | 5 | Nokia | 7 |  |
| MASA 1-2 TC generic audio | 6 | Nokia | 7 |  |

P.800-15 OSBA (1-2 objects)

LL: Dolby

Language: English

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Dolby | 84 |  |
| Idle noise | Nokia | 1 | reused from P.800-5 |
| Background noise | Nokia | 7 | reused from P.800-5 |
|  | Dolby | 4 | reused from P.800-5 |
| 1-object + General audio background | Dolby | 10 |  |
| 2-objects + General audio background | Dolby | 9 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Mono clean speech sentences | 1-4 | Dolby | 84 |  |
| Idle noise | 1-2 | Nokia | 1 | reused from P.800-5 |
| Background noise | 3 | Nokia | 1 | reused from P.800-5 |
|  | 4 | Dolby | 1 | reused from P.800-5 |
| 1-object + General audio background | 5 | Dolby | 7 |  |
| 2-objects + General audio background | 6 | Dolby | 7 |  |

P.800-16 OSBA (3-4 objects)

LL: Ericsson

Language: Swedish

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| 3 speech objects + ambient background | Ericsson | 7 |  |
| 3 speech objects + mixed content and music | Ericsson | 7 |  |
| 3 objects + general audio background | Ericsson | 7 |  |
| 3 speech objects + 1 speech/music/effect object + ambient background | Ericsson | 7 |  |
| 3 speech objects + 1 speech/music/effect object+ mixed content and music | Ericsson | 7 |  |
| 3 objects + general audio background | Ericsson | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| 3 speech objects + ambient background | 1 | Ericsson | 7 |  |
| 3 speech objects + mixed content and music | 2 | Ericsson | 7 |  |
| 3 objects + general audio background | 3 | Ericsson | 7 |  |
| 3 speech objects + 1 speech/music/effect object + ambient background | 4 | Ericsson | 7 |  |
| 3 speech objects + 1 speech/music/effect object+ mixed content and music | 5 | Ericsson | 7 |  |
| 3 objects + general audio background | 6 | Ericsson | 7 |  |

P.800-16 OSBA (1-4 objects)

LL: FhG

Language: German

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| 1 object + indoors background | FhG | 7 |  |
| 2 objects + indoors background | FhG | 7 |  |
| 3 objects + outdoors background | FhG | 7 |  |
| 4 objects + outdoors background | FhG | 7 |  |
| 2 objects + music background | FhG | 7 |  |
| 3 objects + music background | FhG | 7 |  |

Note: Some background material used for producing the content was taken from the QoEVAVE database [3]

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| 1 object + indoors background | 1 | FhG | 7 |  |
| 2 objects + indoors background | 2 | FhG | 7 |  |
| 3 objects + outdoors background | 3 | FhG | 7 |  |
| 4 objects + outdoors background | 4 | FhG | 7 |  |
| 2 objects + music background | 5 | FhG | 7 |  |
| 3 objects + music background | 6 | FhG | 7 |  |

P.800-15 OMASA (1-2 objects)

LL: Ericsson

Language: Swedish

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Ericsson | 84 |  |
| Idle noise | Nokia | 1 | Reused from P.800-5 |
| Background noise | Nokia | 9 | Some items reused from P.800-5 |
|  | Dolby | 4 | Reused from P.800-5 |
| 1-object + General audio background | Ericsson | 7 |  |
| 2-objects + General audio background | Ericsson | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Mono clean speech sentences | 1-4 | Ericsson | 84 |  |
| Idle noise | 1-2 | Nokia | 1 | reused from P.800-5 |
| Background noise | 3 | Nokia | 1 | reused from P.800-5 |
|  | 4 | Dolby | 1 | reused from P.800-5 |
| 1-object + General audio background | 5 | Ericsson | 7 |  |
| 2-objects + General audio background | 6 | Ericsson | 7 |  |

P.800-19 OMASA (3-4 objects)

LL: FhG

Language: German

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| 3-objects + General audio background | FhG | 7 |  |
| 3-objects + General audio background | FhG | 7 |  |
| 3-objects + General audio background | FhG | 14 |  |
| 4-objects + General audio background | FhG | 7 |  |
| 4-objects + General audio background | FhG | 7 |  |
| 4-objects + General audio background | FhG | 14 |  |

Note: Some background material used for producing the content was taken from the QoEVAVE database [3]

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| 3-objects + General audio background | 1 | FhG | 7 |  |
| 3-objects + General audio background | 2 | FhG | 7 |  |
| 3-objects + General audio background | 3 | FhG | 7 |  |
| 4-objects + General audio background | 4 | FhG | 7 |  |
| 4-objects + General audio background | 5 | FhG | 7 |  |
| 4-objects + General audio background | 6 | FhG | 7 |  |

P.800-20 OMASA (1-4 objects)

LL: Nokia

Language: Finnish

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| 1 object + indoors background | Nokia | 7 |  |
| 2 objects + indoors background | Nokia | 7 |  |
| 3 objects + outdoors background | Nokia | 7 |  |
| 4 objects + outdoors background | Nokia | 7 |  |
| 2 objects + music background | Nokia | 7 |  |
| 3 objects + music background | Nokia | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| 1 object + indoors background | 1 | Nokia | 7 |  |
| 2 objects + indoors background | 2 | Nokia | 7 |  |
| 3 objects + outdoors background | 3 | Nokia | 7 |  |
| 4 objects + outdoors background | 4 | Nokia | 7 |  |
| 2 objects + music background | 5 | Nokia | 7 |  |
| 3 objects + music background | 6 | Nokia | 7 |  |

P.800-21 JBM with Stereo

LL: Mesaqin

Language: Czech

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Mesaqin | 84 |  |
| Car IRs | Nokia | 5 |  |
| Idle noise | Nokia | 1 |  |
| Background noise | Nokia | 1 |  |
|  | Orange | 1 |  |
| Mixed-content | VoiceAge | 7 |  |
| Music | VoiceAge | 7 |  |

Selected material

| Material | Category | Provider | Selected | Reused from |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | Mesaqin | 84 |  |
| SAMS Room IRs | 1 | ITU-T P.191 | 12 | P.800-1 |
| LEAB Room IRs | 2 | ITU-T P.191 | 12 | P.800-1 |
| SEBI Room IRs | 3 | ITU-T P.191 | 12 | P.800-1 |
| Car IRs | 4 | Nokia | 5 | P.800-1 |
| Idle noise | 1-2 | Nokia | 1 | P.800-1 |
| Office noise | 3 | Orange | 1 | P.800-1 |
| Car noise | 4 | Nokia | 1 | P.800-1 |
| Mixed-content | 5 | VoiceAge | 7 |  |
| Music | 6 | VoiceAge | 7 | P.800-1 |

P.800-22 JBM with 1-2 objects

LL: Force

Language: Danish

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Force | 84 |  |
| Music and mixed content (1 object) | VoiceAge | 7 |  |
| speech + background (2 objects) | VoiceAge | 7 |  |

Selected material

| Material | Category | Provider | Selected | Comments |
| --- | --- | --- | --- | --- |
| Single sentences of mono clean speech | 1-4 | Force | 84 |  |
| Music and mixed content (1 object) | 5 | VoiceAge | 7 | Reused from P800-9 |
| speech + background (2 objects) | 6 | VoiceAge | 7 | Reused from P800-9 |

P.800-23 JBM with FOA

LL: Force

Language: Danish

Submitted material

| Material | Provider | Submitted | Comments |
| --- | --- | --- | --- |
| Mono clean speech sentences | Force | 84 |  |
| SRIRs | Nokia | 48 |  |
|  | Dolby | 195 |  |
|  | Orange | 196 |  |
| Idle noise | Nokia | 1 |  |
| Background noise | Nokia | 4 |  |
|  | Dolby | 5 |  |
| Mixed-content | FhG | 2 |  |
|  | Dolby | 2 |  |
|  | VoiceAge | 6 |  |
| Generic audio | Dolby | 17 |  |

Selected material

| Material | Category | Provider | Selected items | Comments |
| --- | --- | --- | --- | --- |
| Mono clean speech sentences | 1-4 | Force | 84 |  |
| SRIRs | 1 | Nokia | 8 |  |
|  |  | Dolby | 6 |  |
|  | 2 | Orange | 4 |  |
|  |  | Nokia | 5 |  |
|  |  | Dolby | 4 |  |
|  | 3 | Nokia | 4 |  |
|  |  | Dolby | 10 |  |
|  | 4 | Dolby | 12 |  |
| Idle noise | 1-2 | Nokia | 1 |  |
| Background noise | 3 | Nokia | 1 |  |
|  | 4 | Nokia | 1 |  |
| Mixed-content | 5 | FhG | 2 |  |
|  |  | Dolby | 2 |  |
|  |  | VoiceAge | 3 |  |
| Generic audio | 6 | Dolby | 7 |  |

Material review and selection BS.1534 experiments

Notes:

- Some of the audio content submitted to BS.1534 was reused as pre-produced content in P.800 DCR experiments.

- Input formats of some audio items were derived from other formats, e.g. some multi-channel audio items were derived from ambisonics formats.

- For some of the BS.1534 experiments, items for preliminaries were recycled from items of the main test corpus.

Submitted material per format

| Input Format | Provider | Nb of Items | Comments |
| --- | --- | --- | --- |
| Stereo | Ericsoon | 8 |  |
|  | FhG | 26 |  |
|  | Nokia | 9 |  |
|  | Orange | 36 |  |
|  | VoiceAge | 7 |  |
| FOA | Dolby | 20 |  |
|  | FhG | 13 |  |
|  | Nokia | 4 |  |
|  | Orange | 23 |  |
|  | VoiceAge | 8 |  |
| HOA3 | Dolby | 20 |  |
|  | FhG | 13 |  |
|  | Nokia | 6 |  |
|  | Orange | 23 |  |
|  | VoiceAge | 8 |  |
| MC 5.1 | Dolby | 17 |  |
|  | FhG | 65 |  |
|  | Nokia | 3 |  |
|  | Orange | 24 |  |
| MC 7.1 | Dolby | 17 |  |
|  | FhG | 50 |  |
|  | Nokia | 3 |  |
|  | Orange | 23 |  |
| MC 5.1+2 | Dolby | 17 |  |
|  | FhG | 50 |  |
|  | Nokia | 3 |  |
|  | Orange | 23 |  |
| MC 5.1+4 | Dolby | 17 |  |
|  | FhG | 50 |  |
|  | Nokia | 3 |  |
|  | Orange | 23 |  |
| MC 7.1+4 | Dolby | 17 |  |
|  | FhG | 50 |  |
|  | Nokia | 3 |  |
|  | Orange | 23 |  |
| 1 Object | Ericsson | 2 |  |
|  | FhG | 8 |  |
|  | Nokia | 2 |  |
|  | VoiceAge | 2 |  |
| 2 Objects | Ericsson | 2 |  |
|  | FhG | 8 |  |
|  | Nokia | 2 |  |
|  | VoiceAge | 2 |  |
| 3 Objects | Ericsson | 16 |  |
|  | FhG | 4 |  |
|  | Nokia | 7 |  |
|  | VoiceAge | 8 |  |
| 4 Objects | Ericsson | 16 |  |
|  | FhG | 4 |  |
|  | Nokia | 7 |  |
|  | VoiceAge | 8 |  |
| MASA 1 TC | Dolby | 5 | Converted from HOA3 |
|  | FhG | 4 | Converted from HOA3 |
|  | Nokia | 11 |  |
|  | Orange | 7 | Converted from HOA3 |
|  | VoiceAge | 4 | Converted from HOA3 |
| MASA 2 TCs | Dolby |  | Converted from HOA3 |
|  | FhG |  | Converted from HOA3 |
|  | Nokia | 5 |  |
|  | Orange |  | Converted from HOA3 |
|  | VoiceAge |  | Converted from HOA3 |
| OSBA 1-4 Objects | FhG | 24 |  |
|  | Nokia | 10 |  |
| OMASA 1-4 Objects | FhG | 20 |  |
|  | Nokia | 10 |  |
| Objects 6 DoF | Ericsson | 16 |  |

Selected Material

BS.1534-1 Stereo

LL: Force

| Provider | Selected |
| --- | --- |
| FhG | 5 |
| Nokia | 3 |
| VoiceAge | 2 |
| Orange | 6 |

BS.1534-2 Stereo

LL: Huawei

| Provider | Selected |
| --- | --- |
| FhG | 8 |
| Nokia | 2 |
| VoiceAge | 2 |
| Orange | 4 |

BS.1534-3 FOA

LL: Huawei

| Provider | Selected |
| --- | --- |
| Dolby | 3 |
| FhG | 3 |
| Nokia | 1 |
| Orange | 3 |
| VoiceAge | 2 |

BS.1534-4 FOA

LL: Panasonic/NTT

| Provider | Selected |
| --- | --- |
| Dolby | 3 |
| FhG | 3 |
| Nokia | 1 |
| Orange | 3 |
| VoiceAge | 2 |

BS.1534-5 HOA3

LL: Dolby

| Provider | Selected |
| --- | --- |
| Dolby | 3 |
| FhG | 3 |
| Nokia | 2 |
| Orange | 3 |
| VoiceAge | 1 |

BS.1534-6 Multichannel 5.1

LL: Ericsson

| Provider | Selected |
| --- | --- |
| Dolby | 5 |
| FhG | 7 |
| Nokia | 1 |
| Orange | 3 |

BS.1534-7 Multichannel 5.1, 7.1

LL: Dolby

| Provider | Selected |
| --- | --- |
| Dolby | 5 |
| FhG | 5 |
| Nokia | 1 |
| Orange | 1 |

BS.1534-8 Multichannel 5.1+4, 7.1+4

LL: Nokia

| Provider | Selected |
| --- | --- |
| Dolby | 4 |
| FhG | 4 |
| Nokia | 2 |
| Orange | 2 |

BS.1534-9 Multichannel 7.1+4

LL: FhG

| Provider | Selected |
| --- | --- |
| Dolby | 6 |
| FhG | 5 |
| Nokia | 1 |
| Orange | 4 |

BS.1534-10 1-2 Objects

LL: Qualcomm

| Provider | Selected |
| --- | --- |
| Ericsson | 4 |
| FhG | 5 |
| Nokia | 2 |
| VioceAge | 1 |

BS.1534-11 3-4 Objects

LL: Dolby

| Provider | Selected |
| --- | --- |
| VoiceAge | 16 |

BS.1534-12 3-4 Objects

LL: Philips

| Provider | Selected |
| --- | --- |
| Ericsson | 4 |
| FhG | 1 |
| Nokia | 4 |
| VioceAge | 7 |

BS.1534-13 MASA 1TC

LL: Qualcomm

| Provider | Selected |
| --- | --- |
| Dolby | 3 |
| FhG | 2 |
| Nokia | 3 |
| Orange | 2 |
| VoiceAge | 2 |

BS.1534-14 MASA 1TC

LL: Dolby

| Provider | Selected |
| --- | --- |
| Dolby | 2 |
| FhG | 2 |
| Nokia | 3 |
| Orange | 4 |
| VoiceAge | 1 |

BS.1534-15 MASA 2TC

LL: Nokia

| Provider | Selected |
| --- | --- |
| Dolby | 3 |
| FhG | 2 |
| Nokia | 2 |
| Orange | 3 |
| VoiceAge | 2 |

BS.1534-16 OSBA 1-4 Objects

LL: FhG

| Provider | Selected |
| --- | --- |
| FhG | 9 |
| Nokia | 7 |

BS.1534-17 OSBA 1-4 Objects

LL: Dolby

| Provider | Selected |
| --- | --- |
| Ericsson | 6 |
| Dolby | 6 |

BS.1534-18 OMASA 1-4 Objects

LL: Nokia

| Provider | Selected |
| --- | --- |
| FhG | 4 |
| Nokia | 8 |

BS.1534-19 Stereo downmix for EVS

LL: Orange

| Provider | Selected |
| --- | --- |
| FhG | 6 |
| Orange | 8 |

BS.1534-20 ISM 6 DoF (4 Objects)

LL: Ericsson

| Provider | Selected |
| --- | --- |
| Ericsson | 16 |

Material review and selection P.800 ACR

LL: Nokia

Language: Finnish

Material provided and selected by Nokia.

Immersive spatial material consisted of six different input formats, which were distributed to separate categories. The listened audio was listened in either stereo (stereo input) or rendered to binaural (all other input formats). Since listening testing was conducted in Finnish language and with naïve listeners all material was provided in Finnish language.

| Category | Input format | Signal content | Talkers / sample | Notes |
| --- | --- | --- | --- | --- |
| 1 | Stereo | Advertisement, news and speech over music | 1 | Commercially produced material (DTV capture), expect 1 speech over music sample. |
| 2 | FOA | Clean speech test sentences | 2 | Generated with SRIRs + Idle noise |
| 3 | HOA3 | Speech in car, over-talking | 2 | Generated with SRIRs + Car BG noise |
| 4 | MASA 1TC | Clean speech, over-talking | 4 | True microphone capture indoors |
| 5 | MASA 2TC | Speech outdoors (arboretum) | 2 | True microphone capture outdoors |
| 6 | ISM2 | Clean speech | 2 | 2 static, 2 with one moving, 2 with both moving talkers |

Material review and selection Room Acoustics experiments

Material provided and selected by Philips.

# References

[1] S4-251484 - IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, v1.1.1

[2] S4-251561 - IVAS-7b: Processing plan for characterization phase, v1.1.0

[3] Robotham, T., Singla, A., Rummukainen, O. S., Raake, A., Habets, E. A. P. Sept, 2022. Audiovisual Database with 360° Video and Higher-Order Ambisonics Audio for Perception, Cognition, Behavior, and QoE Evaluation Research. In Proc, 14th International Conference on Quality of Multimedia Experience (QoMEX). Lippstadt, Germany. DOI: 10.1109/QoMEX55416.2022.9900893