3GPP TSG- SA4 Meeting # 134	S4-251659

17-21 November 2025, Dallas, Texas, USA

3GPP TSG RAN WG2 #131bis	R2-2507790

Prague, Czech Republic, October 13th-17th 2025

# Title:	Reply LS on the RAN simulation assumptions, bundling period and SPS for ULBC

# Response to:	R2-2505068 / S4-251584 and R2-2506754 / S4aA250258

# Release:	Release 20

# Work Item:	FS_ULBC

Source:	RAN WG2

To:	SA WG4

Cc:	RAN WG1, SA WG2, CT WG1

Contact Person:

Name:	Bharat Shrestha

E-mail Address:	bshresth AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

RAN2 would like to thank SA4 for the LS. RAN2 would like to provide the following response.

Question 1: the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option.

RAN2 Answer:

For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size.

For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size.

RAN2 may provide further feedback on the expected average RoHC header size later.

Question 2: specifically, whether a packet overhead of 1 byte of MAC header is realistic.

RAN2 answer:

RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes.

Regarding question on whether the 120ms and/or 240ms bundling periods would be valid for the SPS operation in NB-IoT NTN, RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues.

# 2. Actions:

# To SA4:

ACTION:

RAN2 kindly asks SA4 to take the above response into account.

# 3. Dates of Next RAN2 Meetings:

RAN2#132						17th to 21st November 2025		Dallas

RAN2#133						9th to 13th February 2026		Gothenburg