3GPP TSG SA WG4 #134	S4-251664

Dallas, November 17-21, 2025	                                          Revision of S4aA250117

# Title:	RAN simulation methodology for ULBC

Source:	Qualcomm Incorporated, Thales, vivo, Huawei, Aalyria, Viasat

Agenda item:	7.9

# Document for:	Agreement

# 1. Background

The RAN simulation methodology has not been determined yet in the PD [1]. The paper S4aA250072 [2] proposed three approaches. There is also a proposal ‘option 3’ [4] with inputs from Dolby. In this paper, we build upon Approach 3 in [2] which is updated in [3], and propose a methodology that

- jointly optimize per-UE performance (TBS and BLER [6]) and the system capacity (how many UEs are serviced simultaneously in a cell) to create meaningful traces

- fixes the proposal ‘option 3’ [4] and includes it as an option for the joint optimization problem

The per-UE performance can be characterized by TBS and BLER. TBS determines the PHY bitrate and affects the user experience of the voice service. In a GEO NTN, since HARQ is disabled, BLER is equal to the Packet Error Loss Rate (PELR) associated with a QCI in the LTE QoS framework [6], hence a good metric for per-UE performance. There is a tradeoff between per-UE performance and the system capacity. While it is important to maximize the per-UE performance, it is important to also have a reasonable system capacity to make the service commercially meaningful. One way to quantify the tradeoff is to first select one of the target performance levels (TBS and BLER) and optimize the resource allocation to maximize the system capacity or equivalently to minimize the resource allocated to the UE, and then iterate through the target performance levels.

# 2. Simulation Workload Analysis

The number of all possible configurations of the system parameters and channel conditions for simulation are enormous and poses a challenge to the RAN simulation. To see this, consider the following in the table below as defined in the PD:

| Simulation parameter | Range |
| --- | --- |
| direction | U: uplink D: downlink |
| TBS | See TBS tables in the PD |
| Bundling period | 80, 160, 320 (ms) |
| Doppler spread | 1Hz, 5Hz |
| Number of RX | UL: 1 DL: 1, 2 |
| SCS | 3.75, 15 (kHz) |
| Number of tones | UL: 1 for 3.75kHz SCS; 1, 3, 6, 12 for 15kHz SCS DL: 12 |
| Length of NPUSCH or NPDSCH | UL: determined by ITBS, IRU, Nrep, and bundling period DL: determined by ITBS, ISF, Nrep, and bundling period |

First, consider the 80ms bundling period and UL. We have

- 4 TBS values (as in the current PD)

- 2 Doppler spreads

- For 3.75kHz SCS, there are two NPUSCH durations (32ms and 64ms)

- For 15kHz SCS, there are 4 different numbers of tones (1, 3, 6, 12), and the respective NPUSCH numbers of durations are 8, 16, 32 and 64. Note that 15kHz SCS and multiple tones may be reasonable for 31dBm maximum UE TX power.

- There are 4 BLERs (1%, 2%, 6%, 10%), which the traces will target

- There are 10 seeds

resulting in 4*2*2*4*10 + 4*2*(8+16+32+64)*4*10=39,040 loss traces.

For 160ms and 320 bundling periods in the UL, there are even more configurations. So, for UL alone, there are more than 117,120 traces.

Observation 1: generating loss traces for all possible RAN parameter configurations is not feasible, because there are too many (more than 117,120 for the UL alone) traces that would need to be generated.

# 3. Simulation assumptions

The UL and DL share the bundling period and thus the duration of one direction affects the other, and consequently the UL configuration and the DL configuration are not independent. There are many possible combinations of configurations. Reasonable assumptions need to be made for the simulation. To simplify the simulation, we assume:

- UL and DL have the same TBS, which captures the scenario where the UL and DL bitrates are equal

- UL and DL have the same BLER

  - This is consistent with QoS for LTE. A QCI gives the same packet error loss rate (PELR) upper bound for UL and DL of the same connection (EPS bearer) as specified in TS23.203 [5].

With the assumptions, the number of traces to be generated can be significantly reduced. For example, for the 80ms bundling and UL, one set of traces (10 each for a seed) is need for each combination of the following

- 4 TBS values (as in the current PD and could expand)

- 2 Doppler spreads

- 4 BLERs

- 3 UE power classes (23dBm, 26dBm, 31dBm)

resulting in 4*2*4*3*10=960 loss traces.

# 4. Analysis of the proposal for RAN simulation methodology in [4]

The proposed ‘option 3’ in [4], like [3], tries to avoid the brute-force approach to the simulation, but has issues:

- step 4 and step 6 are inconsistent, because by step 4, the BLER will never be higher than 10% whereas step 6 implies that the BLER could be higher than 10%.

- it lacks details on how to do the trade-off between the transmission performance (per-UE performance) and number of multiplexed UEs (system capacity)

More details are shown below.

Figure 1 Issues with option 3 of [4]

# 5. Proposed RAN simulation methodology

The objective is to create meaningful traces for the codec design.

There are numerous scenarios: many system configurations and various settings for the UE parameters (power class, number of receive antennas). We want to simulate reasonable scenarios in terms of per-UE performance and system capacity and thus propose the following RAN simulation methodology. Option 3.1 is from [3], while option 3.2 is an attempt to fix the issues (i.e., self-inconsistent and not actionable) with option 3 in [4].

The following RAN simulation methodology is proposed to be included in the TR or PD.

NOTE 1: when a company simulates step 3, it is up to the company to choose which of the options between 3.1 and 3.2 to simulate.

NOTE 2: it is reasonable to consider SNR values below the maximum achievable SNR value corresponding to the UE power class in clause 5.2.2, because a UE of a power class may not be able to transmit at the maximum power allowed for the power class. As an example, the network may reduce the maximum power of a UE by an Additional Maximum Power Reduction (A-MPR) [TS 36.101]. As another example, for a very high-power UE, e.g., 31dBm, the transmission power may have to be reduced to meet the SAR requirement.

Step 1: For each of the potential bundling times of 80, 160 and 320 ms, agree on a set of TBS to be evaluated.

Step 2: Generate the BLER vs SNR curves per bundling time per TBS with configurations (number of tones, SCS, NSF, NRU, ITBS, Nrep, Doppler spread, number of RX antennas for DL).

Step 3: for each bundling period, each TBS value, each Doppler spread, each UE power class, and each number of RX antennas

3.1 Simulation one (target QoS based simulation):

For each of the x% BLER, where x=1, 2, 6, 10

Find the configuration (number of tones, SCS, NSF, NRU, ITBS, Nrep) for UL and DL that leads to the maximum system capacity that meets the BLER in both UL and DL.

Output the required UL SNR and the required DL SNR

if the required UL and DL SNRs are less than the maximum achievable UL and DL SNRs respectively, where the maximum achievable UL SNR depends on the UE power class and the bandwidth of the UL configuration and the maximum DL SNR depends on the number of UE RX antennas:

The TBS is feasible.

Generate 10 loss traces at the required SNRs, each for a random seed.

end

[3.2 Simulation two (target SNR based simulation):

Find the configuration (number of tones, SCS, NSF, NRU, ITBS, Nrep) that maximizes the system capacity at the maximum achievable UL SNR for the UE power class and the configuration and the maximum achievable DL SNR.

If that both the UL BLER and DL BLER are less than or equal to 10%:

The TBs is feasible.

Generate 10 loss traces at the UL SNR and DL SNR above, each for a random seed.

end

]

NOTE 3: the maximum achievable SNR for the UL is determined by the UE power class and the configuration.

# References

[1] S4-251585, Permanent Document FS_ULBC, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[2] S4aA250072, "Potential approaches to determining feasible TBS values and generating loss traces," July 11, 2025.

[3] S4aA250273, “RAN simulation methodology for ULBC,” Qualcomm, 3GPP TSG SA WG4 Audio Ad Hoc meeting post #133-e, Erlangen, Sept 23-25, 2025.

[4] S4aA250216 revision 3, “Methodology for error trace generation,” vivo, 3GPP TSG SA WG4 Audio Ad Hoc meeting post #133-e, Erlangen, Sept 23-25, 2025.

[5] 3GPP TS 23.203, Policy and charging control architecture, v19.2.0, 2025.

[6] S4-251648, “3GPP NB-IoT NTN system and design parameters,” Qualcomm, 3GPP TSG SA WG4 #134, October 17-21, 2025.

[TS 36.101] 3GPP TS 36.101, “User Equipment (UE) radio transmission and reception.”