3GPP TSG-SA4 Meeting #134	S4-251712

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0010 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Energy-aware Streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-11-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Energy-aware streaming is tight to 3GPP architecture which is overall limiting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add access-independent energy-aware streaming and combine with 3GPP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 26.952 CR 0003 TR 26.952 CR 0004 TR 26.952 CR 0005 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.X	Solution #X: Energy-Aware Streaming

### 7.X.1	Key Issue mapping

This solution pre-dominantly addresses the following key issues:

-	Key Issue #2: Energy-related monitoring and measurement as introduced in clause 6.2

-	Key Issue #4: Energy-related configuration by the Application Service Provider for media delivery services in clause 6.4

-	Key Issue #5: Media Application Server Energy management in clause 6.5

-	Key Issue #6: Client-driven management of media delivery service energy optimisation in clause 6.6

The solution primarily addresses streaming services, for example in the context of 5G Media Streaming, and in particular the following aspects:

-	The key issues introduced above rely to some extent on information that is provided in the user plane, i.e. withn the streaming service itself. No specific solution is presented to address these issues.

-	There is a general hesitation to introduce and particular mandate the collection and reporting to the network of energy-related information in a UE.

-	At this stage, there is no means for a streaming media client, also referred to as media access function (client) to obtain information about energy related metrics related to selected streamed and consumed resources. Generally, a Media client could benefit from the energy attributed to the service and even better on service components, it is consuming.

-	Metrics assigned to energy consumption need to be well-defined. At this stage there is no well-defined single metric, or a well-defined metric, that would define energy consumption for streaming services. These metrics are still under development and require testing and feasibility in practice.

-	Energy consumption may be considered to address regulatory constraints, see for example key issue #1. This may require providing specific signaling and handling of such information, for example comparable to accessibility information. At this stage there is no attribution of energy-related information in the streaming services.

-	Generally, streaming service lack a framework to support reporting of energy-related information. A framerwork may beneficially define be defined.

Based on this discussion, this proposal provides on high-level the following aspects:

-	Integrate Energy-related information into general streaming services

-	Create an extensible framework that can be used by different environments

-	Combine the general framework with 3GPP services

In particular for the first two aspects, collaboration with other organisations may be considered useful.

### 7.X.2	Functional description

#### 7.x.2.1	Overview

Figure 7.X.2.1-1 provides an overview of a generic streaming workflow. The workflow may be considered as simple, and obviously can be extended for redundant operations, adding DRM, adding ad insertion and so on. However, for the purpose of the discussion, this is not relevant. The workflow is that produced content is provided to a distribution encoder, possibly in multiple variants (for example different resolutions, an SDR and an HDR version, and so on). The captured content may have associated static and/or dynamic information related to the energy needed to produce the content. The encoder encodes the content and aggregates the energy information provided upstream, and the energy/power that it required to encode, possibly also for different variants, for example the encoding technology, the bitrate and so on. The aggregated energy information may then be propagated through the distribution chain, by aggregating and accumulating the energy usage at each stage of the distribution. A core aspect is the concept that the information is included as metadata in the media streaming data, and different options are possible. The media may be delivered via a 3GPP delivery network and reach a media client, but may also be delivered via other means that are not 3GPP-based. The media client is very typically not 3GPP specific and hence, information is expected to be media access network agnostic. The media client or any node in the network may use information provided in the metadata to optimize the delivery towards energy-related metrics.

Figure 7.X.2.1-1: Streaming architecture with energy related information workflows

So, in summary, the following functional aspects are considered

-	Propagate energy-related information downstream within the media streaming workflow

-	Make the information accessible on streaming level, such that nodes in the delivery chain can access the information.

-	Allow to aggregate/accumulate the information at specific nodes

-	Collect information at appropriate nodes and use the provided information for operational decisions.

-	Allow to report the aggregated and accumulated information to reporting servers

-	Excluding the measured energy consumption at the client

-	Including the measured energy consumption at the client

-	Allow the client to select the content based on energy-related metrics

-	In order to provide meaningful and trustworthy energy reporting, the nodes adding metrics are preferably to be trusted energy consumption reporters.

In the following, some specific aspects related to issue are discussed.

#### 7.x.2.2	Assignment and Aggregation of Energy

#### Media resources available on a specific storage or distribution node may have assigned the Energy information it was consumed up to the current point, for example including production, encoding, distribution and so on. The information may be scaled to the number of users assigned to this resource. Aggregation may be done by summing the energy information or by other means of aggregation, depending on the assigned metrics.

The energy metric may be assigned to the entire media asset, or to a subset of the media asset, for example to a specific variant, to a specific segment or a time range.

#### 7.x.2.3	Energy-related Metrics Repository

Metrics assigned to energy consumption are not well-defined and may improve over time. A framework is expected to be applicable with an evolution of energy metric. For this purpose it is suitable to create a registry/repository/catalogue, simlar to MP4RA (https://mp4ra.org/) or DASH-IF identifiers (https://dashif.org/identifiers/introduction/) for energy related metrics. To each metric a key may be added, for example using a 4CC code or a URN. In addition, the metric is well-defined, typically by reference to a specification and/or implementation. This allows to use different metrics, and also allows to use a framework without having all definitions of metrics. As hypothetical examples,

-	Greening of Streaming defines a URN urn:org:gos:energyindex:2025 and/or a 4CC 'eeix' with a well-defined value being assigned to it and the energy index expressing the average aggregated consumed energy per second for the media assigned to it.

-	French regulator afnor defines urn:org:afnor:carbonindex:2025 and/or a 4CC 'eecx' with a well-defined value being assigned to and the carbon index expresses the average aggregated consumed carbon per second for the media assigned to it.

Such a repository may be hosted by a registration authority, or for example by an organization such as Greening of Streaming.

#### 7.x.2.4	In-band carriage of Energy-related Metrics

Once the metrics are well-defined, for example following what is presented in clause 7.x.2.2, the information may be carried by different means within the streaming distribution. Examples include, but are not limited to:

-	Metadata Tracks: If the information is dynamic, a metadata track may be used to carry one or different energy-related metrics. Metrics may also be delivered for example as part of Addressable Resource Index (ARI) tracks as defined in ISO/IEC 23009-1.

-	Common Media Client Data (CMCD) metrics: In the reporting, custom energy keys may be defined, for example using the definitions in clause 7.x.2.3, and providing the energy related to the consumed assets.

-	Common Media Server Data (CMSD) information: In the streaming of media assets, information may be assigned to each resource using data for each segment. This may again be using custom keys.

-	SAND information: CMCD and CMSD equivalent information may also be provided as part of SAND messages as defined in ISO/IEC 23009-5, for example using custom keys.

-	DASH metrics: Equivalently to CMCD, DASH metrics may be used to report energy related information.

-	Events and Event Streams: Event Streams may carry energy related information, similarly to CMSD.

-	Information in DASH MPD: A DASH MPD may carry information about energy metrics, for more details refer to clause 7.x.2.5.

-	Equivalently to the information in a DASH MPD, information in other streaming manifest such as HLS manifests may be carried.

-	Service Information and API calls as defined in MPEG DASH allow to carry operational information that defines the API to the client from the application and may provide energy-related information.

The information may be assigned to the entire delivery chain or parts of the delivery chain.

#### 7.x.2.5	Manifest carriage of Energy Information

A streaming manifest may carry energy-related information. In the following, examples for a DASH MPD are provided.

-	Energy information may be assigned to each resource in the MPD, including all information relevant for the service provider.

-	Energy information may be assigned to individual components

-	Content – providing information on aggregate energy consumption

-	An Adaptation Set

-	A Representation

-	A Segment

-	Service Locations – providing information on the energy required to distribute content from this service location, possibly normalized to the bitrate.

-	Adding labels describe energy-related information

-	In order to address regulatory requirements, a dedicated Energy descriptor Energy is added to the DASH manifest that allows carriage and exposure of energy-related information, similar as Accessibility information.

-	If information is available on different levels, it may be combined (added for example or combined after scaling appropriately).

#### 7.x.2.6	Usage of Energy information in client or other functions

The information in the manifest, as documented in clause 7.x.2.5 may be used in the client for different means:

-	Exposing energy related information to the user

-	Exposing energy related information to the application

-	The user and/or application using the information to select content based on energy

-	Reporting the aggregated metric to a reporting server, for example using DASH Metrics or CMCD.

-	Possibly adding client energy consumption to the aggregated metrics

-	Selecting content and/or service locations based on energy related information

Other functions in the delivery chain may use the inband information for operational and reporting purposes as well.

#### 7.x.2.7	Integration into 3GPP/5G network

The generic energy-aware streaming may be integrated into a 3GPP/5G network. Examples include:

-	The CDN may be an AS.

-	CDN1 and CDN2 may be mapped to different service locations. Delivery from multiple service locations may be used to optimize energy usage. For example, the client may select statically and/or dynamically service locations with different energy metrics assigned. Content Steering may be used to steer clients to energy efficient delivery.

-	Reporting from the client may include client UE consumption or may just report what was provided from the network, by aggregating the selection.

-	The AS may collect information from the AF/EIF to include energy-related metrics into the streaming metadata, for example the manifest or CMCD data.

### 7.X.3	Procedures

#### 7.x.3.1	General Procedures

Figure 7.X.3.1-1 provides a general high-level call flow for enery-aware streaming.

Figure 7.X.3.1-1 General call flow for enery-aware streaming

Here is the description of the call flow:

1.	Encoder

a)	Processes ingested media.

b)	processes energy information (production).

c)	Sends encoded media to Packager.

d)	Shares accumulated energy info with Packager.

2.	Packager

a)	Receives encoded media.

b)	Packages media.

c)	Sends packaged media to Manifest Generator.

d)	Shares accumulated energy info with Manifest Generator.

3.	Manifest Generator

a)	Receives packaged media.

b)	Uploads media and manifest to CDN1 and CDN2.

c)	Adds energy info for each media component (Adaptation Set, Representation, etc.)

d) Adds energy info wfor each service location/CDN

4.	Delivery

a)	Delivers manifest to Media Client.

b)	Media Client request media based on energy information from CDN1 and/or CDN2.

5. Reporting

- Media client reports aggregated energy information to Reporting Server.

#### 7.x.3.2	Integration with 5G Media Streaming

For further study.

### 7.X.4	Summary

For further study.