Agenda item: 	10.5

Source: 	Qualcomm Inc.

Title: 	[AIML_IMS-MED] MF Selection for AI/ML DC applications

Document for	Discussion and Agreement

# Introduction

In this contribution, we discuss the usage of the MF profile to assign the right MF for AI/ML processing tasks.

# MF Selection for AI/ML

## Background

CT4 has confirmed that MF registration already supports operator defined media capability labels. TS 29.510 defines MfInfo with a mediaCapabilityList. The list contains MediaCapability strings. The strings are defined by the operator. The data type is a string with an alphanumeric and underscore pattern. The MF can be discovered using these labels. Locality based discovery is also supported. These functions are present in the release-18 and later specifications. They include a query parameter for media-capability-list in NF discovery. They also include a parameter for ims-domain-name under the NG_RTC feature. This allows an IMS entity to find a compatible MF instance for a given domain and capability set.

## 2.2		Problem statement

SA4 work on AI/ML introduces application DC flows for AI/ML media processing. These flows require deterministic MF selection. The AS must assign an MF that offers DC handling and the specific AI/ML capabilities to run the AI/ML processing task. Today the binding between a DC application and MF capabilities is implementation specific. Interoperability suffers in roaming and multi-vendor deployments. The AS also needs a stable way to express capability needs at discovery time.

2.3		MF Profile for AI/ML

A DC application that requires specific MF processing, should be associated with an MF profile at the DC AS. The association identifies the MediaCapability labels needed by the application. The association also identifies optional policy such as locality and codec constraints. The DC AS uses NRF discovery for NF type “MF”. The DC AS includes a media-capability-list that contains “DC” and the application specific labels. The DC AS may include ims-domain-name when the deployment requires domain scoping. The DC AS may prefer MF instances that match the locality of the user or the locality of the P-CSCF or a configured service area. These parameters are already supported in TS 29.510.

The procedure may look as follows:

- The UE requests an application DC as part of the IMS session.

- The DC AS identifies the application.

- The DC AS derives the MF profile bound to the application.

- The DC AS queries the NRF with the media-capability-list.

- The DC AS receives candidate MF profiles that satisfy all requested capabilities.

- The DC AS selects one MF instance.

- The DC AS assigns the MF to the call and provisions the application logic.

- The DC AS continues with DC negotiation and call anchoring.

- The MF performs the requested AI/ML function on the agreed streams or data.

The MF registers a MediaCapabilityList that includes DC. The MF also registers function labels for AI/ML inference support. The labels describe modality, task, codec awareness, latency class, and compute framework. The MF exposes a small set of key attributes in mfInfo. The attributes identify supported model families and input descriptors.

An example mfInfo follows.

{ "mfInfoList": { "default": { "mediaCapabilityList": [ "DC", "AIML_AUDIO_SE_16K", "AIML_AUDIO_VAD", "AIML_AUDIO_ASR_EN", "AIML_VIDEO_VBG", "AIML_VIDEO_POSE2D" ], "aimlInfo": { "modalities": ["audio", "video"], "models": [ {"family":"se","variants":["se_v1","se_v2"],"quant":["fp16","int8"]}, {"family":"vad","variants":["vad_v1"],"quant":["int8"]}, {"family":"asr_en","variants":["conformer_s","conformer_m"],"quant":["int8"]}, {"family":"vbg","variants":["green_v1"],"quant":["fp16"]}, {"family":"pose2d","variants":["light_v1"],"quant":["int8"]} ], "inputs": [ {"modality":"audio","formats":["EVS","AMR-WB","L16"],"sampleRates":[16000,48000]}, {"modality":"video","formats":["H264","H265"],"frameRates":[30,60]} ], "latencyClass":"interactive_20ms", "computeClasses":["GPU",’NPU”] } } } }

The association of inputs to media streams relies on mid but needs to be addressed to ensure interoperability.

# Proposal

We propose take the information in section 2 into account, define MF capability profiles for AI/ML processing, and liaise with SA2 and CT4 to point out the need for assigning a processing task and associating the real-time media of an IMS session.

# References

[1]	 	TS 29.510, 5G System; Network function repository services; Stage 3