Agenda item: 	10.5

Source: 	Qualcomm Inc.

Title: 	[AIML_IMS-MED] Split Inference in DC Apps

Document for	Discussion and Agreement

# Introduction

In this contribution, we describe split inference in the context of the DC web applications and identify the needs for standardization.

# Split Inference

## 2.1		Use case for split inference

Live translation in an IMS context offers a clear split inference pattern. The sending endpoint executes tokenization and embedding on the device. The endpoint transmits only embeddings over the existing application data channel. The receiving side or a service performs decoding and generation. This arrangement reduces uplink volume and protects raw content. Latency improves because the initial stage starts locally. The same concept extends to speech and vision. Lightweight feature extraction runs at the sender. Heavier inference runs where sufficient resources exist. The split remains under application control and does not alter the transport architecture.

## 2.2		Inference in web applications with WebNN

WebNN is a W3C API for on‑device neural network inference. The API exposes navigator.ml for context creation and a graph builder for operators and tensors. It presents a hardware‑agnostic layer over platform ML backends. The current specification is a Candidate Recommendation Draft dated October 31, 2025. The specification also defines an operator support query surface that includes opSupportLimits. The design focuses on execution and does not define a model file format.

Implementation remains in a preview state. Chromium based browsers expose WebNN behind a developer flag. Microsoft documents GPU and NPU support on Windows through DirectML and notes that production use is not advised at this time. Progress is tracked in Chrome Platform Status and related engineering threads.

Framework integration is practical. ONNX Runtime Web provides a WebNN execution provider with fallbacks to WebGPU or WebAssembly when an operation is not accelerated. This path allows existing ONNX models to run in a browser without a new web specific format.

A demonstration [2] illustrates the usage of WebNN with quantized ONNX models in web environments. The WebNN camera segmentation and emotion page loads models as static assets. The page reports backend selection and per‑frame timing. The example shows an application-controlled pipeline that targets WebNN when available.

## 2.3	Standardization needs

Application packaging should carry the model as a static resource. The web application fetches and caches the model with the same mechanisms used for scripts and media. Existing HTTP and Service Worker capabilities cover versioning, integrity, and caching. A separate model hosting architecture or fetching standard is not required.

Input and output formats should remain application specific. Preprocessing and postprocessing belong to the application. Tensors, shapes, and data types are internal constructs within one application. Intermediate data such as embeddings or feature maps are not interoperable protocol elements. Formalizing those internals would freeze a fast-moving space and would not improve interoperability for end users.

Compression does not require a standard. If a model is compressed, the application ships a JavaScript or WebAssembly decompressor as part of its bundle. Control stays with the application and evolves quickly. In practice, compression will not change the feasibility ceiling for extremely large models in the browser. Memory and power budgets remain the main constraints for multi-billion parameter networks across the installed base. Even 7B class models require special loading strategies and careful memory handling to run in a browser. The limiting factor is not a missing compression standard. The limiting factor is device capacity and runtime constraints [3].

The essential standardization work item is capability discovery that is precise and available before model download. The application must learn what a user agent can accelerate and with which limits. The WebNN specification already defines navigator.ml and a structured operator support query through opSupportLimits. The following is an example of the usage of onSupportLimits query:

async function canRunModel(modelRequirements) { const context = await navigator.ml.createContext({ deviceType: 'gpu' }); // Model requires specific operations and constraints const requirements = { conv2d: { inputShape: [1, 3, 224, 224], dataType: 'float32', needsDilation: true }, gemm: { inputShape: [1, 1000], dataType: 'float32' } }; // Check conv2d requirements const convLimits = await context.opSupportLimits('conv2d'); if (!convLimits.dilations?.supported && requirements.conv2d.needsDilation) { console.error('Model requires dilated convolutions, but not supported'); return false; } if (!convLimits.input?.dataType?.includes(requirements.conv2d.dataType)) { console.error(`Model requires ${requirements.conv2d.dataType}, but not supported`); return false; } // Check if input dimensions are within limits const maxDims = convLimits.input?.dimensions?.max ¦¦ []; const requiredDims = requirements.conv2d.inputShape; for (let i = 0; i < requiredDims.length; i++) { if (maxDims[i] && requiredDims[i] > maxDims[i]) { console.error(`Input dimension ${i} exceeds max: ${requiredDims[i]} > ${maxDims[i]}`); return false; } } console.log('All model requirements are satisfied'); return true; }

And the corresponding answer:

// Example of what opSupportLimits might return for conv2d const exampleConv2DLimits = { input: { dataType: ['float32', 'float16'], layout: ['nchw', 'nhwc'], dimensions: { min: [1, 1, 1, 1], max: [1, 2048, 2048, 2048] } }, filter: { dataType: ['float32', 'float16'], layout: ['oihw', 'hwio'], dimensions: { max: [2048, 2048, 11, 11] } }, padding: ['explicit', 'same-upper', 'same-lower'], strides: { min: [1, 1], max: [8, 8] }, dilations: { supported: true, min: [1, 1], max: [8, 8] }, groups: { min: 1, max: 2048 }, bias: { supported: true, dataType: ['float32', 'float16'] }, activation: ['relu', 'relu6', 'leakyRelu', 'sigmoid', 'tanh'] };

The capability discovery functionality should be consistent across platforms and stable in its guarantees. Early capability discovery enables correct variant selection for precision, operator coverage, and tensor limits. Everything else should remain an application concern. WebNN stays as the execution layer. Hosting and fetching stay with HTTP over the application data channel. Intermediate data stays private to the application pipeline. Compression stays a library choice. This division of responsibilities keeps the platform simple and preserves evolution capability while remaining aligned with the current specification direction.

# Proposal

We propose to agree the content of section 2 and address the identified standardization needs.

# References

[1]	 	W3C, Web Neural Network API, W3C Candidate Recommendation Draft, 31 October 2025

[2]		WebNN demo, https://bouazizi.dev/webnn/

[3]		Google Research, Unlocking 7B+ language models in your browser: A deep dive with Google AI Edge's MediaPipe, Link