Agenda item: 	10.3

Source: 	Qualcomm Inc.

Title: 	External resources for DC applications

Document for	Discussion and Agreement

# Introduction

In this document, we discuss different solutions to address the issue of access to external resources raised by GSMA.

# Description of use case

GSMA NG UPG [1] has outlined a DC web application scenario in which a UE-resident web application delivered by the DC Application Server (DC AS) integrates an external AI Large Language Model (LLM) hosted at a third-party origin over HTTPS and authorized via OAuth 2.0. After the DC AS serves the UI over the Data Channel, the running application must contact one or more fully qualified external endpoints to submit prompts, stream partial outputs (e.g., via WebSockets, Server-Sent Events, or HTTP/2/3), and fetch auxiliary sub-resources such as scripts, fonts, images, or embedded authorization panels.

# Potential Solutions

The solutions being considered and feasible are two. The first treats the Media Function (MF) as a generic HTTP(S) proxy that front-fetches all external requests and returns responses back to the UE. The second preserves the web origin model and relies on a UE-enforced Content Security Policy (CSP), combined with Cross-Origin Resource Sharing (CORS) at the external origin, to permit only those cross-origin interactions that the DC AS explicitly declares.

3.1 	MF as TLS tunnel or HTTP(S) proxy

First off, using the MF as an HTTP(s) proxy that terminates the TLS connection is an extreme risk that completely exposes user’s credentials and application data to the MF, which may also leak beyond that. So this solution should not be considered.

In the proxy-tunnel over IMS Data Channel approach, the UE uses the DTLS/SCTP association established during call setup and opens one or more application data channels for tunnelling to external origins via the MF. Application data channels are not created in-band. TS 26.114 requires that each application data channel be represented in SDP with an a=dcmap attribute and that the UE send a subsequent SDP offer to add or update those channels. That offer can be conveyed by SIP re-INVITE or SIP UPDATE according to TS 24.229. After the offer/answer exchange is complete, the UE can send a tunnel establishment message over the negotiated stream. The MF validates the requested authority against local policy, opens a TCP connection to the external origin, and confirms the tunnel. The UE then runs a full TLS handshake to the origin through the tunnel, which preserves end-to-end confidentiality because the MF forwards encrypted bytes and does not hold session keys.

This signalling model has direct performance and scaling consequences as it maps one external destination to one application data channel. Each additional channel requires an SDP change and a new offer/answer round-trip across the IMS signalling path. The P-CSCF and S-CSCF process each update. The MF also participates as it anchors the data channel. Each renegotiation adds a non-trivial delay before any application bytes can flow on the new stream.

Implementation on the UE side is complex because standard browser and OS HTTP stacks do not expose IMS primitives. A web application must intercept fetch() and XMLHttpRequest, determine when a request should use a tunnel, triggers the IMS client to create a subsequent SDP offer with new a=dcmap entries, waits for the answer, and only then serializes HTTP over the SCTP stream. It must parse response bytes and reconstruct JavaScript Response objects. Error handling spans the Service Worker, SIP offer/answer, SCTP stream state, MF policy, and the external origin. Long-lived responses, such as Server-Sent Events for token streaming, increase fragility. The Service Worker must bridge flow control between the browser and the SCTP stream. Any SCTP reset or mid-call session change truncates the stream, and recovery requires application logic to re-establish both the tunnel and the HTTP transaction. All this is different from fetching static resources coming from the same origin as the web application HTML page as that automatically reuses the same connection and bootstrap application.

Provider-side security and operations are also affected. OAuth workflows rely on context such as source network and geolocation. Through the tunnel the provider observes the MF’s egress address rather than the UE’s carrier address. Tokens issued in one network context can appear replayed when used from another. Refresh flows that validate by IP continuity may fail. Cryptographic token binding to the TLS channel still verifies end-to-end, but provider heuristics that depend on IP reputation, ASN alignment, or regional consistency can misclassify traffic. Aggregation behind a single MF address collapses attribution. Rate limiting, quota enforcement, abuse mitigation, and usage-based billing lose per-user fidelity. A single abusive client can trigger blocks that affect all users sharing the MF. Geolocation and regulatory controls reflect the MF’s location instead of the UE’s, which creates both false rejections and potential compliance issues. Audit trails at the provider record the MF as the caller, which complicates incident response and per-user tracing.

3.2	CSP enforced at the UE with CORS at the external origin

A CSP-centric solution keeps policy expression at the DC Application Server and enforcement at the UE. The solution is fully compatible and enforced by all web browsers. The DC AS delivers the DC web application and declares a strict Content-Security-Policy in the same HTTP response over the bootstrap data channel. The policy starts from “default deny” and then allow only the external origins that the application requires for each resource class. The UE enforces the effective CSP locally. Any fully qualified URL that is not admitted by the policy is blocked before a network connection is made.

The policy constrains connections for data exchange with connect-src. Only the authorization server and the specific API endpoints needed by the LLM workflow are listed. Script execution is limited to code that the DC AS authorizes using a per-response nonce or stable hashes.

CSP operates together with CORS. Even when CSP admits an origin, a cross-origin request only proceeds if the external server opts in with the correct Access-Control-Allow-Origin value. When credentials are present, the external server returns the caller origin and does not use wildcards. This keeps OAuth flows predictable. Tokens are sent only to the hosts listed in connect-src. Frames and form submissions are restricted to the explicit authorization domains. The model adds no network element and preserves end-to-end TLS between the UE and the external provider.

The following headers illustrate typical deployments. Each header is emitted by the DC AS. The nonce value is generated uniquely per response and attached to the allowed <script> elements.

Example of a baseline CSP with default deny with nonce-based scripts:

Content-Security-Policy: default-src 'self'; script-src 'self' 'nonce-<per-response-random>' 'strict-dynamic'; style-src 'self'; img-src 'self' data:; font-src 'self'; connect-src 'self'; frame-src 'none'; object-src 'none'; base-uri 'none'; frame-ancestors 'none'; upgrade-insecure-requests; block-all-mixed-content

Example of an LLM integration with OAuth authorization server:

Content-Security-Policy: default-src 'self'; connect-src 'self' https://api.example-llm.com https://auth.example-llm.com; script-src 'self' 'nonce-<per-response-random>' 'strict-dynamic'; style-src 'self'; img-src 'self' data:; font-src 'self'; frame-src https://auth.example-llm.com; form-action 'self' https://auth.example-llm.com; object-src 'none'; base-uri 'none'; frame-ancestors 'none'; upgrade-insecure-requests; block-all-mixed-content

# Proposal

We propose to agree a CSP/CORS-based solution to this problem and suggest agreeing a CR as part of the DC Enhancements WID to add the clarifications to TS 26.114.

# References

[1]	 	S4-251437, GSMA, LS to 3GPP about the external data channel content access requirements