3GPP TSG-SA WG4 Meeting #134	S4-251753

# Dallas, US, 17 – 21 November 2025

Source:	Xiaomi

Title:	[FS_AVFOPS-MED] Discussion on improvement points in TS 26.265

Agenda item:	9.6

Document for:	DISCUSSION

# 1 Introduction

During the development of the conformance of TS 26.265, it has been identified that some normative features of the specifications need some improvements.

This contribution first presents the improvement points. Then, if those points are acknowledged to be unclear, the contributions propose solution to address them.

# 2 Improvement points

## 2.1 Flag for fixed frame rate in HEVC VUI constraint does not exist

### 2.1.1 Description

In the VUI constraints (clause 4.5.3	HEVC Bitstreams), the text reads:

The frame rate shall not change between two RAPs. fixed_frame_rate_flag value, if present, shall be set to 1.

But the fixed_frame_rate_flag syntax element does not exist in HEVC.

### 2.1.2 Proposed solution

Based on similar text (see improvement point 2.2), this seemed to be a left over and the correct flag name should be fixed_pic_rate_general_flag.

## 2.2 Timing constraints in both the HEVC VUI constraint and the HEVC operation point

### 2.2.1 Description

In the VUI constraints (clause 4.5.3	HEVC Bitstreams), the text reads:

For an HEVC/ITU-T H.265 [5] bitstream, VUI constraints are defined:

-	Video Parameter Sets (VPS) NAL units as defined in Recommendation ITU-T H.265 / ISO/IEC 23008-2 [5] may be present, but the Bitstream shall be valid if the Receiver ignores the VPS.

-	The Video Usability Information (VUI) is present in the active Sequence Parameter Set, i.e. the vui_parameters_present_flag shall be set to 1.

-	In the VUI,

-	the aspect ratio information is present, i.e. the aspect_ratio_info_present_flag value shall be set to 1,

-	the colour parameter information is present, i.e.  video_signal_type_present_flag value shall be set to 1 and the colour_description_present_flag value shall be set to 1.

-	only video range signals are used, i.e. the video_full_range_flag shall be set to 0,

-	no overscan signalling is present, i.e. the overscan_info_present_flag shall be set to 0,

-	the chroma location shall be signalled, i.e. chroma_loc_info_present_flag shall be set to 1,

-	the timing information may be present. If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed for each operation point. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level. The frame rate shall not change between two RAPs. fixed_frame_rate_flag value, if present, shall be set to 1.

Then, this VUI constraint is reused by reference in the following operations points 3GPP HEVC HD Operation Point (clause 6.3.2), 3GPP HEVC HDR Operation Point (clause 6.3.3), 3GPP HEVC UHD (clause 6.3.4), 3GPP MV-HEVC Stereo (clause 6.3.6).

However, all of those operation points redefine VUI constraints. For example, in 3GPP MV-HEVC Stereo:

8) The timing information may be present.

-	If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed for each operation point. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level.

-	The frame rate shall not change between two RAPs. fixed_pic_rate_general_flag value, if present, shall be set to 1.

### 2.2.2 Proposed solution

It seems that having the constraints on the VUI timing information in each operation point is a left-over, the proposal is to remove the timing information text from each operation point listed above (see improvement 2.3 why not all) since it is already inherited from the VUI constraint reference.

## 2.3 No VUI constraints in the 3GPP HEVC Stereo

### 2.3.1 Description

In the 3GPP HEVC Stereo operation point (clause 6.3.5), the bitstream constraints reads:

-	the Bitstream shall conform to HEVC/ITU-T H.265 Main 10 Profile, Main Tier, Level 5.2 [5] bitstreams with frame-packing constraints as defined in clause 4.5.3.

However, all the other HEVC operation points do inherit also from the VUI constraints, for example for 3GPP HEVC HD:

-	the Bitstream shall conform to HEVC/ITU-T H.265 Main 10 Profile, Main Tier, Level 4.1 [5] bitstreams with progressive and VUI constraints as defined in clause 4.5.3.

### 2.3.2 Proposed solution

It is not evident why the VUI constraint should not apply to the 3GPP HEVC Stereo operation point too since this operation point doe refine further VUI parameters. This seems to be a left-over and therefore, it is proposed to add the reference to the VUI constraint for this operation too.

## 2.4 Frame rate in the VUI constraints operation point specific or generic?

### 2.4.1 Description

The current frame rate reads as follows:

the timing information may be present. If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed for each operation point. […]

This makes this normative statement ambiguous in terms of conformance.

Indeed, the normative statement for the VUI constraints can be interpreted in three different ways:

- The values of vui_num_units_in_tick and vui_time_scale can be any values allowed by all operation points in the VOPS specification. This would make the VUI constraint generic and not specific to one operation point.

- The values of vui_num_units_in_tick and vui_time_scale can only be the values allows by the operation point being validated. This makes the VUI constraint operation point specific and the validation check is different for each operation point.

- The value of vui_num_units_in_tick and vui_time_scale can only the value of the actual signal that is in the bitstream. In that case, the validation check expects only a given value that is only known of the signal information is provided.

### 2.4.2 Proposed solution

We believe that the VUI constraint should be self-contained and be implementable regardless of external factors which is the case for all of them except this frame rate constraint.

Therefore, we would suggest making those changes:

- Remove normative constraints on vui_num_units_in_tick and vui_time_scale.

- Make the current text on frame rate signalling as a note.

Current text:

-	the timing information may be present. If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed for each operation point. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level. The frame rate shall not change between two RAPs. fixed_frame_rate_flag value, if present, shall be set to 1.

Proposed text:

-	the timing information may be present. If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed for each operation point. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level. The frame rate shall not change between two RAPs. fixed_frame_rate_flag value, if present, shall be set to 1.

NOTE: When the timing information present, the values of vui_num_units_in_tick and vui_time_scale are expected to be consistent with the frame rates allowed for the operation point and with the timing information signalled at the system level.

## 2.5 Ambiguous progressive constraints in multi-layer case

### 2.5.1 Description

Progressive constraints have been described in a single-layer context. In MV-HEVC, a depedent non-base layer (nuh_layer_id != 0) does not have the profile_tier_level() syntax in the SPS as show below:

Therefore, the progressive constraint and in particular “the general_progressive_source_flag shall be set to 1 in the SPS” cannot be applied to a non-base layer.

However, by bitstream conformance requirement, an independent non-base layer does have the profile_tier_level() syntax present in the SPS as defined in HEVC clause F.7.4.3.2.1 General sequence parameter set RBSP semantics:

– If the SPS is referred to by any current picture that belongs to an independent non-base layer, the value of MultiLayerExtSpsFlag derived from the SPS shall be equal to 0.

### 2.5.2 Discussion

If the second view is a dependent layer from the point of view of the bitstream, the profile, tier and level structure is thus not in the SPS of this layer but only in the base layer SPS. Inversely, if the second view is an independent layer from the point of view of the bitstream, the profile, tier and level structure would thus be in the SPS of this layer.

Shall all the layers of the bitstream be conforming to the progressive constraints? Or shall only the base layer be conforming to the progressive constraints? It seems logical that this is a bitstream-level constraint (interlaced vs progressive) but it should be clarified where in the bitstream the constraint needs to be applied.

# 2 Conclusion

We recommend discussing and documenting an agreeable way forward to address those identified issues.

# Reference

- 3GPP TS 26.265: "Media Delivery: Video Capabilities and Operation Points ", version 19.0.0.